3GPP TSG-RAN WG2 Meeting #116bis-e		R2-220xxxx

Electronic, 17th Jan. – 25th Jan. 2022

Source:	vivo

Title:	Summary of 	[AT116bis-e][056][ePowSav] RLM/BFD relaxation (vivo)

Agenda Item:	8.9.2.3

Document for:	Discussion and Decision

- Introduction

The document summarizes the following offline discussion:

- [AT116bis-e][056][ePowSav] RLM/BFD relaxation (vivo)

Scope: based on on-line agreements R2-2201684, and possibly other relevant input, attempt more progress offline, e.g. for configuration part

Intended outcome: Report, with Agreements (and-or Open Issues).

Deadline: Tue W2.

In order for rapporteur to have sufficient time to provide the summary, your comments before 22:00 UTC, 24th January is appreciated.

- Contact information

| Company | Name and email address |
| --- | --- |
| vivo | Chenli (chenli5g@vivo.com) |
| Qualcomm | Linhai He (linhaihe@qti.qualcomm.com) |
| Ericsson | Mattias Bergström (Mattias.a.bergstrom@ericsson.com) |

- Discussion

  - Enable/disable for RLM/BFD relaxation

During the online discussion, it was agreed that:

- Proposal 3b: [For Agreement] BFD relaxation is enable/disable per serving cell (i.e. separately between Pcell/PScell and Scell). FFS on stage-3 details. - Proposal 2b: [For Agreement] RLM relaxation is enable/disable per-CG (i.e. separately between Pcell and PScell). FFS on stage-3 details, FFS if enable/disable is by the UE or by the network.

In pre-meeting summary [12], it can be found some companies [5][6][7] suggest to enable/disable RLM/BFD relaxation separately, while one company [10] suggests to enable/disable RLM/BFD relaxation together for a cell.

Per online agreements above, there is no RLM relaxation enable/disable in Scell. Thus, we only need to discuss whether RLM relaxation and BFD relaxation for a cell (i.e. Pcell or PScell) should be enabled/disabled together.

- Companies are invited to show your views on whether RLM relaxation and BFD relaxation for a cell (i.e. Pcell or PScell) should be enabled/disabled together:

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Qualcomm | No | RLM and BFD serve different purposes and thus may be configured differently. And network may more likely to relax BFD but not RLM, even when they share the same set of reference signals. |
| Ericsson | No | Agree with QC. |

Regarding how to enable/disable RLM relaxation per-CG and how to enable/disable BFD relaxation per-serving cell, there are two options:

- Option A: Explicit indication to enable/disable the RLM/BFD relaxation, e.g. by indictor with 1bit.

- Option B: Implicit indication to enable/disable the RLM/BFD relaxation, e.g. by relaxation criteria configurations.

In RAN2#116e meeting, RAN2 have agreed: RAN2 assumes the presence/absence of configuration for RLM/BFD relaxation criteria in signalling indicates to the UE whether the UE can/should evaluate the criteria

Rapporteur assumes the configuration of RLM/BFD relaxation criteria could also present the enable/disable RLM/BFD relaxation feature implicitly as in option B (while how to perform relaxation/reporting will be discussed later).

For low mobility criterion, it was agreed in RAN4 that:

[Agreements in RAN4 101e] Issue 1-1-B: whether the low mobility criterion is mandatory to be configured, when network would like to enable RLM/BFD relaxation? - Conclusion: No. The criterion is NOT mandatory to be configured to enable RLM/BFD relaxation - Note: UE shall evaluate the low mobility criterion if it is configured.

It can be seen that low mobility criterion is optionally configured when network would like to enable RLM/BFD relaxation. Thus, the implicit indication to enable/disable the RLM/BFD relaxation cannot be based on the present of low mobility criterion.

For serving cell quality criterion, as far as I know, the corresponding discussion is being discussed in RAN4, including the below aspects:

- Whether serving cell quality criterion is configured per-UE/per-CG/per-serving cell

- Whether serving cell quality criterion is mandatorily configured when network would like to enable RLM/BFD relaxation is being discussed (i.e. whether the threshold for cell quality criterion can be pre-defined)

Companies are also invited to check with their RAN4 colleagues on the progress. Further confirmation/ information is appreciated.

In this way, whether serving cell quality criterion could be used to implicitly enable/disable RLM/BFD relaxation depends on the outputs of the above aspects.

If the serving cell quality criterion is configured per-CG for RLM relaxation and per-serving cell for BFD relaxation, as well as serving cell quality criterion is mandatorily configured when network would like to enable RLM/BFD relaxation (i.e. the corresponding threshold should not be pre-defined), then, the present/absent of serving cell quality criterion configuration can be used to implicitly enable/disable RLM/BFD relaxation. Otherwise, it cannot be used to implicitly enable/disable RLM/BFD relaxation.

We could of cause postpone this issue to wait for further progress from RAN4. But considering the timeline for Rel-17, which is close to the end of ePowSav WI, and we are not sure whether RAN4 could conclude above open issues by this meeting, so we could also make some progress in RAN2.

- Companies are invited to show your preference among the following options on how to enable/disable RLM/BFD relaxation:

- Option 1 (making decision): RAN2 assume to introduce explicit indicator to enable/disable the RLM/BFD relaxation.

Note: we could revisit this conclusion after further RAN4 progress.

- Option 2 (temporary decision): Don’t introduce explicit indicator to enable/disable the RLM/BFD relaxation, if RAN4 concluded:

  - serving cell quality criterion is configured per-CG for RLM relaxation and per-serving cell for BFD relaxation; and,

  - serving cell quality criterion is mandatorily configured when network would like to enable RLM/BFD relaxation (i.e. the corresponding threshold should not be pre-defined)Introduce explicit indicator to enable/disable the RLM/BFD relaxation

Otherwise, Introduce explicit indicator to enable/disable the RLM/BFD relaxation.

- Option 3: Postpone the discussion to wait for RAN4 conclusion on the configuration of criteria.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 3 | This decision may depend on RAN4’s decisions on, e.g. whether serving cell quality criterion is configured per cell or at another granularity. |
| Ericsson | Option 1? | RAN4 indicated in R4-2115349: For the relaxation criteria, RAN4 have also achieved agreements as follows. - Network to enable and disable this feature. (in RAN4 #98e) Further details can be discussed on exactly how this configuration should look like. But the NW should for sure be able to enabled/disable this feature. |

  - Configuration and evaluation for RLM/BFD relaxation criteria

In pre-meeting summary [12], it can be found some companies [1][6] suggest criteria for RLM relaxation and BFD relaxation are configured separately, company [3] suggests whether there are separated parameters for RLM/BFD relaxation leaves to RAN4.

Note: whether any criterion(/criteria) should be configured per-UE basis will be discussed later.

From rapporteur point of view, this issue is not being disussed in current RAN4 discussion in this meeting. Thus, it is better to have the discussion in RAN2.

- Companies are invited to show your preference among the following options on whether RAN2 assume the criteria configuration for RLM relaxation and BFD relaxation are configured separately:

- Option 1: Yes, please provide which configuration(s) could be configured separately, e.g. which relaxation criterion, or value(s) of which configuration.

- Option 2: No, please provide your reason.

- Option 3: Postpone the discussion to wait for RAN4 conclusion on the configuration of criteria.

Note: we could revisit this conclusion after further RAN4 progress.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 1 | RLM and BFD can share the same low mobility criterion. RLM and BFD can be configured with different serving cell quality criterion, as network may have different failure detection requirements for them. |
| Ericsson | Option 1 | Separate features so we should have separate configurations. Details TBD. |

In pre-meeting summary [12], how to provide the configurations of and how to evaluate RLM and BFD relaxation criteria in DC/CA scenarios is discussed in the contributions [1][4][5][6][7][8][10]. As summarized in [12]:

For RLM relaxation: All 6 companies think RLM relaxation criteria are configured and evaluated per-CG (i.e. separately between Pcell and PScell). For BFD relaxation: 6 companies think BFD relaxation criteria are configured (or configured and evaluated) per serving cell (i.e. separately between Pcell/PScell and Scell). 1 company thinks good serving cell quality criterion (if configurable) can be configured separately for BFD relaxation of each serving cell. Besides, one company suggests to provide configuration as part of CellGroupConfig rather than measConfig, e.g. in the SpCellConfig for RLM relaxation and SCellConfig for the SCell and the SpCellConfig for the SpCell for BFD relaxation. Rapporteur thinks this stage-3 details could be discussed later after we have conclusions on how to provide the configruations. In this way, rapporteur suggests the below proposal: Proposal 2a: [For Agreement] RLM relaxation criteria are configured and evaluated per-CG (i.e. separately between Pcell and PScell). FFS on stage-3 details (i.e. in which IE). Note: Whether low mobility criterion is configured per-UE basis will be discussed in proposal 5. Proposal 3a: [For Agreement] BFD relaxation criteria are configured and evaluated per serving cell (i.e. separately between Pcell/PScell and Scell). FFS on stage-3 details (i.e. in which IE). Note: Whether low mobility criterion is configured per-UE basis will be discussed in proposal 5.

Besides, for low mobility criterion, how to provide the corresponding configurations is discussed in below contributions. In pre-meeting summary [12]:

2 companies [5][8] mention that only one low mobility criterion for RLM/BFD relaxation can be configured for each UE (i.e. per-UE basis), and one company [8] thinks only SpCell is used for the criterion. 1 company [6] thinks low mobility criterion for RLM/BFD relaxation can be configured per-cell or per-CG. Proposal 5: [For Discussion] RAN2 to discuss whether low mobility criterion should be configured per-UE basis (i.e. only one low mobility criterion for each UE).

Rapporteur: As far as I know, how to provide the configurations of RLM and BFD relaxation criteria including low mobility and cell quality, and which part of parameters should be configured is being discussed in current RAN4 discussion in this meeting.

- For low mobility criterion, companies are invited to show your preference among the following options on RAN2 assume how to provide the criteria configuration for RLM relaxation and BFD relaxation:

- Option 1: per-CG for RLM relaxation, per-serving cell for BFD relaxation

- Option 2: per-UE for RLM relaxation and BFD relaxation, i.e. only one low mobility criterion for each UE

- Option 3: Postpone the discussion to wait for RAN4 conclusion.

- Option 4: others, please specify

Note: we could revisit this conclusion after further RAN4 progress.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 1 | We assume the question is about how to structure ASN.1 for those criteria, if we understand the question correctly, |
| Ericsson | Option 3 | This is being discussed in RAN4. |

Besides, for serving cell quality criterion, how to provide the corresponding configurations is discussed in below contributions. In pre-meeting summary [12]:

3 companies [4][5][6] think RAN2 should wait for more progress from RAN4 on good serving cell quality criterion. 2 companies [2][3] think parameters of offset X on threshold Qx for good serving cell quality is configured in dedicated signaling. Besides, 2 companies think the paramters for good serving cell quality should be configured in the IE RadioLinkMonitoringConfig. Rapporteur thinks the stage-3 details (i.e. value range of the parameters, in which IE) could be discussed later during running CR phase after we concluded whether the paramters for good serving cell quality is configured. Based on the rapporteur’s understaning on RAN4 progress, whether the offset/threshold for good serving cell quality criterion could be configured or pre-defined is still being discussed in RAN4. In this way, rapporteur suggests the below proposal: Proposal 6: [For agreement] RAN2 wait for more progress from RAN4 on Whether/How to configure good serving cell quality criterion.

- For cell quality criterion, companies are invited to show your preference among the following options on RAN2 assume how to provide the criteria configuration for RLM relaxation and BFD relaxation:

- Option 1: per-CG for RLM relaxation, per-serving cell for BFD relaxation

- Option 2: per-UE for RLM relaxation and BFD relaxation i.e. only one cell quality criterion for each UE

- Option 3: Postpone the discussion to wait for RAN4 conclusion.

- Option 4: others, please specify

Note: we could revisit this conclusion after further RAN4 progress.

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 1 |  |
| Ericsson | Option 3 | Already being discussed in RAN4. |

Regarding how to evaluate the criteria for RLM/BFD relaxation, RAN4 has made some related conclusions before:

[Agreements in RAN4 101e] Issue 6-2 Relaxation criteria in NR-DC and inter-band CA Conclusion: For the case of NR-DC and inter-band CA, UE can make the relaxation decisions separately for each serving cell configured for either RLM and/or BFD evaluation.

Based on the above agreement, the decision for RLM/BFD relaxation can be made separately for each serving cell. With this, rapporteur thinks we could assume the evaluation of RLM/BFD relaxation criteria should be performed separately for each serving cell.

Rapporteur: As far as I know, how to evaluate the RLM and BFD relaxation criteria, including per-sering cell (i.e. evaluate on Pcell/PScell/Scell), per-CG (i.e. evaluate on Pcell or PScell), per-UE (i.e. evaluate on Pcell) is not being further discussed in current RAN4 discussion in this meeting.

- For low mobility criterion, companies are invited to show your preference among the following options on how to evaluate the RLM and BFD relaxation criterion:

- Option 1: per-CG for RLM relaxation (i.e. evaluate on Pcell/PScell), per-serving cell for BFD relaxation (i.e. evaluate on Pcell/PScell/Scell)

- Option 2: per-UE for RLM relaxation and BFD relaxation, i.e. evaluate on Pcell

- Option 3: others, please specify

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 3 | Low mobility criterion can be evaluated per CG. Evaluation per UE may require message passing between two CGs in NR-DC configuration. |
| Ericsson | Option 3 - Wait for RAN4 | Already being discussed in RAN4. |

- For cell quality criterion, companies are invited to show your preference among the following options on how to evaluate the RLM and BFD relaxation criterion:

- Option 1: per-CG for RLM relaxation (i.e. evaluate on Pcell/PScell), per-serving cell for BFD relaxation (i.e. evaluate on Pcell/PScell/Scell)

- Option 2: per-UE for RLM relaxation and BFD relaxation, i.e. evaluate on Pcell

- Option 3: others, please specify

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 3 | Per CG for RLM; per serving cell for BFD. |
| Ericsson | Option 3 - Wait for RAN4 | Already being discussed in RAN4. |

  - Mechanism for RLM/BFD relaxation

Regarding how to perform RLM/BFD relaxation is discussed in the below contributions. In pre-meeting summary [12]:

3 companies [1][4][10] think UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. 3 companies [2][7][20] think UE performs RLM/BFD relaxation based on network indication. One company thinks the explicit RLM/BFD relaxation allowed/not allowed indication can be used irrespective if the RLM/BFD relaxation criteria is configured or not. One companythinks the network indication should be included in MAC CE. Besides, 1 company [1] thinks network can configure a maximum RLM/BFD relaxation interval, i.e. UE resumes normal measurement after having relaxed measurement for the configured interval. 1 company [1] thinks BWP switch doesn’t impact evaluation of BFD relaxation or ongoing relaxation of BFD measurement. Rapporteur: whether/how/what need to report fulfillment or not (entry/exit) to network is summarized/discussed in the next question. From rapporteur point of view, if UE performs RLM/BFD relaxation is based on network indication, where to take this indication could be FFS. In this way, rapporteur suggests the below proposals: Proposal 7: [For discussion] RAN2 to discuss the RLM/BFD relaxation mechanism: - Option 1: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria. Note: UE reporting will be discussed later as separate issue. - Option 2: Whether UE performs RLM/BFD relaxation is based on network indication. FFS irrespective if the RLM/BFD relaxation criteria is configured or if only apply to RLM/BFD relaxation criteria is not configured. Proposal 8: [For discussion] RAN2 to discuss whether a maximum RLM/BFD relaxation interval is needed, i.e. UE resumes normal measurement after having relaxed measurement for the configured interval. Proposal 9: [For discussion] RAN2 to discuss whether BWP switch impacts evaluation of BFD relaxation or ongoing relaxation of BFD measurement.

- Companies are invited to show your preference among the following options on how to perform RLM/BFD relaxation:

- Option 1: UE can start/stop RLM/BFD relaxation by itself if it meets/fails the relaxation criteria.

Note: UE reporting will be discussed later as separate issue.

- Option 2: Whether UE performs RLM/BFD relaxation is based on network indication.

FFS irrespective if the RLM/BFD relaxation criteria is configured or if only apply to RLM/BFD relaxation criteria is not configured.

- Option 3: Wait for RAN4 progress

- Option 4: Others, please specify

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 1 | Presence of relaxation criteria and measurement scaling factor is sufficient indication that network allows UE to perform relaxation when relaxation criteria are met |
| Ericsson | Option 1 | A report would be needed though. |

- Companies are invited to show your views on whether a maximum RLM/BFD relaxation interval is needed/configure, i.e. UE resumes normal measurement after having relaxed measurement for the configured interval:

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Qualcomm | See comment | Proponent. We proposed this maximum interval as a compromise if UE is allowed to start/stop relaxation by itself. If network is in full control, then this maximum interval is not needed. |
| Ericsson | No |  |

- Companies are invited to show your views on whether BWP switch impacts evaluation of BFD relaxation or ongoing relaxation of BFD measurement:

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Qualcomm | No | For the same reason that RLM/BFD procedures are not impacted by BWP switch |
| Ericsson | No |  |

For UEs supporting RLM/BFD relaxation, whether/how/what need to report fulfillment or not (entry/exit) to network is discussed in the below contributions. In pre-meeting summary [12]:

5 companies [2][3][5][7][10] think UE should report to network when RLM/BFD relaxation criteria is fulfilled or not (entry/exit). In which, 1 company thinks measurement reporting or UAI could be used for this reporting. One company [9] thinks MAC CE is used for reporting if the UE reporting is supported. 2 companies [4][8] think UE reporting of fulfilling or leaving the relaxation criteria is not needed for RLM and BFD relaxation mechanism. In which, 1 company [4] thinks the UE reporting is not needed when UE is configured by the network to perform relaxation of RLM/BFD. From rapporteur point of view, how to provide report to network when RLM/BFD relaxation criteria is fulfilled or not (entry/exit) could be discussed after we concluded whether the report is needed or not. In this way, rapporteur suggests the below proposals: Proposal 10: [For discussion] RAN2 to discuss whether UE should report to network when RLM/BFD relaxation criteria is fulfilled or not (i.e. entry or entry&exit). Proposal 11: [For discussion] If the report when RLM/BFD relaxation criteria is fulfilled or not (i.e. entry or entry&exit) is needed, RAN2 to discuss how to provide the report: - Option 1: in Measurement Report - Option 2: In UAI - Option 3: In MAC CE

- Companies are invited to show your preference among the following options on whether/how/what need to report fulfillment or not (entry/exit) to network:

- Option 1: Yes, please provide what information, e.g. fulfilled or not (entry/exit)

- Option 2: No, please provide your reason.

- Option 3: Wait for RAN4 progress

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Qualcomm | Option 2 | We don’t such a report is needed. See our comment to DP#8 |
| Ericsson | Option 1 | Entry/exit is sufficient. |

- Companies are invited to show your preference among the following options on how to provide the report, if the report when RLM/BFD relaxation criteria is fulfilled or not (i.e. entry or entry&exit) is needed.

- Option 1: in Measurement Report

- Option 2: in UAI

- Option 3: in MAC CE

- Option 4: Others, please specify

| Company’s name | Option(s) | Comments, if any |
| --- | --- | --- |
| Ericsson | Option 2 | Easiest. |

  - Coordination with RAN4

Regarding how to coordinate with RAN4 on RLM/BFD relaxation is discussed in the below contributions. In pre-meeting summary [12]:

Based on the contribution[1], rapporteur suggests the below proposal: Proposal 12: [For discussion] RAN2 to discuss whether the relaxation approaches as well as the corresponding requirements for relaxed RLM/BFD measurement should be captured in RAN4 specification, while the relaxation criteria as well as the configurations should be captured in RAN2 specification.

- Companies are invited to show your views on whether RAN2 can assume the relaxation approaches as well as the corresponding requirements for relaxed RLM/BFD measurement should be captured in RAN4 specification, while the relaxation criteria as well as the configurations should be captured in RAN2 specification.

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Qualcomm | No | In our view, - Requirements for relaxations should be captured in RAN4 spec; - Relaxation criteria should be captured in RAN1 spec (38.213), because measurement procedures are specified in the RAN1 spec and are transparent to RAN2 procedures; - Configurations aspects are captured in RAN2 spec (38.331) as usual. |
| Ericsson | Yes |  |

Regarding whether and what need to sent LS to RAN4 on RLM/BFD relaxation is discussed in the below contributions. In pre-meeting summary [12]:

3 companies [3][6][8] think RAN2 needs to send LS to RAN4 for following information: - RAN2 concern it is not efficient for BFD relaxation with low mobility criterion, as BFD is a beam level measurement (by evaluating BFD-RS), low mobility criterion is a cell level measurement (by evaluating cell RSRP). - RAN2 conclusions on the configurations and signaling. - Only one low mobility criterion for RLM/BFD relaxation can be configured each UE (i.e. per-UE basis), if agreed. In this way, rapporteur suggests the below proposal: Proposal 13: [For discussion] RAN2 to discuss whether an LS is needed to RAN4 at least including below information: - RAN2 concern it is not efficient for BFD relaxation with low mobility criterion, as BFD is a beam level measurement (by evaluating BFD-RS), low mobility criterion is a cell level measurement (by evaluating cell RSRP). - RAN2 conclusions on the configurations and signaling. - Only one low mobility criterion for RLM/BFD relaxation can be configured each UE (i.e. per-UE basis), if agreed.

From rapporteur point of view, at least the online agreements on how to enable/disable RLM/BFD relaxation is helpful for the RAN4 discussion, so they should be informed to RAN4.

- Companies are invited to show your preference among the following options on whether an LS is needed to RAN4 for RLM/BFD relaxation:

- Option 1: Yes, what information.

- Option 2: No, why

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Qualcomm | Yes | All the three points listed in Proposal 13 |
| Ericsson | Yes | Good to sort out the work split and what is specified where. To avoid duplicated work, etc. Perhaps there are also other relevant agreements. |

- Conclusion

This contribution summarizes the offline discussion [AT116bis-e][056][ePowSav] RLM/BFD relaxation (vivo), and achieves the following proposals:

Aiming to help with the meeting discussion/progress, the proposals are categorized starting with:

- [To agree] when there is large support and hence proposed for easy agreement.

- [To discuss] when there is substantial level of support and agreement may be possible.

- [FFS] when there is low support or companies propose new solutions or options to possibly consider further e.g. if there is sufficient support (understanding that these topic have not been discussed by all companies when providing their views in the different discussion points).

The proposals also start with a number: for the format [x], ‘x’ represents the number of supportive companies (i.e. these solutions are marked as FFS as the proposed solutions were not discussed by all companies) and, for the format [x/y], ‘x’ represents the number of supportive companies, and (y-x) the number of companies with different view.

The following list shows the proposals above organized based on the suggested priority aiming to help during its meeting discussion:

- Reference

- R2-2200186	Issues on RLM-BFD relaxations	Qualcomm Incorporated	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2200241	Discussion on RAN2’s impact of RLM/BFD relaxation	OPPO	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2200381	Discussion on RLM_BFD measurement relaxation	NEC Europe Ltd 	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2200451	Further considerations for RLM/BFD relaxation	Intel Corporation	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2200465	Discussion on RLM_BFD measurement relaxation	Beijing Xiaomi Mobile Softwar	discussion

- R2-2200594	Discussion on configurations of RLM/BFD relaxation for power saving	vivo	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201156	Discussion on RLM/BFD relaxation and DCI-based power saving adaptation	Huawei, HiSilicon	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201271	Consideration on RLM and BFD relaxation	CATT	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201544	RLM/BFD Relaxation Reporting	Interdigital, Inc.	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201578	Discussion on RLM/BFD Relaxation	LG Electronics Finland	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201614	On RLM/BFD relaxation	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_UE_pow_sav_enh-Core

- R2-2201684	Summary of 8.9.2.3 RLM BFD relaxation	vivo 	discussion	Rel-17	NR_UE_pow_sav_enh-Core