3GPP TSG RAN WG2 Meeting #116bis-e	R2-2201712

Electronic meeting, 17th -25th  January, 2022

Title:	LS to RAN3 on CPAC

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

- Name:	Erlin Zeng

- E-mail Address:	erlin.zeng@catt.cn

Attachments:

1. Overall description:

RAN2 discussed some remaining open issues on SN-initiated CPC and MN-initiated CPAC and reached the following agreements:

SN-initiated CPC

- RAN2 to confirm that in SN initiated inter-SN CPC, the S-SN is always informed about which candidates were accepted/rejected by T-SN.

- RAN2 thinks that MN is optional to indicate S-SN the candidates accepted or rejected by T-SN after receiving the SN addition acknowledge message from T-SN. And if MN skips the indication to S-SN before sending the CPC configuration to the UE, it sends the indication of accepted cells by T-SN to S-SN in some later step in the procedure. Up to RAN3 how the signalling is done efficiently.

- It is up to RAN3 to decide the message used for indicating the accepted PSCells from MN to S-SN, before sending the RRC Reconfiguration message including the CPC configurations to the UE.

- It is up to RAN3 to decide the message used for indicating the accepted PSCells from MN to S-SN, after sending CPC configuration to the UE.

- It is up to RAN3 to decide the message used for S-SN to provide the updated configuration.

- It is up to RAN3 to decide the message used for providing the RRCReconfigurationComplete message from MN to S-SN.

MN-initiated CPAC

- MN provides separate list of proposed PSCells to T-SN, and uses legacy candidate cell information (candidateCellInfoListMN) to provide the candidate cells recommended by MN to T-SN.

- For MN initiated CPAC, only the cells within the list recommended by MN can be chosen by T-SN.

2. Actions:

To 3GPP RAN3

ACTION:

RAN2 respectfully asks RAN3 to take the above agreements into account, in their further work on SN-initiated CPC and MN-initiated CPAC When related decisions are taken in RAN3, RAN2 kindly requests RAN3 to share relevant information with regard to above.

3. Date of next TSG RAN WG2 meetings:

RAN2#117-e		21st February to 03 March 2022, E-meeting

RAN2#118-e		16th to 27th May 2022, E-meeting