3GPP TSG-RAN WG2 Meeting #117-e	R2-2202408

# Electronic meeting, Feb 21st -  Mar 3rd, 2022

Agenda item:		8.11.2.1

Source:				CATT

Title:					Discussion and TP on areaID for Latency enhancements

Document for:	Discussion and Agreement

1	Introduction

The pre-meeting discussion proposed that how to define the area ID for pre-confguerd PRS should be addressed based on the companies’ contribution to the future meetings.

[Pre117-e][607][POS] Open issues on positioning latency enhancements (Huawei)

  - Issue3: definition of area ID

Currently in the LPP CR, the Area ID is listed as FFS and it has also been listed in the open issue list above. Since this issue has not been discussed in any detail in the previous meeting, we the issue should be handled by the company input to the next R2 meeting.

Summary:

Thus we propose the following:

Proposal : How to define the area ID for pre-confguerd PRS should be addressed based on the companies’ contribution to the future meetings.

In this contribution, we discuss how to define the area ID for pre-confguerd PRS and propose a TP on it.

2	Discussion

2.1 Motivation of area ID

The motivation of area ID is: when the pre-configured assistance data are larger than the assistance data in ProvideAssistanceData (more DL-PRS), the area ID may help UE improve the efficiency of these assistance data.

The same thing happens when the pre-configured assistance data is broadcast. The area ID in pre-configured assistance data may also help UE improve the efficiency of searching DL-PRS with area ID.

The area ID associated NR-DL-PRS-AssistanceDataPerTRPs show the DL-PRS valid with the same area-ID of the serving cell where UE stays.

2.2 What the area ID?

The area ID in Running LPP CR for NR positioning enhancements in R2-2201723 is defined but not completely. The Area-ID-r17 can be a INTEGER (0.. 255) which is similar with the definition of RAN-AreaCode, instead of sequence, because the INTEGER can save more on air resouces than cell list.

Taking 256 TRPs of DL-PRS as an example, the signalling analysis on two options is below:

- Option A: each TRP carries a list of cells which are 16 cells in maximum with PhysCellID in 10 bits

- Option B: each TRP carries area ID (INTEGER (0.. 255)) which indicate what the area the TRP belongs.

| Candidate options | Signalling resources |
| --- | --- |
| Option A: Cell list for each TRP | 256(TRP)*16 (cells)*10(PhysCellID) = 40Mbit |
| Option B: Area ID for each TRP | 256 TRP * 24(area ID) = 6Mbit |

85% signalling with Option B (area ID) can be saved compared with option A (cell list).

Proposal 1: RAN2 to agree to area ID as a INTEGER (0 .. 255) .

–	NR-DL-PRS-AssistanceData

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...,

[[

prs-OnlyTP-r16				ENUMERATED { true }		OPTIONAL	-- Need ON

]],

[[

area-ID-r17					Area-ID-r17				OPTIONAL	-- Need ON

]]

}

Area-ID-r17 ::= 					INTEGER (0..255)

}

-- ASN1STOP

The area ID associated NR-DL-PRS-AssistanceDataPerTRPs show the DL-PRS valid with the same area-ID of the serving cell where UE stays. Accordingly the description of area-ID can be added:

area-ID This field, if present, specifies the Area ID of the network area to which the TRP for which the NR-DL-PRS-AssistanceDataPerTRP is provided belongs to, The associated NR-DL-PRS-AssistanceDataPerTRPs with the same area-ID are available in the concerned area.

Proposal 2: RAN2 to agree to add “The associated NR-DL-PRS-AssistanceDataPerTRPs with the same area-ID are available in the concerned area.” in the description of area-ID.

Proposal 3: RAN2 to agree to capture the TP in the annex for area-ID in running CR of TS37.355.

3	Conclusion

Based on the discussions in section 2, the following observations and proposals have been made:

Proposal 1: RAN2 to agree to area ID as a INTEGER (0 .. 255) .

Proposal 2: RAN2 to agree to add “The associated NR-DL-PRS-AssistanceDataPerTRPs with the same area-ID are available in the concerned area.” in the description of area-ID.

Proposal 3: RAN2 to agree to capture the TP in the annex for area-ID in running CR of TS37.355.

4	References

- R2-2200304 Discussion on latency reduction enhancement  CATT

- R2-220xxxx [Pre117-e][607][POS] Open issues on positioning latency enhancements (Huawei)   Huawei, HiSilicon

- R2-2201723	Running LPP CR for NR positioning enhancements	Qualcomm

- R2-220xxxx	Coordinated Company Input For Rel-17 Open Issues Planning R2 117-e and impacts to R2 116bis-e	MediaTek (R2 Chairman)

- TS38.331, NR: Radio Resource Control (RRC) protocol specification

5	Annex

--------------------------------Start of TS 37.355----------------------------------------------------------------------------------------

–	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. the serving TRP.

NOTE 2:	The nr-DL-PRS-ReferenceInfo defines the "assistance data reference" TRP whose DL-PRS configuration is included in nr-DL-PRS-AssistanceDataList. The nr-DL-PRS-SFN0-Offset's and nr-DL-PRS-expectedRSTD's in nr-DL-PRS-AssistanceDataList are provided relative to the "assistance data reference" TRP.

NOTE 3:	The network signals a value of zero for the nr-DL-PRS-SFN0-Offset, nr-DL-PRS-expectedRSTD, and nr-DL-PRS-expectedRSTD-uncertainty of the "assistance data reference" TRP in nr-DL-PRS-AssistanceDataList.

NOTE 4:	For NR DL-TDOA positioning (see clause 6.5.10) the nr-DL-PRS-ReferenceInfo defines also the requested "RSTD reference".

For DL-PRS processing, the LPP layer may inform lower layers to start performing DL-PRS measurements and provide to lower layers the information about the location of DL-PRS, e.g. DL-PRS-PointA, DL-PRS Positioning occasion information.

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 		DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...,

[[

prs-OnlyTP-r16				ENUMERATED { true }		OPTIONAL	-- Need ON

]],

[[

area-ID-r17					Area-ID-r17				OPTIONAL	-- Need ON

]]

}

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16	INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...}

Area-ID-r17 ::= 					INTEGER (0..255)

}

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.

nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.

nr-SSB-Config This field specifies the SSB configuration of the TRPs.

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.

nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

nr-PhysCellID This field specifies the physical cell identity of the TRP. When the field prs-OnlyTP is included, this field is not included.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, as defined in TS 38.331 [35]. When the field prs-OnlyTP is included, this field is not included.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. When the field prs-OnlyTP is included, this field is not included.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes.

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

nr-DL-PRS-Info This field specifies the PRS configuration of the TRP.

dl-PRS-SubcarrierSpacing This field specifies the subcarrier spacing of the DL-PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2. All DL-PRS Resources and DL-PRS Resource Sets in the same Positioning Frequency layer have the same value of dl-PRS-SubcarrierSpacing.

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-StartPRB.

dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.

dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N.

dl-PRS-CyclicPrefix This field specifies the Cyclic Prefix length of the DL-PRS Resource. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-CyclicPrefix.

prs-OnlyTP This field, if present, indicates that the NR-DL-PRS-AssistanceData is provided for a PRS-only TP. Whether the field is present or absent should be the same for all the NR-DL-PRS-AssistanceData of all the PRS transmitted under the same TP. The target device shall not assume that any other signals or physical channels are present for the TRP other than DL-PRS.

area-ID This field, if present, specifies the Area ID of the network area to which the TRP for which the NR-DL-PRS-AssistanceDataPerTRP is provided belongs to. The associated NR-DL-PRS-AssistanceDataPerTRPs with the same area-ID are available in the concerned area.

------------------------------- End of TS 37.355-------------------------------------------------------------------------------------------