3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2202692

Online, February 21 – March 03, 2022

Source:	CATT

Title:	Analysis on UE capability for RAN slicing enhancement

Agenda Item:	8.8.4

Document for:	Discussion and Decision

Introduction

In RAN2#116e meeting, agreements on UE capability for RAN slicing were achieved [1]:

- As baseline, consider the following capabilities. FFS on details, can consider changes in the next meeting. - #1: UE indicates its support of slice based cell reselection in the UE capability signalling with the following TS38.306 description. Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. sliceInfoforCellReselection-r17 Indicates whether the UE supports sliceInformation on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE as defined in TS 38.304 [21].; UE; No; No; No. - #2: Since slice based RACH is only applicable for UE in RRC IDLE and RRC INACTIVE, there is no need for explicit capability to inform network and should just be “Optional without UE capability” as follow under Section 5.4 Other features: Definitions for feature. Slice based random access It is optional for UE to support slice based random access as specified in TS 38.321 [8]..

In this document, we continue to discuss the issues for UE capability for RAN slicing enhancements.

Discussion

  - UE capability on slice specific cell reselection

FFS#1

FFS#1 on the need of an optional without capability signalling for UE using only slice info in the SIB for slice based cell reselection in idle and inactive mode (i.e. there is no need for gNB to know such UE).

Regarding the UE feature without UE radio access capability parameters for slice specific cell reselection with slice specific frequency priority signalled in SIB, it can be supported in Clause 5.4 in 38.306 without too much complexity. And it can be captured as:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

So, we propose that:

Proposal 1: Introducing one optional feature in Clause 5 for slice-specific cell reselection with the following TS38.306 description:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

FFS#2

FFS#2 if there is a need to know such UE as in FFS#1 (i.e. it is not optional without capability signalling), whether same capability signalling as with UE indicating the support of slice info in RRC Release or a separate one is needed for UE indicating the support of slice info in SIB)?

It has been agreed in principle that UE will report capability to the network indicating whether it will support slice information on RRCRelease for slice based cell reselection in RRC_IDLE and RRC_INACTIVE. Since this capability is used to indicae the support of slice info in SIB, solution in Proposal 1 is more suitable. Hence, we think there is no need to support such a capability signalling indicating the support of slice info in SIB.

  - UE capaiblity on slice-based RA

UE capability on slice based prioritization

UE feature to support Random access prioritization for MPS and MCS has been supported in 38.306 in Rel-16. A similar UE feature to support Random access prioritization for slice based RA prioritization should also be supported in 38.306. It can be captured as:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

Slice-based RA partitioning

Regarding the UE capability for slice based RA partitioning, we think this can also be supported as one optional feature similarly. It can be one separate capability from slice based RA prioritization. And it can be captured as:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

General issue

FFS#3

FFS on whether to separate the support of slice based RACH prioritization and slice based RACH partitioning

Regarding issue#3, we think it is flexible to define separate UE capaiblity. To align with the proposal in [2], we think separate UE capability can be supported slice baed RACH prioritization and slice based RACH partitioning.

So we propose that:

Proposal 2: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

Proposal 3: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

FFS#4

FFS on whether to support dedicated RACH resources (RACH prioritization) – if so, there is a need for the gNB to know the capability.

There is no conclusion on whether the parameters configured for slice based RA can be sent in RRCRelease. From our perspective, the configuration for RA prioritization can be inherited from the previous serving cell when the UE performs cell reselection. However, the configuration for RA partitioning may be different in different serving cells. For simplicity, we think there is no necessity to support dedicated RACH configuration for slice. If so, the dedicated capability for RACH resources (RACH prioritization) is not needed either.

Proposal 4: No UE capability is reported for slice based RA.

- Conclusion

In this document, we analyse the issues for UE capability for RAN slicing enhancement, and we propose that:

Proposal 1: Introducing one optional feature in Clause 5 for slice-specific cell reselection with the following TS38.306 description:

Slice specific cell reselection It is optional for UE that is configured to support slice specific cell reselection with slice specific frequency priority[8].

Proposal 2: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access prioritization for Slice It is optional for UE that is configured to support random access prioritization for slice as specified in TS 38.321 [8].

Proposal 3: Introducing one optional feature in Clause 5 for slice-specific RA prioritization with the following TS38.306 description:

Random access partitioning for Slice It is optional for UE that is configured to support random access partitioning for slice as specified in TS 38.321 [8].

Proposal 4: No UE capability is reported for slice based RA.

- Reference

- RAN2#116e meeting report;

- R2-2202691 The remaining issues on slice specific random access;