3GPP TSG RAN WG2 #117-e		      R2-2202788

Online, 21st Feb. – 3rd Mar. 2022

Source:	vivo, China Telecom, CMCC, SoftBank, China Unicom, Vodafone, Ericsson

Title:	Early measurement for EPS Fallback

Agenda Item:	8.21.2

Document for:	Discussion and Decision

- Introduction

In RAN2#115-e meeting, [1] and [2] proposed additional measurements for EPS Fallback and load Distribution, and it can be observed that most companies are interested in the reduction of latency on EPS Fallback. And in last meeting, a method of early measurements for EPS Fallback was given in [3], however, some companies had concerns on the measurement accuracy, gains and complexity while other companies believed the early measurements can benefit.

In this contribution, we will give the analysis on the measurement accuracy, gains and complexity.

- Discussion

  - General Issues in EPS/RAT Fallback Procedure and load distribution

NG-RAN decides to trigger EPS Fallback or RAT Fallback taking account of the indication from AMF(i.e. Redirection for EPS fallback), gNB’s capability(e.g. NG-RAN supports handover or redirection to E-UTRAN connected to 5GC, or NG-RAN supports EPS fallback for IMS voice) and UE’s capability(e.g., UE supports IMS voice over EPS Fallback, or E-UTRAN via 5GC, or NR and so on) when it receives the request to set up QoS flow for IMS voice.

From the analysis in [1], the latency of the IMS voice setup from the calling perspective based on the EPS Fallback is around 2s to 4s. And the main aspects which impact the latency of EPS Fallback contains measurement configuration and report, EPS Fallback methods and whether caller and callee perform EPS fallback in parallel or in series.

Observation 1: There are three main aspects to impact the latency of EPS fallback

- Measurement configuration and report, i.e., radio quality and Number of LTE frequency

- EPS fallback methods, i.e. handover or redirection,

- Procedure of EPS fallback in parallel or in series for caller and callee.

Besides, when NW doesn’t have the measurement result from UE, it may perform EPS/RAT Fallback based on blind redirection, which may result in a longer delay or call failure.

Observation 2: The LTE frequency measurement configuration is only done after RRC connection setup, UE may not report measurement results in time due to radio quality and the Number of LTE frequencies, which leads to blind redirection procedure and results in a longer delay.

Based on the above, some enhancements to reduce the call latency of EPS/RAT Fallback are in need to prevent the degradation of user experience and reduce the call failure rate.

  - Early Measurement Configuration

In the last meeting, [3] proposed a simple method on early measurement for EPS Fallback which reuses the legacy idle/inactive measurement as much as possible. More specifically, reuse the configurations in RRCRelease and SIB11, including the measIdleCarrierListEUTRA, validityAreaList and measIdleDuration.  And reuse the idle/inactive measurement report mechanisms.

However, opponents argue that early measurement may bring power consumption. Considering this issue, we propose to introduce an additional indication per carrier frequency to indicate whether the carrier frequency can be used for EPS Fallback since the legacy carrier for idle/inactive measurement in RRCRelease and SIB11 may be unsuitable for EPS/RAT fallback. In this case, the UE which has the corresponding capability can perform early measurement for EPS/RAT Fallback on the carrier which is indicated to be used for EPS/RAT fallback.

Furthermore, when UE starts to perform early measurement for EPS/RAT Fallback can be left to UE implementation, i.e., not follow the measIdleDuration. Considering the measIdleDuration is mandatory in dedicated message, early measurement for EPS/RAT Fallback will only follow the configuration in SIB.

Note: 	activation of the measurements in the UE could be linked to e.g. the opening of the phone application on the UE originating the call, or, reception of paging on the UE that (might) receive a call. Besides, the UE starts to perform measurements can be the time when UE receives the configuration in SIB if the UE thinks itself is not sensitive to power. Release 17 features such as MUSIM-paging cause and PEI-CN based subgroup could further optimise UE power consumption on the mobile terminating side.

Proposal 1: RAN2 can agree with the enhancement on EPS/RAT Fallback:

- Introducing indications on carrier frequency for idle/inactive measurement for EPS Fallback to indicate whether the carrier can be used for EPS Fallback.

- When UE starts to perform early measurement for EPS/RAT Fallback can be left to UE implementation.

- Early measurement for EPS/RAT Fallback will only follow the configuration in SIB11.

  - Analysis on measurements for EPS/RAT Fallback

2.3.1 The measurement accuracy of early measurement

Opponents have concerns on the accuracy of early measurements for EPS/RAT Fallback. To evaluate the performance of early measurement for EPS/RAT Fallback based on handover, we have checked RAN 4 specification. Besides, we designed two related simulations.

RAN4 measurement accuracy

According to chapter 4.4.2.3 measurement of E-UTRAN inter-RAT DC candidate cells in TS 38.133, which is as follows:

the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in clauses in 9.1.3B.3 and 9.1.6B.2 [15], respectively.

It can be observed that the legacy idle/inactive measurement has the measurement accuracy requirements, which can be 6 dB in the normal condition.  In the early version e.g. TS 36.133-am0, the accuracy requirement for inter-frequency is 6 dB also. Hence, we think although the accuracy requirement of idle/inactive is relaxed compared with handover, it won’t bring severe impact.

Observation 3:  The accuracy of EPS/RAT Fallback and load distribution based on early measurement can be guaranteed from the technical analysis perspective.

Proposal 2: The accuracy requirements of early measurement for EPS Fallback can re-use R16 EMR requirements defined in RAN4.

Simulations results on handover based on early measurement

To evaluate the performance of early measurement, we compare the measurement results and handover failure rate of the following two cases:

- Case1: the enhancement method: UE performs EPS/RAT Fallback based on handover which according to the early measurement results in idle/inactive and UE doesn’t have the capability that performs measurement when initiating RACH

- Case2: legacy method: UE performs EPS/RAT Fallback based on the handover which according to the connected measurement results in connected.

The simulation parameters are given in Table 1.

Table 1 Simulation Assumptions for early measurement for EPS/RAT Fallback

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | NR：4 GHz LTE：2 GHz |
| TRP Number | Both LTE and NR：7 sites, 3 sector per site |
| Bandwidth | Both LTE and NR：20MHz |
| Subcarrier Spacing | 15kHz |
| Channel Model | 3D-Uma in TR38.901 |
| BS antenna element radiation pattern | Antenna_pattern_dir_bs_3sector |
| UE antenna element radiation pattern | Omnidirectional |
| ISD | 200m |
| BS Tx power | 44dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2) (dH,dV) = (0.5, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 15 degree |
| UE Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1;1,1) The polarization angles are 0 and 90 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-Uma in TR38.901 |
| Noise figure for BS | 5dB |
| UE receiver noise figure | 7dB |
| UE distribution | 100% Outdoor in cars: 100 users per TRP |
| ue mobility | 3km/h, 30km/h, 60km/h, 120km/h |
| Mobility management | L1 measurement period: 40ms Handover execution time: 40ms |
| RLM | L1 measurement period: 10ms Qout :-8dB |
| Handover failed decision | The average SINR of UE in target cell handover execution time is lower than that of Qout |

For EPS Fallback based on early measurement, assuming the DRX cycle of a UE is 320 ms, then UE at least measures the E-UTRAN carrier twice in 1.28s according to TS 38.133, and UE handover based on the early measurement results.

For the legacy EPS Fallback based on Handover, assuming the measurement gap is 40 ms in RRC_CONNCTED, then UE measures the E-UTRAN carrier every 40ms, and UE handover based on the connected measurement results.

We assume 7 candidate neighbour cells around the UE, and UE has the chance to measure all neighbour cells in above cases. Figure1 shows the comparison of RSRP and Geometry SINR results of the selected cell between the two cases.

Note:  in the figure, early measurement represents that UE handover to E-UTRAN cells based on early measurement; connected measurement represents that UE handover to E-UTRAN cells based on legacy connected measurement.

Figure1: measurement results of the selected cell in the two cases

From figure1, it seems that the measurement results of early measurement are same as the connected measurement when UE’s speed is 3km/h, hence we think the measurement results of early measurement is reliable.

Observation 4: The accuracy of EPS/RAT Fallback based on early measurement is very close to the connected measurement from the simulation.

According to [1], there is a 200ms delay between UE receiving paging and entering RRC_CONNECTED, assuming the UE doesn’t have the capability that perform measurement while initiating RACH, then the early measurement will be almost 200ms earlier than handover.

To verify whether the 200ms has severe impact, we simulated four scenarios in which the speed of UE is different, and the failure rate on handover is as below:

Table 2 The failure rate of handover in different speed

| Scheme UE speed | 3km/h | 30km/h | 60km/h | 120km/h |
| --- | --- | --- | --- | --- |
| Legacy method | 6.7% | 6.8% | 6.8% | 6.9% |
| Early measurement while UE doesn't have capability that performs measurement while initiating RACH | 7.4% | 7.5% | 9.1% | 18.4% |
| Early measurement while UE has capability that performs measurement while initiating RACH | 6.7% | 6.8% | 6.8% | 6.9% |

From the first two results in table 2, we can see the handover failure rate increases when UE’s speed grows, however, when UE’s speed is not too fast, the failure rate closes to legacy handover.

Furthermore, when UE has the capability that performs measurement while initiating RACH, then the handover failure rate will not be affected compared to connected measurement, since the early measurement is the latest considering UE may continually update the measurement result when performing RACH.

Observation 5: If UE doesn’t have the capability that performs measurement while initiating RACH, the failure rate of handover based on early measurement will be affected, but close to normal handover if UE’ speed is not too fast, e.g. 3 km/h, 30km/h.

Observation 6: If UE has the capability that performs measurement while accessing the Network, the failure rate of handover based on early measurement may not be affected compared to normal handover.

2.3.2 The reduction of delay for EPS/RAT Fallback

In [1], we have shown the voice set up procedure via EPS Fallback, and the following table shows the rough time of every step in the procedure and how much time the enhancement can save in each step.

Table 3: latency reduction of voice setup from the calling party perspective

| Step | Legacy procedure | Time reduction for early measurement |
| --- | --- | --- |
| UE triggers RRC setup/resume connection | 0ms | 0ms |
| Idle/inactive UE setup/resume connection with gNB and gNB triggers EPS/Fallback | ~250ms | 0ms |
| UE receive measurement configuration; UE performs measurement and reports the results in RRC_CONNECTED. | ~600ms | ~600ms |
| Handover/redirection and TAU | ~300ms for handover procedure+TAU | 0 ms |
|  | ~1300ms for redirection procedure+TAU |  |
| Wait for the caller response and finish the procedure assuming caller using handover for EPS Fallback | ~1s when caller and callee perform EPS fallback in parallel | ~600 ms |
|  | ~2s when caller and callee perform EPS fallback in series |  |

Table 5 gives four cases of EPS Fallback and the total time of the legacy procedure and enhancements. It can be seen that the early measurement can significantly save time, almost from 28.9% to 55.8%.

Table 4: Total time reduction of voice setup from the calling party perspective

| cases |  | Legacy procedure | Early measurement |  |
| --- | --- | --- | --- | --- |
|  |  |  | Total Time | gain |
| Total time | caller and callee perform EPS fallback in parallel and callee performs EPS Fallback based on handover | 2.15s | 0.95s | 55.8% |
|  | caller and callee perform EPS fallback in parallel and callee performs EPS Fallback based on redirection | 3.15s | 1.95s | 38.1% |
|  | caller and callee perform EPS fallback in series and callee performs EPS Fallback based on handover | 3.15s | 1.95 s | 38.1% |
|  | caller and callee perform EPS fallback in series and callee performs EPS Fallback based on redirection | 4.15s | 2.95 s | 28.9% |

Observation 7:  The early measurement enhancement can help reduce latency significantly, almost saving 28.9% to 55.8% time.

- Conclusion

In this paper, we discuss the latency of EPS/RAT Fallback. Based on the discussion, we have the following observations and proposals:

Observation 1: There are three main aspects to impact the latency of EPS fallback

- Measurement configuration and report, i.e., radio quality and Number of LTE frequency

- EPS fallback methods, i.e. handover or redirection,

- Procedure of EPS fallback in parallel or in series for caller and callee.

Observation 2: The LTE frequency measurement configuration is only done after RRC connection setup, UE may not report measurement results in time due to radio quality and the Number of LTE frequencies, which leads to blind redirection procedure and results in a longer delay.

Observation 3:  The accuracy of EPS/RAT Fallback and load distribution based on early measurement can be guaranteed from the technical analysis perspective.

Observation 4: The accuracy of EPS/RAT Fallback based on early measurement is very close to the connected measurement from the simulation.

Observation 5: If UE doesn’t have the capability that performs measurement while initiate RACH, the failure rate of handover based on early measurement will be affected, but close to normal handover if UE’ speed is not too fast, e.g. 3 km/h, 30km/h.

Observation 6: If UE has the capability that perform measurement while access the Network, the failure rate of handover based on early measurement may be not affected compared to normal handover.

Observation 7:  The early measurement enhancement can help reduce latency significantly, almost saving 28.9% to 55.8% time.

Proposal 1: RAN2 can agree with the enhancement on EPS/RAT Fallback:

- Introducing indications on carrier frequency for idle/inactive measurement for EPS Fallback to indicate whether the carrier can be used for EPS Fallback.

- When UE starts to perform early measurement for EPS/RAT Fallback can be left to UE implementation.

- Early measurement for EPS/RAT Fallback will only follow the configuration in SIB11.

Proposal 2: The accuracy requirements of early measurement for EPS Fallback can re-use R16 EMR requirements defined in RAN4.

- Reference

- R2-2108670	Reduce the blind redirection for EPS Fallback vivo, China Telecom, CMCC, China Unicom	discussion	Rel-17

- R2-2107259	 Discussion on Idle/Inactive Measurement for Load Distribution NTT DOCOMO INC.	discussion

- R2-2111091	Early measurement for EPS Fallback and Load Distribution	vivo, China Telecom, CMCC, SoftBank, NTT DOCOMO INC, China Unicom, Ericsson, Vodafone	discussion	Rel-17	TEI17

- R2-2110485	EPS fallback enhancements for UEs in IDLE/INACTIVE	Huawei, HiSilicon, CMCC, China Telecom, China Unicom, LG Uplus	discussion	Rel-17	TEI17

- R2-2111092	38331 CR for Early measurement for EPS Fallback and Load Distribution	vivo, China Telecom, CMCC, SoftBank, NTT DOCOMO INC, China Unicom, Ericsson Vodafone	 CR	Rel-17	38.331	16.6.0	2861	-	B	TEI17