3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2202884

Elbonia, 21 February – 03 March 2022

Agenda item:	8.24.1

Source:	Nokia, Nokia Shanghai Bell

Title:	PUCCH group definition

WID/SID:	NR_RRM_enh2-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

The issue on PUCCH SCell activation was discussed via email discussion [AT116bis-e][033][NR17] (Huawei) in the previous meeting with the following proposals proposed. It was not treated due to lack of time.

Proposal 1: RAN2 agree to introduce the new UE capability indicated in RAN1 LS. The capability attributes (e.g. definition, release, granularity, optional/conditional mandatory, e.t.c.) are up to RAN1 decision. Proposal 2: RAN2 can discuss whether/how to update the stage 2 description of PUCCH group in maintenance. Proposal 3: RAN2 confirm that the existing RRC signalling is enough to configure the CSI reporting indicated in RAN1 LS. Proposal 4: RAN2 send LS to RAN1/RAN4 with the RAN2 agreements related to P1 and P3.

We discuss the issue of update the stage 2 description of PUCCH group.

# 2	Discussion

PUCCH group is currently defined as following (note they are only in LTE stage 2, but not in NR):

Primary PUCCH group: a group of serving cells including PCell whose PUCCH signalling is associated with the PUCCH on PCell. PUCCH group: either primary PUCCH group or a secondary PUCCH group. Secondary PUCCH group: a group of SCells whose PUCCH signalling is associated with the PUCCH on the PUCCH SCell.

With the possibility of cross-PUCCH group CSI reporting, it was proposed to change the PUCCH group defining. Two options were proposed during the email discussion, either to limit the PUCCH group to HARQ feedback or to capture the exceptional case for PUCCH SCell activation. Another option would of course be to keep the definition as it is.

-	Option 1: keep the PUCCH group definition as it is;

-	Option 2: change the definition as only for HARQ feedback;

Primary PUCCH group: a group of serving cells including PCell whose HARQ feedback is associated with the PUCCH on PCell. Secondary PUCCH group: a group of SCells whose HARQ feedback is associated with the PUCCH on the PUCCH SCell.

-	Option 3: add exceptional case for PUCCH SCell activation;

Primary PUCCH group: a group of serving cells including PCell whose PUCCH signalling is associated with the PUCCH on PCell. Secondary PUCCH group: a group of SCells whose PUCCH signalling is associated with the PUCCH on the PUCCH SCell, expect for PUCCH SCell for which CSI may be reported via an active serving cell in the Primary PUCCH group during the PUCCH SCell activation phase [reference to RAN4 spec].

Option 2 of limiting the PUCCH group to HARQ feedback would be against the intention of PUCCH group since it has been for PUCCH resources/signalling in general.

We understood the intention for cross-PUCCH group CSI reporting was only to address the issue of beam information reporting for the PUCCH SCell when it is activated. To do that, the NW could either configure PUCCH of the PUCCH SCell on PCell for CSI reporting before it is activated, which means PUCCH SCell is temporarily on the primary PUCCH grant, thus does not really violate the current definition, or request aperiodic CSI reporting for the PUCCH SCell via PUSCH, which does not impact the current definition either since it is not PUCCH signalling. Thus we do not see any problem with current definition.

Proposal: no need to change current PUCCH group definition.

# 4	Conclusion

PUCCH group definition impacted by cross group CSI reporting for PUCCH SCell activation is discussed in this contribution with the following proposal proposed:

Proposal: no need to change current PUCCH group definition.

# References

- R2-2201933, Summary of [AT116bis-e][033][NR17] (Huawei), Huawei