3GPP TSG-RAN2 Meeting #117 electronic	R2-2202885

Online, Feb21st - Mar 3rd, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the MUSIM UE capability reporting

Agenda item:		8.3.5

Document for: 	Discussion and Decision

- Introduction

About the UE capabilities, in the last meeting it has been agreed that:

1: Two capability bits are introduced, one for support gaps and another for “leaving connected”. There is no need for different capability bits for periodic and aperiodic gaps. 2: Confirm that the MUSIM related capability is per UE (without FRx and xDD differentiation).

Three running CRs [1] [2][3] were also provided based on these two agreements. Meanwhile papers are also required to discuss below issue in [4].

Whether to indicate which MUSIM gap patterns are supported by UE (similar with UE capability supportedGapPattern in 38.306 ) based CR in RAN4 R4-2202760[5]

In this paper, we give our views on this issue.

- Discussion

For the measurement Gap configuration, the UE would report the supported gap patterns to the Network.

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

| supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE for NR SA, for NR-DC, for NE-DC and for independent measurement gap configuration on FR2 in (NG)EN-DC. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2. | UE | CY | No | No |
| --- | --- | --- | --- | --- |

Now for the MUSIM, as endorsed in [5], some Gap patterns are also defined:

Table 9.1.2x-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) | MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 14 | 6 | 320 |
| 1 | 6 | 80 | 15 | 6 | 640 |
| 2 | 3 | 40 | 16 | 6 | 1280 |
| 3 | 3 | 80 | 17 | 6 | 2560 |
| 4 | 6 | 20 | 18 | 10 | 320 |
| 5 | 6 | 160 | 19 | 10 | 640 |
| 6 | 4 | 20 | 20 | 10 | 1280 |
| 7 | 4 | 40 | 21 | 10 | 2560 |
| 8 | 4 | 80 | 22 | 20 | 320 |
| 9 | 4 | 160 | 23 | 20 | 640 |
| 10 | 3 | 20 | 24 | 20 | 1280 |
| 11 | 3 | 160 | 25 | 20 | 2560 |
| 12 | 10 | 80 | [26] | 20 | 5120 |
| 13 | 20 | 160 | 27 | 20 | NA |
| Note 1: Measurement gap pattern #27 is the aperiodic gap pattern without MGRP. |  |  |  |  |  |

Similar to the measurement Gap, the UE can also report the supported Gap patterns to the network for the scheduling Gap configuration.

Proposal 1: UE report the supported MUSIM gap pattern information to the network.

Furthermore, in the last meeting, it has been agreed that there is no need for different capability bits for periodic and aperiodic gaps.

=>There is no need for different capability bits for periodic and aperiodic gaps.

However in the RAN4 spec, the different Gap patterns are defined for both periodic and aperiodic Gaps. Then there are two options:

- Option 1: The UE report both periodic Gap and aperiodic Gap patterns.

- Option 2: The UE only report the periodic Gap patterns, and the network will take the aperiodic gap patterns with the same gap length (as the periodic Gap) as supported. E.g. once the UE reports one of periodic Gap pattern (13/22~26), the network will also take the gap pattern 27 as supported for that they share the same gap length.

Both options can work, considering that the option 2 is more bits-saving and is also aligned with RAN2’s agreements, the option 2 is slightly preferred.

Proposal 1a: UE only report the periodic Gap patterns, for the supported periodic Gap, the UE shall also support the aperiodic Gap that with the same gap length as the supported periodic Gap.

Based on the proposal 1/1a, the related TP was also provided as shown in Annex.

- Conclusion

With the above analysis, we have the following proposals:

Proposal 1: UE report the supported MUSIM gap pattern information to the network.

Proposal 1a: UE only report the periodic Gap patterns, for the supported periodic Gap, the UE shall also support the aperiodic Gap that with the same gap length as the supported periodic Gap.

- Reference

[1] R2-2202008    Running CR to 38.822 for MUSIM UE Capabilities   38.822  16.2.0

[2] R2-2202010    Running CR to 38.306 for MUSIM UE Capabilities   38306 16.7.0

[3] R2-2202009    Running CR to 38.331 for MUSIM UE Capabilities   38331  16.7.0

[4] R2-2201999   [Post116bis-e][202][MUSIM] Open issues for MUSIM Vivo

[5] R4-2202760   draftCR on new MGPs for MUSIM  38.133 17.4.0 Endorsed

- Annex

    - Annex 1: Text proposal for38.822 (R2-2202008)

    - 5.2.X	LTE_NR_MUSIM-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X. LTE_NR_MUSIM-Core | X-1 | MUSIM NW Switching- MUSIM Gap request | Indicates whether the UE supports providing MUSIM assistance information with MUSIM gap preference as defined in TS 38.331. |  | musimGapPreference-r17 | UE-NR-Capability-v17xy | No | No |  | Optional with capability signalling |
|  | X-2 | MUSIM NW Switching-leaving RRC_CONNECTED state | Indicates the supported MUSIM gap patterns. Only supported periodic Gap patterns are included, for the supported periodic Gap, the UE shall also support the aperiodic Gap that with the same gap length as the supported periodic Gap. If present, it also indicates that the UE supports providing MUSIM assistance information with indication of leaving RRC_CONNECTED state as defined in TS 38.331. |  | musimLeaveConnected-r17 | UE-NR-Capability-v17xy | No | No |  | Optional with capability signalling |

    - Annex 2: Text proposal for 38.306 based on the running CR (R2-2202010)

    - 4.2.2	General parameters

| musimGapPreference-r17 Indicates the supported MUSIM gap patterns. Only supported periodic Gap patterns are included, for the supported periodic Gap, the UE shall also support the aperiodic Gap that with the same gap length as the supported periodic Gap. If present, it also indicates that the UE supports providing MUSIM assistance information with MUSIM gap preference as defined in TS 38.331 [9]. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| musimLeaveConnected-r17 Indicates whether the UE supports providing MUSIM assistance information with indication of leaving RRC_CONNECTED state as defined in TS 38.331 [9]. | UE | No | No | No |

    - Annex 3: Text proposal for 38.331 based on the running CR (R2-2202009)

#### –	UE-NR-Capability

The IE UE-NR-Capability is used to convey the NR UE Radio Access Capability Parameters, see TS 38.306 [26].

UE-NR-Capability information element

-- ASN1START

-- TAG-UE-NR-CAPABILITY-START

UE-NR-Capability ::=            SEQUENCE {

accessStratumRelease            AccessStratumRelease,

pdcp-Parameters                 PDCP-Parameters,

rlc-Parameters                  RLC-Parameters                                                        OPTIONAL,

mac-Parameters                  MAC-Parameters                                                        OPTIONAL,

phy-Parameters                  Phy-Parameters,

rf-Parameters                   RF-Parameters,

measAndMobParameters            MeasAndMobParameters                                                  OPTIONAL,

fdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

tdd-Add-UE-NR-Capabilities      UE-NR-CapabilityAddXDD-Mode                                           OPTIONAL,

fr1-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

fr2-Add-UE-NR-Capabilities      UE-NR-CapabilityAddFRX-Mode                                           OPTIONAL,

featureSets                     FeatureSets                                                           OPTIONAL,

featureSetCombinations          SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination         OPTIONAL,

lateNonCriticalExtension        OCTET STRING (CONTAINING UE-NR-Capability-v15c0)                      OPTIONAL,

nonCriticalExtension            UE-NR-Capability-v1530                                                OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1530 ::=               SEQUENCE {

fdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

tdd-Add-UE-NR-Capabilities-v1530         UE-NR-CapabilityAddXDD-Mode-v1530                            OPTIONAL,

dummy                                    ENUMERATED {supported}                                       OPTIONAL,

interRAT-Parameters                      InterRAT-Parameters                                          OPTIONAL,

inactiveState                            ENUMERATED {supported}                                       OPTIONAL,

delayBudgetReporting                     ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1540                                       OPTIONAL

}

UE-NR-Capability-v1540 ::=              SEQUENCE {

sdap-Parameters                         SDAP-Parameters                                               OPTIONAL,

overheatingInd                          ENUMERATED {supported}                                        OPTIONAL,

ims-Parameters                          IMS-Parameters                                                OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1540        UE-NR-CapabilityAddFRX-Mode-v1540                             OPTIONAL,

fr1-fr2-Add-UE-NR-Capabilities          UE-NR-CapabilityAddFRX-Mode                                   OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1550                                        OPTIONAL

}

UE-NR-Capability-v1550 ::=               SEQUENCE {

reducedCP-Latency                        ENUMERATED {supported}                                       OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1560                                       OPTIONAL

}

UE-NR-Capability-v1560 ::=               SEQUENCE {

nrdc-Parameters                         NRDC-Parameters                                               OPTIONAL,

receivedFilters                         OCTET STRING (CONTAINING UECapabilityEnquiry-v1560-IEs)       OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1570                                        OPTIONAL

}

UE-NR-Capability-v1570 ::=               SEQUENCE {

nrdc-Parameters-v1570                   NRDC-Parameters-v1570                                         OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1610                                        OPTIONAL

}

-- Late non-critical extensions:

UE-NR-Capability-v15c0 ::=               SEQUENCE {

nrdc-Parameters-v15c0                    NRDC-Parameters-v15c0                                        OPTIONAL,

partialFR2-FallbackRX-Req                ENUMERATED {true}                                            OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v15g0                                       OPTIONAL

}

UE-NR-Capability-v15g0 ::=               SEQUENCE {

rf-Parameters-v15g0                      RF-Parameters-v15g0                                          OPTIONAL,

nonCriticalExtension                     SEQUENCE {}                                                  OPTIONAL

}

-- Regular non-critical extensions:

UE-NR-Capability-v1610 ::=               SEQUENCE {

inDeviceCoexInd-r16                     ENUMERATED {supported}                                        OPTIONAL,

dl-DedicatedMessageSegmentation-r16     ENUMERATED {supported}                                        OPTIONAL,

nrdc-Parameters-v1610                   NRDC-Parameters-v1610                                         OPTIONAL,

powSav-Parameters-r16                   PowSav-Parameters-r16                                         OPTIONAL,

fr1-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

fr2-Add-UE-NR-Capabilities-v1610        UE-NR-CapabilityAddFRX-Mode-v1610                             OPTIONAL,

bh-RLF-Indication-r16                   ENUMERATED {supported}                                        OPTIONAL,

directSN-AdditionFirstRRC-IAB-r16       ENUMERATED {supported}                                        OPTIONAL,

bap-Parameters-r16                      BAP-Parameters-r16                                            OPTIONAL,

referenceTimeProvision-r16              ENUMERATED {supported}                                        OPTIONAL,

sidelinkParameters-r16                  SidelinkParameters-r16                                        OPTIONAL,

highSpeedParameters-r16                 HighSpeedParameters-r16                                       OPTIONAL,

mac-Parameters-v1610                    MAC-Parameters-v1610                                          OPTIONAL,

mcgRLF-RecoveryViaSCG-r16               ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredMCG-SCells-r16          ENUMERATED {supported}                                        OPTIONAL,

resumeWithStoredSCG-r16                 ENUMERATED {supported}                                        OPTIONAL,

resumeWithSCG-Config-r16                ENUMERATED {supported}                                        OPTIONAL,

ue-BasedPerfMeas-Parameters-r16         UE-BasedPerfMeas-Parameters-r16                               OPTIONAL,

son-Parameters-r16                      SON-Parameters-r16                                            OPTIONAL,

onDemandSIB-Connected-r16               ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1640                                        OPTIONAL

}

UE-NR-Capability-v1640 ::=               SEQUENCE {

redirectAtResumeByNAS-r16               ENUMERATED {supported}                                        OPTIONAL,

phy-ParametersSharedSpectrumChAccess-r16  Phy-ParametersSharedSpectrumChAccess-r16                    OPTIONAL,

nonCriticalExtension                    UE-NR-Capability-v1650                                        OPTIONAL

}

UE-NR-Capability-v1650 ::=               SEQUENCE {

mpsPriorityIndication-r16                ENUMERATED {supported}                                       OPTIONAL,

highSpeedParameters-v1650                HighSpeedParameters-v1650                                    OPTIONAL,

nonCriticalExtension                     UE-NR-Capability-v1700                            OPTIONAL

}

UE-NR-Capability-v1700 ::=               SEQUENCE {

supportedmusimGap-r17                   BIT STRING (SIZE (FFS))                                       OPTIONAL,

musimLeaveConnected-r17                 ENUMERATED {supported}                                        OPTIONAL,

nonCriticalExtension                    SEQUENCE {}                                                   OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode ::=         SEQUENCE {

phy-ParametersXDD-Diff                  Phy-ParametersXDD-Diff                                        OPTIONAL,

mac-ParametersXDD-Diff                  MAC-ParametersXDD-Diff                                        OPTIONAL,

measAndMobParametersXDD-Diff            MeasAndMobParametersXDD-Diff                                  OPTIONAL

}

UE-NR-CapabilityAddXDD-Mode-v1530 ::=    SEQUENCE {

eutra-ParametersXDD-Diff                 EUTRA-ParametersXDD-Diff

}

UE-NR-CapabilityAddFRX-Mode ::= SEQUENCE {

phy-ParametersFRX-Diff              Phy-ParametersFRX-Diff                                            OPTIONAL,

measAndMobParametersFRX-Diff        MeasAndMobParametersFRX-Diff                                      OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1540 ::=    SEQUENCE {

ims-ParametersFRX-Diff                   IMS-ParametersFRX-Diff                                       OPTIONAL

}

UE-NR-CapabilityAddFRX-Mode-v1610 ::=    SEQUENCE {

powSav-ParametersFRX-Diff-r16            PowSav-ParametersFRX-Diff-r16                                OPTIONAL,

mac-ParametersFRX-Diff-r16               MAC-ParametersFRX-Diff-r16                                   OPTIONAL

}

BAP-Parameters-r16 ::=                   SEQUENCE {

flowControlBH-RLC-ChannelBased-r16       ENUMERATED {supported}                                       OPTIONAL,

flowControlRouting-ID-Based-r16          ENUMERATED {supported}                                       OPTIONAL

}

-- TAG-UE-NR-CAPABILITY-STOP

-- ASN1STOP

UE-NR-Capability field descriptions

featureSetCombinations A list of FeatureSetCombination:s for supportedBandCombinationList in UE-NR-Capability. The FeatureSetDownlink:s and FeatureSetUplink:s referred to from these FeatureSetCombination:s are defined in the featureSets list in UE-NR-Capability.

UE-NR-Capability-v1540 field descriptions

fr1-fr2-Add-UE-NR-Capabilities This instance of UE-NR-CapabilityAddFRX-Mode does not include any other fields than csi-RS-IM-ReceptionForFeedback/ csi-RS-ProcFrameworkForSRS/ csi-ReportFramework.