3GPP TSG-RAN WG2 Meeting #117-e	R2-2202929

# Online, 21st Feb. – 03rd Mar. 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4766 | rev |  | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor changes collected by Rapporteur |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2021-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The changes included in this CR aim to correct minor errors in the specification. - In 5.3.16.2, some bullets with “4>” have indent error. - PUR-NRSRP-ChangeThreshold-r16 is a NB-IoT specific IE. the suffix ‘-NB’ is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR includes the following change: - In 5.3.16.2, correct the style for some bullets with “4>”. - Change PUR-NRSRP-ChangeThreshold-r16 to PUR-NRSRP-ChangeThreshold-NB-r16. Impact analysis Impacted functionality: None i.e. these minor corrections do not involve any functional changes Inter-operability: No interoperability issue (as no functional change). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miscellaneous non-controversial errors will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.16.2, 6.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.16.2	Initiation

Except for NB-IoT, upon initiation of the procedure, the UE shall:

1>	if T309 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else if timer T302 is running and the Access Category is neither '2' nor '0':

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else if SystemInformationBlockType25 is not broadcasted:

3>	consider the access attempt as allowed;

2>	else if ab-PerRSRP is included:

3>	if the establishmentCause received from higher layers is set to a value other than emergency:

4>	if ab-PerRSRP is set to thresh0:

5>	consider access to the cell as barred when in enhanced coverage as specified in TS 36.304 [4];

4>	else if ab-PerRSRP is set to thresh1:

5>	if the measured RSRP is less than the first entry in rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first CE level are configured;

4>	else if ab-PerRSRP is set to thresh2:

5>	if the measured RSRP is less than the second entry in rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first and second CE levels are configured;

4>	else if ab-PerRSRP is set to thresh3:

5>	if the measured RSRP is less than the third entry in rsrp-ThresholdsPrachInfoList:

6>	consider access to the cell as barred;

5>	else:

6>	consider that only the resources indicated for the first, second, and third CE levels are configured;

2>	if the Access Category is not '0', and SystemInformationBlockType25 is broadcasted, and access to the cell is not barred due to ab-PerRSRP:

3>	if SystemInformationBlockType25 includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 24.501 [95]):

4>	select the UAC-BarringPerPLMN entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-BarringPerPLMN entry (i.e. presence or absence of access barring parameters in this entry) irrespective of the uac-BarringForCommon included in SystemInformationBlockType25;

3>	else if SystemInformationBlockType25 includes uac-BarringForCommon:

4>	in the remainder of this procedure use the uac-BarringForCommon (i.e. presence or absence of these parameters) included in SystemInformationBlockType25;

3>	else:

4>	consider the access attempt as allowed;

3>	if uac-BarringForCommon is applicable or the uac-AC-BarringListType indicated that uac-ExplicitAC-BarringList is used:

4>	if the corresponding UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

5>	if the uac-BarringInfoSetList contain a UAC-BarringInfoSet entry corresponding to the uac-barringInfoSetIndex in the UAC-BarringPerCat:

6>	select the UAC-BarringInfoSet entry;

6>	perform access barring check for the Access Category as specified in 5.3.16.5, using the UAC-BarringInfoSet as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

4>	else:

5>	consider the access attempt as allowed;

3>	else if the uac-AC-BarringListType indicated that uac-ImplicitAC-BarringList is indicated:

4>	select the uac-BarringInfoSetIndex corresponding to the Access Category in the uac-ImplicitACBarringList;

4>	if the uac-BarringInfoSetList contain the UAC-BarringInfoSet entry corresponding to the selected uac-BarringInfoSetIndex:

5>	select the UAC-BarringInfoSet entry;

5>	perform access barring check for the Access Category as specified in 5.3.16.5, using the UAC-BarringInfoSet as "UAC barring parameter";

4>	else:

5>	consider the access attempt as allowed;

3>	else:

4>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	if timer T302 is running:

4>	if timer T309 is running for Access Category '2':

5>	inform the upper layer that access barring is applicable for all access categories except categories '0', upon which the procedure ends;

4>	else:

5>	inform the upper layer that access barring is applicable for all access categories except categories '0' and '2', upon which the procedure ends;

3>	else:

4>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends;

For NB-IoT, upon initiation of the procedure, the UE shall:

1>	if T309 is running for the Access Category:

2>	consider the access attempt as barred;

1>	else:

2>	if the Access Category is '0':

3>	consider the access attempt as allowed;

2>	else if ab-Barring-5GC in MasterInformationBlock-NB / MasterInformationBlock-TDD-NB is set to FALSE:

3>	consider the access attempt as allowed;

2>	else:

3>	if SystemInformationBlockType14-NB includes uac-BarringCommon:

4>	in the remainder of this procedure, use the UAC-BarringCommon as UAC-Barring;

3>	else if SystemInformationBlockType14-NB includes uac-BarringPerPLMN-List and the uac-BarringPerPLMN-List contains an UAC-Barring entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers (see TS 24.501 [95]):

4>	select the UAC-Barring entry with the plmn-IdentityIndex corresponding to the PLMN selected by upper layers;

4>	in the remainder of this procedure, use the selected UAC-Barring entry as UAC-Barring;

3>	else:

4>	consider the access attempt as allowed;

3>	if UAC-Barring is applicable:

4>	if one or more Access Identities are indicated according to TS 24.501 [95]; and

4>	if for at least one of these Access Identities the corresponding bit in the uac-BarringForAccessIdentity is set to zero:

5>	consider the access attempt as allowed;

4>	else if the UAC-BarringPerCatList contains a UAC-BarringPerCat entry corresponding to the Access Category:

5>	select the UAC-BarringPerCat entry;

6>	perform access barring check for the Access Category as specified in 5.3.16.5, using the uac-BarringForAccessIdentity and the UAC-BarringPetCat entry as "UAC barring parameter";

5>	else:

6>	consider the access attempt as allowed;

1>	if the access barring check was requested by upper layers:

2>	if the access attempt is considered as barred:

3>	inform upper layers that the access attempt for the Access Category is barred, upon which the procedure ends;

2>	else:

3>	inform upper layers that the access attempt for the Access Category is allowed, upon which the procedure ends;

1>	else:

2>	the procedure ends;

#### 6.7.3.2	NB-IoT Radio resource control information elements

#### –	PUR-Config-NB

The IE PUR-Config-NB is used to specify PUR configuration.

PUR-Config-NB information element

-- ASN1START

PUR-Config-NB-r16	::=				SEQUENCE {

pur-ConfigID-r16					PUR-ConfigID-NB-r16				OPTIONAL,	--Need OR

pur-TimeAlignmentTimer-r16			INTEGER (1..8)				OPTIONAL,	--Need OR

pur-NRSRP-ChangeThreshold-r16		SetupRelease {PUR-NRSRP-ChangeThreshold-NB-r16}

OPTIONAL,	--Need ON

pur-ImplicitReleaseAfter-r16		ENUMERATED {n2, n4, n8, spare}	OPTIONAL,	--Need OR

pur-RNTI-r16						C-RNTI							OPTIONAL,	--Need ON

pur-ResponseWindowTimer-r16			ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64}

OPTIONAL,	--Need ON

pur-StartTimeParameters-r16			SEQUENCE {

periodicityAndOffset-r16			PUR-PeriodicityAndOffset-NB-r16,

startSFN-r16						INTEGER (0..1023),

startSubframe-r16					INTEGER (0..9),

hsfn-LSB-Info-r16					BIT STRING (SIZE(1))

}																	OPTIONAL,	--Need ON

pur-NumOccasions-r16				ENUMERATED {one, infinite},

pur-PhysicalConfig-r16				SEQUENCE {

carrierConfig-r16					CarrierConfigDedicated-NB-r13,

npusch-NumRUsIndex-r16				INTEGER (0..7),

npusch-NumRepetitionsIndex-r16		INTEGER (0..7),

npusch-SubCarrierSetIndex-r16		CHOICE {

khz15								INTEGER (0..18),

khz3dot75							INTEGER (0..47)

},

npusch-MCS-r16						CHOICE {

singleTone							INTEGER (0..10),

multiTone							INTEGER (0..13)

},

p0-UE-NPUSCH-r16					INTEGER (-8..7),

alpha-r16							ENUMERATED {al0, al04, al05, al06,

al07, al08, al09, al1},

npusch-CyclicShift-r16				ENUMERATED {n0, n6},

npdcch-Config-r16					NPDCCH-ConfigDedicated-NB-r13

}	OPTIONAL,	-- Need ON

...,

[[

pur-PhysicalConfig-v1650			SEQUENCE {

ack-NACK-NumRepetitions-r16		ACK-NACK-NumRepetitions-NB-r13

}														OPTIONAL	--Need ON

]]

}

PUR-NRSRP-ChangeThreshold-NB-r16 ::=	SEQUENCE {

increaseThresh-r16						NRSRP-ChangeThresh-NB-r16,

decreaseThresh-r16						NRSRP-ChangeThresh-NB-r16	OPTIONAL	--Need OP

}

NRSRP-ChangeThresh-NB-r16 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

-- ASN1STOP

PUR-Config-NB field descriptions

ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured via pur-Config, the value of ack-NACK-NumRepetitions used for HARQ response to NPDSCH containing this RRCConnectionRelease-NB message applies.

alpha Parameter: αc(3). See TS 36.213 [23], clause 16.2.1.1.1.

carrierConfig Carrier used for PUR.

hsfn-LSB-Info LSB of the H-SFN corresponding to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config.

npdcch-Config NPDCCH configuration for PUR.

npusch-CyclicShift Parameter: ncs. See TS 36.211 [21], clause 10.1.4.1.2. Value n0 corresponds to value 0 and value n6 corresponds to value 6.

npusch-MCS Index to tables specified in TS 36.213 [23], Table 16.5.1.2-1 and Table 16.5.1.2-2 for single tone and multi tone respectively, that defines modulation and TBS index for NPUSCH for PUR.

npusch-NumRepetitionsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-3, that defines number of repetitions for NPUSCH for PUR.

npusch-NumRUsIndex Index to a table specified in TS 36.213 [23], Table 16.5.1.1-2, that defines number of resource units for NPUSCH for PUR.

npusch-SubCarrierSetIndex For NPUSCH transmission with subcarrier spacing 3.75 kHz, indicates the subcarrier used for PUR specified in TS 36.213 [23]. For NPUSCH transmission with subcarrier spacing 15 kHz, index to a table specified in TS 36.213 [23], Table 16.5.1.1-1, that defines the set of subcarriers for NPUSCH for PUR.

p0-UE-NPUSCH Parameter: . See TS 36.213 [23], clause 16.2.1.1.1, unit dB.

pur-ImplicitReleaseAfter Number of consecutive PUR occasions that can be skipped before implicit release of PUR configuration. Value n2 corresponds to 2 PUR occasions, value n4 corresponds to 4 PUR occasions, and so on.

pur-NRSRP-ChangeThreshold Threshold(s) of change in serving cell NRSRP in dB for TA validation. Value dB4 corresponds to 4 dB, value dB6 corresponds to 6 dB, and so on. When pur-NRSRP-ChangeThreshold is set to setup, if decreaseThrsh is absent the value of increaseThresh is also used for decreaseThresh.

pur-NumOccasions Number of PUR occasions. Value one corresponds to 1 PUR occasion, and value infinite corresponds to an infinite number of PUR occasions.

pur-PeriodicityAndOffset Indicates the periodicity for the PUR occasions and time offset until the first PUR occasion.

pur-ResponseWindowTimer Duration of the PUR response window in TS 36.321 [6]. Value in PDCCH periods. Value pp2 corresponds to 2 PDDCH periods, pp3 corresponds to 3 PDCCH periods, and so on. The value considered by the UE is: pur-ResponseWindowSize = Min (signaled value x PDCCH period, 10.24s).

pur-TimeAlignmentTimer Value of the time alignment timer for PUR. Value in number of periodicity of PUR.