3GPP TSG RAN WG2 #117	R2-2203060

e-Meeting, Feb 21th– Mar 3rd, 2022

Agenda Item:	8.22.3.2.1

Source: 	Xiaomi Communications

Title:  	Discussion on Pre-configured MG

Document for:	Discussion and Decision

Introduction

A new work item “NR and MR-DC Measurement Gap Enhancements” [1] has been approved. In the WID, one of the objectives is to consider Pre-configured MG pattern(s), shown as below.

Pre-configured MG pattern(s) (fast MG configuration) [RAN4, RAN2] - RRM requirements for pre-configured MG pattern(s) [RAN4] - Study requirements of the mechanisms of activation/deactivation of MG following a DCI or timer based BWP switch, e.g., per BWP MG configuration - Specification of rules and UE behaviour for activation/deactivation of a MG following a DCI or timer based BWP switch - Define measurement period requirements with pre-configured MG pattern(s) in the presence of one or more BWP switch per measurement period - Specification of applicability of pre-configured MG pattern(s) [RAN4] - Procedures and signaling for pre-configured MG pattern(s) [RAN2] - Specification of protocol impacts of the mechanisms of activation/deactivation of MG following a DCI or timer based BWP switch, e.g., per BWP MG configuration based on RAN4 input

In this contribution, we provide our views on Pre-configured MG pattern(s).

Discussion

Open issues for pre-configured MG are [2]:

P1-1: Discuss support of case 4 where NW signals the pre-configured gap and BWP status via RRC, then UE follows BWP status to activates/deactivates gap upon BWP switching

P1-2: Support pre-configured MG under CA based on BWP switching on a single CC

For P1-1, RAN2 discussed how to active and de-active pre-configured MG and reach the following agreement:

At least case 5 is supported for pre-configured gap. FFS for case 4.

- Case 4: NW signals the pre-configured gap (A+B in Q1) via RRC, then UE follows BWP status (B) to activates/deactivates gap upon BWP switching

- Case 5: NW signals the pre-configured gap (A in Q1) via RRC, then UE determines whether the pre-configured gap should be activated or not upon BWP switching.  For example, if it is overlapped with SSB, then pre-configured gap is deactivated, otherwise it is activated.

- Reference to A and B are as follow:

- -	A) measurement gap configuration parameters such as MGRP, MGL etc

- -	B) MG status (active/de-active) per BWP

Case 4 is NW-Controlled and Case 5 is UE autonomous pre-configured MG activation/deactivation mechanism. Case 5 has been supported by RAN2 but Case 4 is FFS. In LS [3] from RAN4, NW-Controlled solution has been supported and it can be configured via RRC message only. So, RAN 2 should follow RAN4’s conclusions and support Case 4. And NW-Controlled mechanism is a simple way and it is clearer for UE and NW on the understanding of gap status.

Proposal 1: RAN2 should support NW-Controlled pre-configured MG activation/deactivation mechanism (i.e. Case 4).

For NW-Controlled mechanism, the MG status (active/de-active) for different BWP should be provided to UE via RRC signalling, and UE follows the status to activates/deactivates gap upon BWP switching. For MG status configuration, two options have been proposed in last meeting:

Option 1: The MG status (active/de-active) per BWP is included in the configuration of pre-configured MG

Option 2: The MG status (active/de-active) per BWP is included in the configuration of BWP

Because the MG status is independent from MG configuration and associated with BWP configuration. For option 1, when a configured BWP needs to be modified or removed or a new BWP needs to be added, the configuration of pre-configured MG should be updated. For option 2, even though MG has been modified, the MG status in BWP configuration may not need to be updated. So, option 2 may be better for pre-configured MG.

Proposal 2: For NW-Controlled activation/deactivation mechanism, the MG status (active/de-active) per BWP can be included in the configuration of BWP.

In last meeting, a 1 bit indication in gapConfig to indicate pre-configured measurement gap has been introduced. If both NW-Controlled and UE autonomous pre-configured MG activation/deactivation mechanism are supported, it can be determined which type mechanism will be used according to the indicator of pre-configured MG and the MG status per BWP.

If the pre-configured MG indicator is present in MG configuration but the MG status is absent in the BWP configuration, UE apply UE autonomous mechanism when the BWP becomes active BWP. If the pre-configured MG indicator is present in MG configuration and the MG status is present in the BWP configuration, UE apply NW-Controlled mechanism for the BWP. If the pre-configured MG indicator is absent, UE apply R16 legacy measurement gap configuration mechanism and ignore MG status if it configured.

Proposal 3: Whether to use NW-Controlled or UE autonomous pre-configured MG activation/deactivation mechanism can be determined by the presence/absence of pre-configured MG indicator and MG status.

For P2-2, pre-configured MG under CA based on BWP switching on a single CC has been supported by RAN4 according to RAN4 LS [4]. RAN2 also should follow RAN4’s agreement and support it.

Proposal 4: Pre-configured MG under CA based on BWP switching on a single CC should be supported by RAN2.

Conclusions

In this contribution, we provide some proposals as following:

Proposal 1: RAN2 should support NW-Controlled pre-configured MG activation/deactivation mechanism (i.e. Case 4).

Proposal 2: For NW-Controlled activation/deactivation mechanism, the MG status (active/de-active) per BWP can be included in the configuration of BWP.

Proposal 3: Whether to use NW-Controlled or UE autonomous pre-configured MG activation/deactivation mechanism can be determined by the presence/absence of pre-configured MG indicator and MG status.

Proposal 4: Pre-configured MG under CA based on BWP switching on a single CC should be supported by RAN2.

References

- RP-211591, Revised WID on NR and MR-DC measurement gap enhancements, Intel Corporation, MediaTek Inc.

- R2-2202054, [Post116bis-e][085][MGE] Open Issues, Intel Corporation

- R4-2120302, LS on R17 NR MG enhancements – Pre-configured MG, CATT, Intel Corporation.

- R4-2202615, LS on R17 NR MG enhancements – Pre-configured MG, Intel Corporation.