3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2203117

Online, February 21 – March 03, 2022

Agenda item:	8.24.1

Source: 	China Telecom, Huawei, HiSilicon

Title:	Discussion on remaining issues for UL Tx switching enhancement

WID/SID:	NR_RF_FR1_enh

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, RRC configuration and UE capability reporting to support Rel-17 UL Tx switching enhancement were discussed and many agreements were reached. Only the following remaining issues on UE capability reporting were left for further discussion.

Regarding whether switching option can be reported differently for 1T2T and 2T2T, RAN2 waits for RAN1 conclusion.

Regarding UL MIMO coherence capability reporting for Rel-17 2Tx-2Tx switching, RAN2 waits for RAN1

In this contribution, we would like to share our opinions on the above remaining issues on UE capability reporting as well as some clarifications on RRC configuration for Rel-17 UL Tx switching enhancement.

- Discussion

  - UE capability reporting

    - UL MIMO coherence capability reporting for 2Tx-2Tx switching

Regarding the UL MIMO coherence capability reporting for Rel-17 UL 2Tx-2Tx switching, the following agreements were reached in RAN2#116bis-e meeting.

Add a new per-band per BC UE capability in BandCombination-UplinkTxSwitch to indicate UL MIMO coherent capability specific for 2Tx-2Tx switching.

Regarding UL MIMO coherence capability reporting for Rel-17 2Tx-2Tx switching, RAN2 waits for RAN1

As per RAN1 discussion in RAN1#107-e meeting , the discussion scope is limited to 4Tx UL MIMO coherence with no impact on UL Tx switching, and whether to introduce a new per-FS UL MIMO coherence capability proposed by some companies mainly focuses on a more generic case, which is a different issue from the previous RAN4 discussion on UL MIMO coherence capability for UL Tx switching. Based on that, companies have common understanding that the RAN1 discussion on UL MIMO coherence capability is for non-Tx switching case and has no impact on UL MIMO coherence capability reporting for Tx switching as discussed in the last RAN2 meeting [2].

Besides, whether a new per-FS UL MIMO coherence capability for non-Tx switching case can be introduced by RAN1 is still not sure. Considering the Rel-17 tight timeline for RAN2 work, we suggest RAN2 can first discuss the detail design for this issue based on the clear agreement from RAN4, and we can certainly revisit it if needed when RAN1 makes clear conclusion on non-Tx switching case in the future.

Proposal 1: RAN2 discusses the detail design of UL-MIMO coherence capability reporting for Rel-17 2Tx-2Tx switching based on RAN4 agreement, and can revisit it if needed when RAN1 makes clear conclusion on non-Tx switching case in the future.

As discussed in the last RAN2 meeting, regarding the detail design for UL MIMO coherence capability for Rel-17 2Tx-2Tx switching, companies have different opinions on which IE shall be extended to report the UL MIMO coherence capability for Rel-17 2Tx-2Tx switching, which may depend on the RAN1 discussion for non-Tx switching. The following 2 options were raised in the discussion.

- Option 1: Extend the BandParameters in BandCombinationList

- Option 2: Introduce a new field UplinkTxSwitchingBandParameters-v17xx in BandCombination-UplinkTxSwitch-v17xx

Since the motivation of RAN4 to introduce a new UL MIMO coherence capability for UL Tx switching is to allow the UE to indicate a different UL MIMO coherence capability when UL Tx switching is configured, the capability for UL Tx switching will not be used for non-UL Tx switching case. In this sense, Option 1 may be confusing to include the UL-MIMO coherence capability for UL Tx switching in the general band parameter. Therefore, we prefer to go for Option 2 to introduce a new field UplinkTxSwitchingBandParameters-v17xx to report the UL Tx switching specific band parameters for a given band combination, which can also minimize the potential spec maintenance work in the future.

The new field UplinkTxSwitchingBandParameters-v17xx comprises of the following parameters:

- bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination.

- uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is used to report UL MIMO coherence capability for Rel-17 2Tx-2Tx switching.

An example of TP for TS 38.331 is given below:

BandCombination-UplinkTxSwitch-v17xx ::=    SEQUENCE {

supportedBandPairListNR-v17xx         SEQUENCE (SIZE (1..maxULTxSwitchingBandPairs)) OF ULTxSwitchingBandPair-v17xx         OPTIONAL,

uplinkTxSwitchingBandParametersList-r17     SEQUENCE (SIZE (1.. maxSimultaneousBands)) OF ULTxSwitchingBandParameters-r17         OPTIONAL

}

ULTxSwitchingBandParameters-r17 ::=       SEQUENCE {

bandIndex-r17                                      INTEGER(1..maxSimultaneousBands),

uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17     ENUMERATED {nonCoherent, fullCoherent}            OPTIONAL

}

Proposal 2: Introduce a new field UplinkTxSwitchingBandParameters-v17xx to report the UL Tx switching specific band parameters for a given band combination, which comprises of the following parameters:

-	bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination.

-	uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is used to report UL MIMO coherence capability for Rel-17 2Tx-2Tx switching.

The draft CRs to TS 38.306 and TS 38.331 are provided in R2-2203115 and R2-2203116 respectively to show the explicit spec changes to implement P2.

    - Whether switching option can be reported differently for 1Tx-2Tx and 2Tx-2Tx

In the baseline running CR R2-2201940, there is one FFS on switching option reported for 2Tx-2Tx switching. Companies didn’t reach consensus on how to handle the FFS in the last RAN2 meeting.

Regarding whether switching option can be reported differently for 1Tx-2Tx and 2Tx-2Tx, in our understanding, it is not necessary to introduce a new UE capability to report different switching option supported by the UE for 1Tx-2Tx and 2Tx-2Tx switching. If UE can support 2Tx-2Tx switching and Tx switching between 2 UL bands for Rel-17 Tx switching, it can easily support 1Tx-2Tx switching between 2 UL carriers for Rel-16 Tx switching for the same option. Since this issue is under discussion in RAN1, we are fine to wait for RAN1 decision.

However, considering the Rel-17 tight timeline for RAN2 work, if RAN1 can’t reach an agreement to allow different switching options reported for 1Tx-2Tx and 2Tx-2Tx in the Feb RAN1 meeting, we kindly suggest RAN2 can remove the FFS captured in the baseline running CR and we can revisit it if needed based on RAN1 conclusion in the future.

To make progress, we suggests RAN2 take the following way-forward for switching option capability reporting for 1Tx-2Tx and 2Tx-2Tx.

Proposal 3: For switching option capability reporting for 1Tx-2Tx and 2Tx-2Tx, RAN2 takes the following way-forward.

- Way-forward: Remove the sentence of “FFS: whether switching option can be reported differently for 1T2T and 2T2T” from the running CR, if RAN1 can’t reach an agreement to allow different switching options reported for 1Tx-2Tx and 2Tx-2Tx in the Feb RAN1 meeting. And RAN2 can revisit it if needed based on RAN1 conclusion in the future.

  - RRC configuration

In the last RAN2 meeting, RRC configuration to support UL Tx switching enhancement were discussed and all the open issues were fixed. Regarding the detail wording in the baseline CR R2-2201873, we think some clarifications might be needed for the field description of uplinkTxSwitchingCarrier.

For the configuration of uplinkTxSwitchingCarrier for UL Tx switching, RAN2 confirmed the following clarification.

- 1Tx-2Tx with 2 uplinks or 3 uplinks, band A (capable of 1T, 1 CC) will be configured as carrier1, band B (capable of 2T, 1CC or 2CC) will be configured as carrier2.

- 2Tx-2Tx with 2 uplinks or 3 uplinks, band A (capable of 2T, 1 CC) will be configured as carrier1, band B (capable of 2T, 1CC or 2CC) will be configured as carrier2.

Based on the current field description of uplinkTxSwitchingCarrier in the baseline running CR R2-2201873, there is no explicit description on which band can be configured as carrier1 in case of 1Tx-2Tx switching. To make it clear, we suggest adding “only the uplink carrier(s) on the band where UE supports 2-layer UL MIMO capability can be configured as carrier2” in the field description.

An example of TP for TS 38.331 is given below:

uplinkTxSwitchingCarrier Indicates that the configured carrier is carrier1 or carrier2 for dynamic uplink Tx switching, as defined in TS 38.101-1 [15] and TS 38.101-3 [34]. In case of (NG)EN-DC, network always configures the NR carrier as carrier 2. In case of inter-band UL CA or SUL, for dynamic uplink Tx switching between 2 bands with 2 uplink carriers or 3 uplink carriers as defined in TS 38.101-1 [15], network configures the uplink carrier(s) on one band as carrier1 and the uplink carrier(s) on the other band as carrier2, and only the uplink carrier(s) on the band where UE supports 2-layer UL MIMO capability can be configured as carrier2. This field is set to the same value for the carriers on the same band.

Proposal 4: Add “only the uplink carrier(s) on the band where UE supports 2-layer UL MIMO capability can be configured as carrier2” in the field description of uplinkTxSwitchingCarrier in the baseline running CR R2-2201873.

- Conclusion

In this contribution, we discuss the remaining issues on UE capability reporting as well as some clarifications on RRC configuration for Rel-17 UL Tx switching enhancement. We kindly ask RAN2 to consider the corresponding proposals listed below.

Proposal 1: RAN2 discusses the detail design of UL-MIMO coherence capability reporting for Rel-17 2Tx-2Tx switching based on RAN4 agreement, and can revisit it if needed when RAN1 makes clear conclusion on non-Tx switching case in the future.

Proposal 2: Introduce a new field UplinkTxSwitchingBandParameters-v17xx to report the UL Tx switching specific band parameters for a given band combination, which comprises of the following parameters:

-	bandIndex-r17 indicates a band on which UE supports dynamic UL Tx switching with another band in the band combination.

-	uplinkTxSwitching2T2T-PUSCH-TransCoherence-r17 is used to report UL MIMO coherence capability for Rel-17 2Tx-2Tx switching.

Proposal 3: For switching option capability reporting for 1Tx-2Tx and 2Tx-2Tx, RAN2 takes the following way-forward.

- Way-forward: Remove the sentence of “FFS: whether switching option can be reported differently for 1T2T and 2T2T” from the running CR, if RAN1 can’t reach an agreement to allow different switching options reported for 1Tx-2Tx and 2Tx-2Tx in the Feb RAN1 meeting. And RAN2 can revisit it if needed based on RAN1 conclusion in the future.

Proposal 4: Add “only the uplink carrier(s) on the band where UE supports 2-layer UL MIMO capability can be configured as carrier2” in the field description of uplinkTxSwitchingCarrier in the baseline running CR R2-2201873.

- Reference

Draft Report of 3GPP TSG RAN WG2 meeting #116bis-e

R2-2201871 	Summary of [AT116bis-e][036][NR17] UL TX switching Enh	China Telecom