3GPP TSG-RAN WG2 #117-e	Tdoc R2-2203168

Electronic meeting, Feb 21st – Mar 3th 2022	Revision of R2-2201598

Agenda Item:	8.19.2

Source:	Ericsson

Title:	Further issues on msg3 repetitions

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#116bis-e the following agreements were taken in the Coverage Enhancement WI:

Agreements:

- Non-CE to CE fallback is not supported in Rel-17

- UE cannot fallback from CFRA or 2-step RACH to CE-RACH

Agreements via email - from offline 111:

- CE-capable UEs use the legacy threshold, rsrp-ThresholdSSB-SUL, in its selection of UL carrier for RACH.

- Msg3 repetition is modelled in the same way as dynamically scheduled bundles in the MAC spec.

Working assumption:

- From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.

Agreements via email - from offline 111 - second round

- From CE’s perspective, it does not matter whether UE first selects RA type or CE when initiating a RACH procedure.

- From CE’s perspective, it is confirmed that the eligibility criteria for CE is determined before the selection of RACH partition.

- From CE’s perspective, the RSRP threshold for requesting Msg3 repetition can be configured per BWP on both NUL and SUL.

- When CE is configured in RACH partitions, the configuration granuality for the RSRP threshold for requesting Msg3 repetition should be decided by the common RACH session.

- From CE’s perspective, CE RACH can be configured with a separate RSRP threshold for SSB selection and this threshold can be configured per BWP.

- When CE is configured in RACH partitions, the configuration granularity for the RSRP threshold for SSB selection in a CE RACH procedure should be decided by the common RACH session.

Agreements:

- For now there is no consensus in RAN2 on whether to support CFRA with msg3 repetition or not, based on the assumption that RAN1 should discuss this first.

In this contribution we will continue to discuss the msg3 repetitions.

# 2 CFRA

In the RAN1#107-e meeting the following working assumption was made:

Working assumption

- support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH with repetitions.

  - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

  - No additional optimization specific for CFRA PUSCH is considered for CFRA PUSCH with repetition.

  - No additional RAN1 specification impact

This means that msg3 repetition (repetition of the third message during random access) shall be supported for CFRA. However, since CFRA covers a range of cases, we need to determine which of the CFRA cases that would apply to this case. CFRA is used for reconfiguration with sync, BFR and some more specific cases.

Where msg3 repetition could be most relevant is likely only for reconfiguration with sync, due to the fact that for BFR and the other cases, there is no third message, or so called PUSCH scheduled by RAR. This is also supported by the fact that 2-step Random access is only applicable for

CFRA for Msg3 (PUSCH scheduled by RAR) is only applicable to reconfiguration with sync.

With it being clear that CFRA for msg3 is only applicable for reconfiguration with sync, we need to look at how to achieve this. In RAN1#107-e the following agreement was made on how to signal the amount of msg3 repetitions for CBRA:

Agreement

- For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted.

  - Four candidate MCS indexes can be configured by SIB1 for Msg3 initial transmission. MCS 0~3 are applied if the configuration is absent.

  - If the four candidate repetition factors are not configured, the default values are {1, 2, 3, 4}.

This means that the UE need to determine how to interpret the RAR in case Msg3 repetitions are scheduled. For CBRA this is quite clear that the UE should interpret the RAR in a legacy fashion or potentially indicating Msg3 repetitions depending on what PRACH resources that were chosen using the configured RSRP threshold. Thus if the UE selects the resources to indicate that Msg3 repetitions are needed, then it shall be clear for the UE that the RAR should be interpreted as if Msg3 repetitions are signalled (#nrRepetitions can still be equal to 1). Similarly, if the network receives a preamble on resources that indicates that Msg3 repetitions are signalled, the network shall only reply with the suitable RAR – the PRACH resources for Msg3 repetitions essentially acts as early support of Msg3 repetitions. In the case where legacy 4-step preambles are transmitted/received it shall also be clear for the UE/network how RAR shall be interpreted.

For CBRA it is clear for the network and UE what RAR should be transmitted, received and interpreted for performing or not performing msg3 repetitions.

For CFRA the situation is different. The difference between CBRA and CFRA is that for CBRA, the UE selects the PRACH resources and will after that know how the RAR shall be interpreted. In CFRA, the UE will not select its own resources but rather be given the preamble resource. If Msg3 repetitions are introduced for CFRA, then the UE will be given specific preambles and not select on its own. If msg3 repetitions are allowed for CFRA, the UE would need to be told how to interpret Msg3.

CFRA for Msg3 (PUSCH scheduled by RAR) can be enabled by the network signalling how the UE shall interpret RAR in the CFRA/RACH-ConfigDedicated configuration.

Introduce a flag in CFRA configuration on how RAR shall be interpreted for CFRA.

We have provided an RRC example in sub-section 2.2.1. For MAC procedures we see little impact, which is due to the fact that 1) the resource selection related to RACH indication and Partitioning does not need to be changed as CFRA does not go through these procedures, 2) the interpretation of the RAR as indicating msg3 repetitions are only specified in RAN1 procedures.

While RAN1 has a working assumption, RAN1 has already started adding spec text related to PUSCH scheduled by RAR as is evident from feature list:

| 30. NR_cov_enh | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. |
| --- | --- | --- | --- |

Furthermore in R1-2200699, in the updated higher layer parameter list for coverage enhancement [1] for the parameter numberOfMsg3Repetitions the following description is stated:

The number of repetitions for PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI

Thus we think that while RAN1 has a working assumption, the specifications are already indicating support for CFRA.

RAN1 have already specified cases for repetitions for PUSCH scheduled by RAR.

## 2.1 RRC example

An RRC example of how the indication that RAR should be interpreted as a msg3 repetition RAR is shown below:

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                       OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization          OPTIONAL, -- Need N

...,

[[

ra-PrioritizationTwoStep-r16    RA-Prioritization          OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16           OPTIONAL  -- Need S

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}    OPTIONAL  -- Cond Mandatory

}                                                          OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)        OPTIONAL -- Cond Occasions

]],

[[

rar-PuschRepetition-r17            ENUMERATED {true}    OPTIONAL,   -- Need S

]]

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

CFRA field descriptions

...

rar-PuschRepetition Signals whether the UE shall interpret the RAR fields as indicating repetitions for PUSCH scheduled by RAR, see 38.213 [X]

Take the RRC excerpt as a baseline for introducing Msg3 repetitions for CFRA.

# 3 UE capabilities

RAN1 has recently provided RAN2 with the feature list for coverage enhancements. The feature list as seen in the Appendix contains 16 capabilities, where one of them is the capability for msg3 repetitions. This means that RAN2 will no longer have to decide whether to introduce a new capability for msg3 repetitions.

RAN1 has introduced a capability for msg3 repetitions.

The msg3 capability has the title “Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI”, to reflect the fact that the repetitions are applied both for msg3 as well as for PUSCH scheduled by RAR, which is RAN1-speak for CFRA msg3.

| 30. NR_cov_enh | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. |
| --- | --- | --- | --- |

To convert this to a capability in 38.306 there could be a need add some clarifications, such as need to support features such as random access group B and that it can be configured as part of a preamble group as discussed in RACH Indication and Partitioning WI.

Clarifications on capability for msg3 repetition should be introduced.

With these changes, we think that the example below can be taken as a baseline:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| msg3Repetition-r17 Indicates support of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. The UE supports the following for repetition of msg3 in the 4-step random access procedure: - Determining PRACH resource to signal msg3 repetitions; - Configuring Random access preamble group B; - Signalled as part of a preamble group; - CFRA/reconfig with sync part (as RAN2 will agree above) A UE supporting this shall also support pusch-RepetitionTypeA-r16. |  | No | No |  |

RAN2 to take capability signalling of msg3 repetitions as baseline.

# 4 Conclusion

In the previous sections we made the following observations:

Observation 1	For CBRA it is clear for the network and UE what RAR should be transmitted, received and interpreted for performing or not performing msg3 repetitions.

Observation 2	RAN1 have already specified cases for repetitions for PUSCH scheduled by RAR.

Observation 3	RAN1 has introduced a capability for msg3 repetitions.

Observation 4	Clarifications on capability for msg3 repetition should be introduced.

Based on the discussion in the previous sections we propose the following:

Proposal 1	CFRA for Msg3 (PUSCH scheduled by RAR) is only applicable to reconfiguration with sync.

Proposal 2	CFRA for Msg3 (PUSCH scheduled by RAR) can be enabled by the network signalling how the UE shall interpret RAR in the CFRA/RACH-ConfigDedicated configuration.

Proposal 3	Introduce a flag in CFRA configuration on how RAR shall be interpreted for CFRA.

Proposal 4	Take the RRC excerpt as a baseline for introducing Msg3 repetitions for CFRA.

Proposal 5	RAN2 to take capability signalling of msg3 repetitions as baseline.

# 5 References

R2-2202111, LS on updated Rel-17 NR higher-layers parameter list, RAN WG1, RAN2#117-e, February 2022