3GPP TSG-RAN WG2 Meeting #117-e                       R2-2203179

Online, Feb 21th – March 3rd, 2022

Agenda item:	8.8.2

Source:	Samsung

Title:	Remaining open points on RAN slicing

Document for:	Discussion & Decision

# Introduction

In RAN2#116bis-e, RAN2 discussed slice-based cell reselection and reached the following agreements [1]:

- Slice-based cell re-selection algorithm: Working assumption: We go with proposal A without formula, e.g. as proposed by Samsung or Apple. Exact details to be worked out for the next meeting. - Remaining details for Slice groups: No change to previous agreement that there can be different slice groups for RACH and reselection. Align with SA2 (if they tell us differently). 2.1: Among multiple TAs in the same RA, RAN2’s understanding is that the configuration on slice grouping should be homogeneous. 2.2: RAN2 assumes that for purpose of UE checking supported slices on the highest ranked cell at TA/RA boundary, gNB can provide in SIB the slice group that supported by these neighbour cells. If this conflicts with SA2, RAN2 will align with SA2. - FFS if the slice group is mapped by the mapping relationship in current RA or not. - FFS PCI list and/or TAC per slice group are provided. - FFS what is the UE behaviour if gNB doesn’t provide supported slice group info on the best ranked cell.

This contribution we focus on solving the remaining open issues needed to close the WI from RAN2 perspective (e.g. issues indicated in RAN2#116bis (above) or discussed as part of email discussion [203] in [2]).

# Discussion

## List of open issues for RAN slicing WI [2]

In the following we discuss the open issues on the list of issues for RAN slicing provided in Annex A.

OI 3.1: Option A without formula: Solution 4, all NAS-prioritised slices with frequency priorities as well as legacy frequency priorities are considered, without iteration, without formula

In [3] we provide our proposed method for realizing prioritization in Solution 4 (Option A without formula). More specifically, we illustrate that there are multiple methods for realizing prioritisation with slice priorities and frequency priorities. In our view, choosing a specific method is an implementation problem rather than a specification problem. So it is enough for the specification to define the rules for slice prioritization.

Observation 1: There are multiple methods for implementing Option A without formula.

Proposal 1: Specification needs to define the rules for prioritization and leave the issue of how the prioritization is realized to the UE implementation.

Proposal 2: RAN2 is kindly asked to agree the proposed TP for 38.304 in [3].

OI 3.2: In case prioritised slice is not supported in the highest ranked cell on the target frequency, what’s the UE behaviour, e.g., uses legacy frequency priority or recalculate frequency priority?

For OI 3.2, as mentioned in OI, there are two possible options:

- Consider only the legacy frequency priority

- Recalculate Frequency Priority.

In our understanding many companies preferred using legacy frequency priority during earlier discussions based on option-4 with formula, since it was complex to re-calculate the frequency priority with formula. But as illustrated in [3], it is relatively simpler for option-A without formula.

If the UE performs slice-based cell reselection and if the highest ranked cell, according to neighbouring cell information, does not support the highest priority slice supported by its frequency, the UE only considers the slices supported by the highest ranked cell among the NAS provided slice or slice group for slice specific prioritisation as in section 5.2.4.X [3] for this frequency, until the highest ranked cell changes or NAS provides a new set of slices or slice groups.

Proposal 3: In case prioritised slice is not supported in the highest ranked cell on the target frequency, the UE should use the slice priority of the highest priority slice supported by highest ranked cell.

Irrespective of the option chosen, if the priority reduction leads to a higher priority frequency becoming an equal priority or lower priority frequency or if an equal priority frequency becomes a lower priority frequency, the UE needs to re-evaluate the cell reselection criteria with the reduced priority before performing the cell reselection.

Proposal 4: If the priority reduction leads to a higher priority frequency becoming an equal priority or lower priority frequency or if an equal priority frequency becomes a lower priority frequency, the UE needs to re-evaluate the cell reselection criteria with the reduced priority before performing the cell reselection.

OI 3.4: After the UE fallbacks to legacy cell reselection, the next trigger of slice-based cell reselection.

In our understanding, following adoption of the proposed Option A without formula [3], the UE does not fall back to legacy cell reselection. That is, cell reselection happens as a continuous operation with legacy priorities considered as lower priority than slice specific frequency priority.

Observation 2: For the proposed solution 4 - Option A without formula [3], the UE does not fallback to legacy cell reselection, as cell reselection becomes a continuous operation with legacy frequency priorities considered as lower priorities than slice specific frequency priorities.

Proposal 5: For option A without formula, the UE does not fallback to legacy cell reselection and legacy frequency priorities are considered as lower priorities than slice specific frequency priorities during slice based cell reselection.

OI 3.5: If the UE is configured with slice based dedicated priority, but the UE cannot find a suitable cell, whether and how to fallback to legacy cell reselection.

As we explained earlier, when we use option A without formula, there is no “fallback” to legacy cell reselection and cell reselection measurements and evaluation happens continuously for slice based frequencies and legacy frequencies based on their relevant priorities.

So the question could be as below for option A without formula

-Whether the UE needs to consider legacy cell reselection priorities when this UE is configured with slice based dedicated priority

In RAN2#113bis-e, it was decided that UE is only configured with either the existing dedicated priority configuration or the slice info in RRC Release. We also note that there is a general principle for cell reselection that if priorities are provided in dedicated signaling, the UE shall ignore all the priorities provided in system information. This means that legacy cell reselection priorities should not be applicable when the UE is configured with slice based dedicated priority.

Observation 3: Based on RAN2 agreement, only one of sliceInfo or legacy dedicated priority can be configured in RRC Release. As per existing cell reselection principles, if priorities are provided in dedicated signaling, all the broadcast priorities are ignored.

But we also note that in such a case, there is no way for the network to configure inter-RAT cell reselection priorities. So it would be good to discuss whether inter-RAT cell reselection priorities can be included independently from dedicated signaling of slice based priorities.

Proposal 6: If the UE is configured with slice based dedicated priority, the UE doesn’t consider legacy NR priorities.

Proposal 7: RAN2 to discuss whether inter-RAT cell reselection priorities can be included independently from dedicated signaling of slice based priorities.

OI 3.6: Whether the inter-RAT frequency should be considered in slice-based cell reselection.

When we use option A without formula, there is no fallback and cell reselection measurements and evaluation happens continuously for slice based frequencies and legacy frequencies based on their relevant priorities. So inter-RAT frequency can also be considered during slice-based cell reselection. There will not be any associated slice for inter-RAT frequencies and they will be considered as lower priority than frequencies associated to a slice. Inter-RAT frequencies would then follow the legacy priorities

Observation 4: There will not be any associated slice for inter-RAT frequencies and they will be considered as lower priority than frequencies associated to a slice.

Proposal 8: Inter-RAT frequency can be considered in slice-based cell reselection with legacy priorities.

An associated question is how to handle the slice specific priorities from dedicated signalling after an Inter-RAT cell reselection. In legacy cell reselection, dedicated priorities are reused in other RAT also.

However, this is neither feasible nor required for slice aware cell reselection. Once the UE has performed inter-RAT cell reselection, it shouldn’t perform slice based cell reselection. We also propose that the UE should delete any dedicated slice based information once it performs inter-RAT cell reselection, as even maintaining slice information will need changes in LTE/UMTS/GSM implementation.

Proposal 9: On performing inter-RAT cell reselection, the UE deletes dedicated slice information and doesn’t perform slice aware cell reselection.

OI 3.7: The definition of slice group is FFS. A group which is associated with one or multiple slices. And a slice is associated to none or one slice group. FFS associated to multiple slice groups.

In conclusion to the report of email discussion in [4], the majority of companies (15/22) supported not changing the previous RAN2 agreement:

Slice group: A group which is associated with one or multiple slices. And a slice is associated to none or only one slice group.

The supporting argument of those companies was:

  - RAN2 has agreed this in previous meeting, if we are to change it, more issues have to be considered. And companies don’t see clear reason to allow to associate a slice with multiple slice groups.

Observation 5: no clear reason for changing RAN2 agreement so that a slice could be associated to more than one slice groups.

Proposal 10: RAN2 to keep its previous agreement on slice group definition “A group which is associated with one or multiple slices. And a slice is associated to none or only one slice group.”

OI 3.9: Whether to confirm the granularities of the slice groups for cell reselection are per TA. Whether AS is aware of the TA or TAs where a specific slice group is used

Regarding this open point, in our understanding, there has been no progress in SA2 on the topic of slice group. However, SA2 may discuss the topic this week, so we prefer to wait for the outcome of SA2 discussion before confirming slice group granularity for cell reselection.

Proposal 11: RAN2 to postpone discussion on the slice group granularity pending further input from SA2.

# Conclusion

This contribution we focused on solving the remaining open issues needed to close the WI from RAN2 perspective (e.g. issues indicated in RAN2#116bis (above) or discussed as part of email discussion [203] in [2]). The following are the observations and proposals in this contribution:

Observation 1: There are multiple methods for implementing Option A without formula.

Observation 2: For the proposed solution 4 - Option A without formula [3], the UE does not fallback to legacy cell reselection, as cell reselection becomes a continuous operation with legacy frequency priorities considered as lower priorities than slice specific frequency priorities.

Observation 3: Based on RAN2 agreement, only one of sliceInfo or legacy dedicated priority can be configured in RRC Release. As per existing cell reselection principles, if priorities are provided in dedicated signaling, all the broadcast priorities are ignored.

Observation 4: There will not be any associated slice for inter-RAT frequencies and they will be considered as lower priority than frequencies associated to a slice.

Observation 5: no clear reason for changing RAN2 agreement so that a slice could be associated to more than one slice groups.

Proposal 1: Specification needs to define the rules for prioritization and leave the issue of how the prioritization is realized to the UE implementation.

Proposal 2: RAN2 is kindly asked to agree the proposed TP for 38.304 in [3].

Proposal 3: In case prioritised slice is not supported in the highest ranked cell on the target frequency, the UE should use the slice priority of the highest priority slice supported by highest ranked cell.

Proposal 4: If the priority reduction leads to a higher priority frequency becoming an equal priority or lower priority frequency or if an equal priority frequency becomes a lower priority frequency, the UE needs to re-evaluate the cell reselection criteria with the reduced priority before performing the cell reselection.

Proposal 5: For option A without formula, the UE does not fallback to legacy cell reselection and legacy frequency priorities are considered as lower priorities than slice specific frequency priorities during slice based cell reselection.

Proposal 6: If the UE is configured with slice based dedicated priority, the UE doesn’t consider legacy NR priorities.

Proposal 7: RAN2 to discuss whether inter-RAT cell reselection priorities can be included independently from dedicated signaling of slice based priorities.

Proposal 8: Inter-RAT frequency can be considered in slice-based cell reselection with legacy priorities.

Proposal 9: On performing inter-RAT cell reselection, the UE deletes dedicated slice information and does not perform slice aware cell reselection.

Proposal 10: RAN2 to keep its previous agreement on slice group definition “A group which is associated with one or multiple slices. And a slice is associated to none or only one slice group.”

Proposal 11: RAN2 to postpone discussion on the slice group granularity pending further input from SA2.

# References

[1] RAN2#116bis-e, Report on LTE legacy, 71 GHz, DCCA, Multi-SIM and RAN slicing, Vice Chairman notes

[2] R2-2201730 List of open issues for RAN slicing WI [Post116bis-e][203][Slicing] Open issues for RAN slicing

[3] R2-2203271 Text Proposal for 38.304 on cell reselection for RAN slicing, Samsung, Qualcomm Incorporated, OPPO.

[4] R2-2201708, Report for [AT116bis-e][240][Slicing] Remaining details for slice groups

[5] R2-2111235 Reply LS on Slice list and priority information for cell reselection (SA2)

[6] R2-2111308 Summary of [AT116-e][240][Slicing] LS reply on slice list and priority information

# Annex A (List of open issues for RAN slicing WI [2])

| Issue | Source or Relevant section in TS 38.304 | Suggested handling |
| --- | --- | --- |
| OI 3.1: Option A without formula: Solution 4, all NAS-prioritised slices with frequency priorities as well as legacy frequency priorities are considered, without iteration, without formula | Chairman notes | Top priority Suggest to have both offline and online discussion. Company tdocs with additional details are invited. |
| OI 3.2: In case prioritised slice is not supported in the highest ranked cell on the target frequency, what’s the UE behaviour, e.g., uses legacy frequency priority or recalculate frequency priority? | Email discussion [Post116-e][242][Slicing] | Suggest to have both offline and online discussion. Company tdocs with additional details are invited. |
| OI 3.3: Whether additional exit condition needed for fallback to legacy cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.4: After the UE fallbacks to legacy cell reselection, the next trigger of slice-based cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.5: If the UE is configured with slice based dedicated priority, but the UE cannot find a suitable cell, whether and how to fallback to legacy cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.6: Whether the inter-RAT frequency should be considered in slice-based cell reselection. | Email discussion [Post116-e][242][Slicing] | Proposed in comments of [242] email discussion and can be discussed based on company contributions. |
| OI 3.7: The definition of slice group is FFS. A group which is associated with one or multiple slices. And a slice is associated to none or one slice group. FFS associated to multiple slice groups. | 3.1 | If no agreement achieved in SA2, RAN2 can make decision then informs SA2. |
| OI 3.8: Slice specific cell reselection parameters. | 5.2.4.7.0 | CR rapporteurs to update aligned with RRC spec. |
| OI 3.9: Whether to confirm the granularities of the slice groups for cell reselection are per TA. Whether AS is aware of the TA or TAs where a specific slice group is used. | Chairman notes | If no agreement achieved in SA2, RAN2 can make decision then informs SA2. |
| OI 3.10: whether the slice specific cell reselection information provided by the network in SIB or RRCRelease message is slice or slice group specific. | May impact 38.304 and 38.300 | Company tdocs are invited. |