3GPP TSG-RAN2 #117-e  	R2-2203263

# Electronic meeting, Februrary, 2022

Agenda item:		8.17.3.2 (NR_FeMIMO)

Source:	LG Electronics Inc.

Title: 	Signaling support for UL power control for BM

Document for:	Discussion and Decision

1. Introduction

In this contribution, we discuss signalling support for UL power control for BM and propose the following:

2. Discussion

The signalling of PUCCH power control parameters for BM is under the discussion in [Pre117-e][009]. The relevant question is also present in the LS sent to RAN1 at RAN2#116bis.

Question 1.18: Is it correct understanding that network may provide UE the UL pc configuration in either UL TCI states or in BWP-UL-dedicated or should RAN2 choose one? If UL PC configuration is signalled in BWP-UL-dedicated only, how can the specific PC configuration (actually applied) be decided in PHY layer?

While RAN2 is waiting for answer from RAN1, we believe that it is good to understand the RAN1 Agreement as captured below.

Agreement (RAN1#105-e)

On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework,

- For each of PUSCH and PUCCH, the setting of (P0, alpha, closed loop index) can be associated with UL or (if applicable) joint TCI state per BWP.

  - In this case, multiple settings are configured. Each setting can be associated with at least one TCI state, and, for a given TCI state, only one setting for PUSCH and only one setting for PUCCH can be associated at a time.

  - (Working Assumption) In this case, for each of the PUSCH and PUCCH, each of the activated UL or (if applicable) joint TCI states is associated with one of the settings.

- If not associated, for each of the PUSCH and PUCCH, the setting(s) of (P0, alpha, closed loop index) per channel/signal per BWP is independent of the UL or (if applicable) joint TCI states

- FFS: If the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state.

- FFS: (to be decided in RAN1#106-e) whether to configure the same setting of (P0, alpha, closed loop index) per TCI state across channels and apply a channel dependent component, or configure a channel dependent setting of (P0, alpha, closed loop index) per TCI state

Agreement (RAN1#106-e)

On the setting of UL PC parameters except for PL-RS (P0, alpha, closed loop index) for Rel.17 unified TCI framework, the setting of (P0, alpha, closed loop index) for SRS can also be associated with UL or (if applicable) joint TCI state.

- If not associated, the setting(s) of (P0, alpha, closed loop index) for SRS per BWP is independent of the UL or (if applicable) joint TCI states

- This is only applicable for SRS sets using Rel-17 TCI state to determine their spatial relation.

FFS: Whether more than one parameter sets can be configured, e.g. for different traffics

From the RAN1 agreement, it is clear that the intended functionality is that UL PC parameters and PL RS are updated together when the UL/joint TCI state is updated as highlighted in green.

To enable the simultaneous update of TCI for PUCCH and PUSCH, UL PC parameters and PL RS should be associated with UL/joint TCI state, and there exist an associated PC parameter set for PUCCH and that for PUCCH respectively.

The most efficient way to implement the required seems to configure a list of PC parameter set entries, where each entry has PC parameter set for PUCCH and that for PUSCH, and to configure association between the entries in the list and TCI states of UL/joint TCI. For the signalling of the association, we can optionally include a pointer/reference to one entry of the UL PC parameter set list for each UL BWP. For association between PL RSs and TCI states, we can take a similar approach as UL PC parameter association, or we can instead encode PL RS directly into UL/joint TCI state, i.e. under UL/joint TCI state IE. For the signalling granularity of the list of PC parameter sets, we think per cell group can work but per serving cell is better.

Proposal 1. To support a list of PC parameter set entries, where each entry has PC parameter set for PUCCH and that for PUSCH, and to configure association between the entries in the list and TCI states of UL/joint TCI. The list is configured within ServingCellconfig.

Proposal 2: To optionally add a reference to one entry of the UL PC parameter set list within each UL BWP (i.e., entry ID per UL BWP)

It should also be possible that UL PC parameters are not associated with UL/joint TCI state. In this case the UL PC configuration follows the legacy methods and it is independent of UL/joint TCI state. The relevant RAN1 agreements are highlighted in yellow above. That is, if the association is not configured, legacy PC parameters, not TCI-specific PC parameters, shall be applied.

Proposal 3. To support no association between the entries of PC parameter set list and TCI states of UL/joint TCI.

3. Conclusion

In this contribution, we discuss signalling support for UL power control for BM and propose the following:

Proposal 1. To support a list of PC parameter set entries, where each entry has PC parameter set for PUCCH and that for PUSCH, and to configure association between the entries in the list and TCI states of UL/joint TCI. The list is configured within ServingCellconfig.

Proposal 2: To optionally add a reference to one entry of the UL PC parameter set list within each UL BWP (i.e., entry ID per UL BWP)

Proposal 3. To support no association between the entries of PC parameter set list and TCI states of UL/joint TCI.