3GPP TSG-RAN WG2 Meeting #117e	R2-2203377

Online, 21 Feb– 3 Mar, 2022

Agenda item:	8.2.2.2

Source: 	Huawei, HiSilicon

Title: 	MAC CE based SCG activation

Document for: 	Discussion and Decision

# 1. Introduction

In the last meeting, RAN2 discussed the MAC CE based SCG (de)activation and did not have conclusions. In this contribution, we will further discuss this issue.

# 2. Discussion

As discussed in [1], we see clear benefit of using MAC CE for latency reduction and also several companies support the MAC CE based SCG (de)activation. We have the following observations:

- Using the MAC CE is very useful in the frequent activation and deactivation.

- Co-located MN and SN, or low latency MN-SN link are very common scenarios, in which the MN-SN delay is very low.

- In R15, there are some cross CGs MAC CEs (e.g. PDCP duplication, PHR).

Some companies argues that it would be RAN3 impacts. In our understanding, it will be a small impact.

RAN3 has agreed to use the SN modification request and SN modification required procedure to trigger the SCG (de)activation. In Xn/X2, the SCG configuration from the SN is optional in these messages. Therefore the MN can decide whether to use the MAC CE to (de)activate the SCG based on whether the network need to modify the RRC configuration. Therefore we think there is no Xn/X2 impact.

In F1, it is the CU to decide whether to trigger the SCG (de)activation and the CU will send the SCG (de)activation request to the DU. In our understanding, the CU of the MN only need send on MAC CE based (de)activation command to the DU of the MN. Then the DU of the MN knows whether the SCG (de)activation MAC CE is needed. it will be a small impact. In the Annex, we also provide the TP for F1 of RAN3.

- Introducing MAC CE for SCG (activation will only bring one small impact on F1.

Support the SCG activation using MAC CE.

Impacts on specification

In R15 and R16, RAN2 has designed the two SCell activation/deactivation MAC CEs. One has a fixed size and consists of a single octet, which is used when the number of SCells is smaller than 8. Another has a fixed size and consists of four octets, which is used when the number of SCells is equal to or larger than 8. Also RAN2 designs two LCH IDs for these two MAC CEs. In the existing SCell activation/deactivation MAC CEs, there is a reserved bit.

In our understanding, we can design the SCG (de)activation MAC CE like the design of SCell (de)activation MAC CE. One solution is to use the reserved bit in the existing SCell (de)activation MAC CE. The reserved bit is used to indicate whether to activate the SCG. Another solution is to introduce one new MAC CE for SCG activation. In the last meetings, RAN2 has agreed that all the SCG SCells are in deactivated state when the SCG is deactivated. Therefore, only one indication of SCG activation is needed for the SCG activation or deactivation command. RAN2 does not have conclusions on the SCell states when the SCG is activated. In our understanding, activation of PSCell is enough for the transmission of the bursty traffic. If the network wants to activate the SCG SCells, the network can activate the SCG SCells after the SCG PSCell is activated. As we are approaching the end of the WI, we suggests focusing on the simplest solution: a MAC CE only indicating "SCG activation" without any additional information.

Define one new eLCID for a zero-octet MAC CE indicating SCG activation.

# 3. Conclusion

Based on the discussion in this paper, we have the following proposals:

- Using the MAC CE is very useful in the frequent activation and deactivation.

- Co-located MN and SN, or low latency MN-SN link are very common scenarios, in which the MN-SN delay is very low.

- In R15, there are some cross CGs MAC CE design.

- Introducing MAC CE for SCG (de)activation will only bring one small impact on F1.

- Support the SCG activation using MAC CE.

Define one new eLCID for a zero-octet MAC CE indicating SCG activation.

# 4. References

[1] R2-2201097, Reply LS on efficient activation/de-activation mechanism for one SCG, Huawei, HiSilicon

# 5. TP for 38.321 (based on R2-2201998 with changes accepted)

## 5.18	Handling of MAC CEs

### 5.18.1	General

This clause specifies the requirements upon reception of the following MAC CEs:

-	SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE;

-	Aperiodic CSI Trigger State Subselection MAC CE;

-	TCI States Activation/Deactivation for UE-specific PDSCH MAC CE;

-	TCI State Indication for UE-specific PDCCH MAC CE;

-	SP CSI reporting on PUCCH Activation/Deactivation MAC CE;

-	SP SRS Activation/Deactivation MAC CE;

-	PUCCH spatial relation Activation/Deactivation MAC CE;

-	Enhanced PUCCH spatial relation Activation/Deactivation MAC CE;

-	SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE;

-	Recommended Bit Rate MAC CE;

-	Enhanced SP/AP SRS Spatial Relation Indication MAC CE;

-	SRS Pathloss Reference RS Update MAC CE;

-	PUSCH Pathloss Reference RS Update MAC CE;

-	Serving Cell set based SRS Spatial Relation Indication MAC CE;

-	SP Positioning SRS Activation/Deactivation MAC CE;

-	Timing Delta MAC CE;

-	Guard Symbols MAC CEs;

-	SCG activation MAC CE.

### 5.18.x	SCG activation

The network may activate the SCG by sending the SCG activation MAC CE described in clause 6.1.3.x. The network can send this MAC CE while the SCG is deactivated.

The MAC entity shall:

1>	perform SCG activation as specified in clause 5.x,

1>	indicate SCG activation to upper layers.

## 5.X	Activation/Deactivation of SCG

Editor note: for terminology” activation/deactivation of SCG”, further discuss if a better wording is needed.

The network may activate and deactivate the configured SCG. Upon configuration of an SCG, the SCG is activated unless the parameter scg-State is set to deactivated for the SCG by upper layers.

The configured SCG is deactivated by:

-  receiving scg-State per SCG;

Editor note: FFS if MAC CE is used for SCG activation/deactivation.

The MAC entity shall for the configured SCG:

1>	if upper layers indicate that the SCG is activated; or

1>	upon reception of the SCG activation MAC CE as specified in clause 5.18.x:

2>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

3>	initiate a Random Access Procedure (as specified in clause 5.1.1).

2>	activate the SCG according to the timing defined in TS 38.xxx [xx] for direct SCG activation; i.e. apply normal SCG operation including:

3>	SRS transmissions on the PSCell;

3>	CSI reporting for the PSCell;

3>	PDCCH monitoring on the PSCell;

3>	PUCCH transmissions on the PSCell.

- else if upper layers indicate that the SCG is deactivated:

2>	deactivate all the SCells of the configured SCG according to clause 5.9;

2>	deactivate PSCell according to the timing defined in TS 38.xxx [xx], including:

3>	not transmit SRS on the PSCell:

3>	not report CSI for the PSCell:

3>	not transmit on UL-SCH on the PSCell:

3>	not transmit PUCCH on the PSCell:3>	not monitor the PDCCH for the PSCell:

3>	not monitor the PDCCH on the PSCell.

2>	reset MAC according to clause 5.12:

Editor note: FFS if the BWP associated with PSCell is NOT deactivated upon SCG deactivation.

#### 6.1.3.xx	SCG Activation MAC CE

The SCG Activation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1. It has a fixed size of zero octets. It indicates activation of the SCG.

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 243 | 64 to 307 | Reserved |
| 244 | 308 | SCell activation |
| 245 | 309 | Serving Cell Set based SRS Spatial Relation Indication |
| 246 | 310 | PUSCH Pathloss Reference RS Update |
| 247 | 311 | SRS Pathloss Reference RS Update |
| 248 | 312 | Enhanced SP/AP SRS Spatial Relation Indication |
| 249 | 313 | Enhanced PUCCH Spatial Relation Activation/Deactivation |
| 250 | 314 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 251 | 315 | Duplication RLC Activation/Deactivation |
| 252 | 316 | Absolute Timing Advance Command |
| 253 | 317 | SP Positioning SRS Activation/Deactivation |
| 254 | 318 | Provided Guard Symbols |
| 255 | 319 | Timing Delta |

# 6. TP for F1 38.473 of RAN3

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <unrelated part is omitted> |  |  |  |  |  |  |
| F1-C Transfer Path | O |  | 9.3.1.207 |  | YES | reject |
| SCG Indicator | O |  | ENUMERATED(released,...) | This IE is used at the MN in NR-DC and NE-DC and it indicates the release of an SCG | YES | ignore |
| SCG Activation Request | O |  | 9.3.1.x1 |  | YES | ignore |
| SCG Activation Request Using MAC CE | O |  | ENUMERATED(true,...) |  | YES | ignore |