3GPP TSG-RAN WG2 #117-e	Tdoc R2-2203483

Electronic meeting, 2022-02-21 - 2022-03-03

Agenda Item:	9.2.3.2

Source:	Ericsson

Title:	User plane aspects of NB-IoT and LTE-M in NTNs

Document for:	Discussion, Decision

# 1	Introduction

In this contribution, we give an overview of the remaining user plane changes when adapting NB-IoT and LTE-M to support NTNs.

# 2	Discussion

## 2.1 OI 1.10 [Company Tdocs Invited]: Whether SR can be triggered if there is no available or sufficient UL-SCH resources for the triggered TA reporting?

The UE should never trigger an SR if the UE has UL-SCH resources. When there is a grant available, then the UE should include a TA report if a TA report have been triggered.

If UE has no UL data and eNB has DL data, the eNB can send an UL grant if eNB suspects the UE need to report the TA or eNB can send a DL MAC CE to update the UE-specific-Koffset (for example set it to the cell specific Koffset).

If there has been no communication with the UE for a while and data arrives (either DL data or UL data resulting in an SR), and the eNB is not sure if the UE has a UE-speciffic Koffset that is sufficiently large: the eNB can send a DL MAC CE to update the UE-specific Koffset (for example set it to the cell specific Koffset). Alternatively, eNB can send an PDCCH order and the UE will reply with a TA report in the Msg3 (RAN1 have agreed that for PDCCH order, the UE shall always apply the cell-specific Koffset).

We propose

A triggered TA report does not trigger an SR.

## 2.1 OI 2.8 [Company Tdocs invited]: Configuration of event-triggered TA report

As a TA report shall be triggered if the timing Advance changes more than an offset from last successful reported TA, we need an offset.

The range must include values lower than 1 ms (if frequent reports are wanted more often than the need to change the UE specific Koffset), the value 1 ms, and values larger than 1 ms (in case strategy is to change the Koffset in steps larger than 1 ms). For example {0.1 ms, 0.3 ms, 0.5ms, 1 ms, 1.5 ms, 2 ms, 2.5 ms, 3 ms}. The threshold may be in MAC-MainConfig.

Add an offset threshold for TA report triggering in the MAC-MainConfig.

Add the offset thresholds {0.1 ms, 0.3 ms, 0.5ms, 1 ms, 1.5 ms, 2 ms, 2.5 ms, 3 ms} for TA reporting

# 3	Conclusions

Based on the discussion in the previous sections we propose the following:

Proposal 1	A triggered TA report does not trigger an SR.

Proposal 2	Add an offset threshold for TA report triggering in the MAC-MainConfig.

Proposal 3	Add the offset thresholds {0.1 ms, 0.3 ms, 0.5ms, 1 ms, 1.5 ms, 2 ms, 2.5 ms, 3 ms} for TA reporting