3GPP TSG-RAN WG2 Meeting #116 electronic						R2-2203516

Online, November 1-12, 2021

Agenda Item:	10.6

Source: 			Session chair (CMCC)

Title: 			Report from SON/MDT session

Document for:	Approval

Organizational:

- LSs – contact companies should flag LSs that need presenting.  Otherwise we will directly note them

- Running CRs will be endorsed to be used as baseline and moved to email discussion.  Further agreements will be captured on that baseline CR.

- Only Email discussions and summary discussions will be treated during e-meetings (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT117][800][SON/MDT] Organizational Hu

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to SON/MDT

    - Share meetings notes and agreements for review and endorsement

## 6.4	SON/MDT support for NR

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776).

Documents in this agenda item will be handled in a break out session

Tdoc Limitation: See tdoc limitation for Agenda Item 6

### 6.4.1	General and stage-2 corrections

Including incoming LSs, TS 37.320 corrections

R2-2202223	Corrections to SON/MDT capabilities	Lenovo, Motorola Mobility	CR	Rel-16	38.306	16.7.0	0675	-	F	NR_SON_MDT-Core

### 6.4.2	TS 38.314 corrections

R2-2202707	Correction to R16 38.314 on PRB Usage for MIMO	CMCC	CR	Rel-16	38.314	16.4.0	0021	-	F	NR_SON_MDT-Core

### 6.4.3	RRC corrections

R2-2202502	Addition of missing information into RA-InformationCommon-r16	Apple, Ericsson	CR	Rel-16	38.331	16.7.0	2892	-	F	NR_SON_MDT-Core

R2-2202737	Correction on LTE UE RLF Report	China Telecom, CATT, Ericsson	discussion

R2-2202783	Corrections on LTE UE RLF Report	China Telecom, CATT, Ericsson	CR	Rel-16	38.331	16.7.0	2906	-	F	NR_SON_MDT-Core

R2-2203330	On DAPS handover failure handling	Ericsson	CR	Rel-16	38.331	16.7.0	2943	-	F	NR_SON_MDT-Core

R2-2203332	On including SSB and CSI-RS measurements in RLF report	Ericsson	CR	Rel-16	38.331	16.7.0	2944	-	F	NR_SON_MDT-Core

R2-2203333	On ObtainCommonLocation related configuration	Ericsson	CR	Rel-16	38.331	16.7.0	2945	-	F	NR_SON_MDT-Core

R2-2203334	On sensor information configuration	Ericsson	CR	Rel-16	38.331	16.7.0	2946	-	F	NR_SON_MDT-Core

## 8.13	SON/MDT

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

### 8.13.1	Organizational

Tdoc Limitation: 0

LS in. For LSes that need action or has impact beyond taking into account by CR rapporteurs: One tdoc by contact company (one company) to address the LS and potential reply is considered Rapporteur Input and may be provided.

R2-2202116	LS on UP measurements for Successful Handover Report (R3-212935; contact: Ericsson)	RAN3	LS in	Rel-17	To:RAN2

R2-2202117	Reply LS on UE context keeping in the source cell (R3-212944; contact: Ericsson)	RAN3	LS in	Rel-17	To:RAN2

R2-2202118	LS Reply on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (R3-214429; contact: Ericsson)	RAN3	LS in	Rel-17	To:SA5, RAN2

R2-2202120	Reply LS on scenarios need to be supported for MRO in SCG Failure Report (R3-216159; contact: Samsung)	RAN3	LS in	Rel-17	To:RAN2

R2-2202125	Reply LS on Area scope configuration and Frequency band info in MDT configuration (R3-221178; contact: Huawei)	RAN3	LS in	Rel-17	To:RAN2

R2-2202133	Reply LS to SA5 on beam measurement reports (R3-221383; contact Ericsson)	RAN3	LS in	Rel-17	To:SA5	Cc:RAN2

R2-2202177	Reply LS on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (S5-213499; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2202178	Reply LS on Report Amount for M4, M5, M6, M7 measurements (S5-214523; contact: Nokia)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2202179	Reply LS on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (S5-215493; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2202180	Reply LS on the Beam measurement reports for the MDT measurements (S5-216628; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2203029	Draft Reply LS on Area scope configuration and Frequency band info in MDT configuration	Huawei, HiSilicon	LS out	Rel-17	NR_ENDC_SON_MDT_enh-Core	To:RAN3

R2-2203468	Reply LS on user plane measurements in successful handover report	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

### 8.13.2	CRs and Rapporteur Resolutions

Tdoc Limitation: 0.

CR Rapporteurs to provide running CRs, potentially updated, and Provide resolution proposals to Rapporteur Handled Open Issues (directly in the running CR). See also R2-2201991, R2-2202015, and R2-2201986.

R2-2202706	Running 38.314 CR for R17 layer 2 measurements	CMCC	CR	Rel-17	38.314	16.4.0	0020	-	B	NR_ENDC_SON_MDT_enh-Core

=>	CR is agreed.

R2-2203394	Introduction of Rel-17 MDT enhancements	Nokia, Nokia Shanghai Bell	CR	Rel-17	37.320	16.7.0	0115	-	B	NR_ENDC_SON_MDT_enh-Core	Late

=>	Endorsed as baseline.

R2-2203025	NR RRC CR for introducing R17 MDT	Huawei, HiSilicon	CR	Rel-17	38.331	16.7.0	2922	-	B	NR_ENDC_SON_MDT_enh-Core

=> Endorsed as baseline.

R2-2203470	Enhancement of data collection for SON	Ericsson	CR	Rel-17	38.331	16.7.0	2865	1	B	NR_ENDC_SON_MDT_enh-Core	R2-2200004

=> Endorsed as baseline.

- [Post117e][888][SON/MDT] 38.331 CR of introducing R17 SON/MDT (Ericsson, Huawei)

Step 1: 4 days discussion for endorsing SON CR, which based on R2-2203470 and capturing new agreements from this and previous meetings. (Ericsson)

Step 2: 3 days for merging MDT CR in and reviewing the big SON/MDT CR

All the related invited inputs on these proposals should be taken into account.

Intended outcome: Agreed SON/MDT RRC CR(s)

Deadline of Step1: 23:23 UTC, March 9th

Deadline of Step2: 23:23 UTC, March 11th

- [Post117e][898][SON/MDT] MDT satge-3 CR (Huawei)

Based on R2-2203025, capturing all the agreements of R17 MDT

Intended outcome: Agreeable draft CR, which will be merged into post meeting #888.

Deadline: 23:23 UTC, March 9th

- [Post117e][855][SON/MDT] 37.320 on introduction of Rel-17 MDT enhancements (Nokia)

Based on R2-2203394, capturing all the necessary agreements of R17 MDT

Intended outcome: Agreed 37.320 CR.

Deadline: 23:23 UTC, March 9th

### 8.13.3	SON related Open Issues

Including Pre117-e discussions to gather company input on specific Open Issues

Including company input on Open Issues

See also R2-2201991, and R2-2202015

R2-2202570	SON Enhancements for CHO	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2202571	SON Enhancements for SHR	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2203010	Open issues on SHR	NEC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203210	Discussion on SON HO left issues	OPPO	discussion	Rel-17	NR_pos_enh-Core

#### 8.13.3.1  Pre-discussions

Tdoc Limitation: 0

R2-2203526	SON related open issue lists	Ericsson	revised to R2-2203754

R2-2203754	SON related open issue lists	Ericsson

Agreements:

1	The time elapsed between the DAPS HO initialization and the RLF in the source cell after fallback is represented by the timeConnFailure (no changes needed to the current running CR).

2	The modeling of the UE actions in the case of consecutive failures in the current running CR is considered as baseline. Further clarifications (if any) may be addressed during the running CR review.

3	The timeUntilReconnection in the RLF report for the consecutive CHO failure cases represents the time from first failure to the time of reconnection.

4	To include the ‘t312-expiry’ as a new rlf-cause in the RLF-Report.

5	The UE shall generate the SHR due to RLF in the source cell during a DAPS HO, only if it is configured to do so in the SHR configuration (i.e. in the successHO-Config).

6	To include PLMN checking before sending the availability indicator for the SHR (as in RLF Report).

7	RAN2 to confirm that the UE includes the RA resource related parameters (frequency start, FDM, and SubcarrierSpacing of the msgA RA resource) under following scenarios:

a.	RA procedure involves only 2 step RA (i.e. no switching to 4-step RA)

b.	When 2 step RA to 4 step RA switching occurs, only those parameters that are different in 4 step RA resources compared to the 2 step RA resources are included.

8	TS 36.331 modifications are not introduced to handle the scenario of LTE MN fetching the list of NR RA reports in Rel-17.

9	TS 38.331 modifications are not introduced to handle the scenario of NR MN fetching the LTE RA report in Rel-17.

10	RAN2 confirms (UE behaviour from Rel-15/Rel-16) that the UE sets the failureType to randomAccessProblem in the SCGFailureInformationNR, when the UE experiences random access problem indication from the SCG MAC whileT304 is running for the SCG. Otherwise, if the UE initiates transmission of the SCGFailureInformationNR message to provide reconfiguration with sync failure information for an SCG (T304 expiry), the UE sets the failureType to synchReconfigFailureSCG.

11	The total number of PSCell (across all PCells) related information that should be stored by the UE in the MHI in 16.

12	When the UE reaches the maximum number of PSCell, if it gets a new PSCell, the UE removes the oldest stored PSCell entry and stores the newly configured PSCell entry.

13	The UE includes the time spent with no PSCell in the MHI, when connected to a certain PCell.

14	Keep the CHO candidate cell list and the CHO configuration only in the RLF-Report (not in the SHR), as in the current running CR. This agreement can be revisit depending on RAN3 progress.

15	UP interruption measurements should be considered: Only at DAPS HO.

16	The SHR configuration is provided in the otherConfig which can be provided by the source cell before the HO, and/or by the target cell as part of the HO command (as in the current running CR).

17	Clarify in the field descriptions of the successHO-Config IE which node (source/target) configures the specific triggering condition:

a.	T312/T310 thresholds are configured by the source (confirm agreement from RAN2#115)

b.	T304 threshold is configured by the target (confirm agreement from RAN2#116)

c.	Source cell(s) configure(s) the DAPS source RLF condition.

- [AT117e][888][SON/MDT] SON related Open Issues (Ericsson)

Including all the proposals not treated in R2-2203895

Invite companies to show their view on whether or not to agree these proposals.

There will be no technical discussion on CB session and conclusions will be made following majority and no objection.

Intended outcome: Report  to the CB session.

New Deadline: 11:11 UTC, March, 2nd

R2-2203895

Agreement:

Inclusion of one or more of the following PUSCH resource parameters only when the UE uses random access resources provided in dedicated signalling, or only when configured with CFRA:

a.	msgA-MCS (4 bits)

b.	nrofPRBs-PerMsgA-PO (5 bits)

c.	msgA-PUSCH-TimeDomainAllocation (4 bits)

d.	frequencyStartMsgA-PUSCH (9 bits)

e.	nrofMsgA-PO-FDM (2 bits)

R2-2203989	[AT117e][888][SON/MDT] SON related Open Issues (Ericsson) – Phase 2

Agreements:

1	The RA related Information associated to the SCG failure are included in the SCGFailureInformation.

the SCG failure due to random access problem indication in the SCG MAC

2	The UE only includes the perRAInfoList rather than the full RA-Information in the SCGFailureInformation message.

3	A single T312 threshold common to all measurement identities is configured in the SHR configuration

4	The SHR shall be generated only if the T312 associated to the measurement identity of the target cell is running.

5	RAN2 does not see the need to include the following:

a.	Indicator in the RLF-Report (SHR) indicating that there is an SHR (RLF-Report) associated to the same HO

b.	Timestamps in the SHR and RLF-Report to link them in time. FFS how to represent this timestamp (e.g. absolute or relative timestamp)

7	Amend the running CR such that the SHR will not be generated when the UE succeeds with the CHO recovery, in line with the agreement from RAN2#114-e .

Agreements:

1	For the 2-step RA, the payload reported by the UE in the RA-Report is the overall payload available in the UE buffer at the time of initiating the 2 step RA procedure.

2	A 3-bit bitstring in RA report is adopted, where the value of the 3-bit bitstring refers to one of the indexes of the 5-bit BSR table in TS 38.321 (similar to the definition of the messageSize field within SL-TrafficPatternInfo)

3	The RA Information associated to a SCG failure are included in the SCGFailureInformation for the following scenarios

a.	when failureType is set to randomAccessProblem while T304 is running

c.	when failureType is set to synchReconfigFailureSCG

4	RAN2 to include the following information in the SCGFailureInformation in case of SCG failure

a.	previousPSCellID (i.e. PCI)

b.	failedPSCellID (i.e. PCI)

c.	timeSCGFailure

5	There is no need for the UE to include a 1 bit flag in the SCGFailureInformation to indicate that the T304 was running when the UE declared

6	The C-RNTI of the target cell is included in the SHR .

=>	Regarding Bullet 4, if RAN3 prefers CGI, we will follow RAN3’s preference when reviewing running CR.

#### 8.13.3.2  Invited Input

Company input by tdocs

R2-2202591	MRO-related remaining open issues	Apple	discussion

R2-2202730	Leftovers for consecutive CHO failures	CMCC, CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202731	Leftovers for SHR	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202732	Leftovers for MRO for SN	CMCC	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202778	Discussion on SON related open issues	LG Electronics	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202801	Discussion on SON Related Open Issues	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202802	Discussion on Open Issue in Stage-2 Running CR	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202971	Discussion on SHR enhancements	vivo	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202973	Consideration on SON open issues	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2203014	Discussion on SHR related open issues	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203015	Discussion on SgNB MRO related open issues	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203395	Detailed information required for MRO for SN change failure	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203397	SHR and RLF report generation for same handover	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203420	HO related SON changes	Qualcomm Incorporated 	discussion	Rel-17

R2-2203464	Handover-related SON aspects	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2203465	On PSCell MHI and SCG MRO enhancements	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

### 8.13.4	MDT related Open Issues

Including Pre117-e discussions to gather company input on specific Open Issues

Including company input on Open Issues

See also R2-2201986

#### 8.13.4.1  Pre-discussions

Tdoc Limitation: 0

R2-2203026	Pre-meeting discussion report for R17 MDT	Huawei	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	Late

Agreements:

1: When the UE occurs a new CEF, if the failed cell id of the CEF is the same as the failed cell id in the last entry in VarConnEstFailReportList, the UE replaces the last CEF report with the new CEF report and the numberOfConnFail is summed. Otherwise (two cell ids are different), the UE appends the new CEF into VarConnEstFailReportList.

2: For excess delay configuration in NR-DC, Node owning the PDCP terminating point configures the UE

•	Similar to the solution for D1 configuration, for all the bearers, the CU-CP of the node owning the PDCP terminating point configures the UE with excess delay measurement configuration. To enable this solution UE is allowed to be configured with at most one excess delay measurement per PDCP, which follows the D1 measurement for NR DC.

3: D1 delay ratio measurement results should include DRB id and excessDelay info, and they can be included in the IE MeasResults.

4: For D1 delay threshold values, at least the following values can be included:

250us, 0.5ms, 1ms, 2ms, 4ms, 10ms, 20ms, 50ms, 100ms, 500ms (10 values)

5: Introduce AreaConfiguration-r17 (including areaConfig-r16 and interFreqTargetList-r16 inside it with both fields being optional) in Rel-17.

- [AT117e][899][SON/MDT] MDT related Open Issues (Huawei)

Whether Network should be able to configure different delay threshold for different DRBs.

All the related invited inputs on this proposal should be taken into account.

Other real critical issues from the invited inputs.

Intended outcome: Report for the real final round discussion.

Deadline: 23:55 UTC, Feb, 25th

R2-2203900	Report of [AT117e][899][SON/MDT] MDT related Open Issues (Huawei)

Agreements:

1: For UL PDCP Excess Packet Delay (related to section 4.3.1.e in TS 38.314 CR), network should be able to configure different delay threshold for different DRBs.

3: UE reports that whether the on-demand SI acquiring was successful or not.

4: RAN2 liaise RAN3 that not introducing SN configuration in DC scenarios is applicable to all the DC scenarios such as EN-DC, NGEN-DC, NE-DC and NR-DC. ([8], Ericsson can handle the LS in #805)

=>	CB on Wednesday FFS: EMR is not supported in R17 log MDT

After offline discussion, Ericsson propose the following:

Agreements:

1	If the earlyMeasIndication-r17 is not included, the UE is not allowed to log EM in logged MDT report (following legacy logged MDT behaviours).

2	If the earlyMeasIndication-r17 is included, for the following frequencies:

-          If interFreqTargetInfo is included, for frequencies included in both interFreqTargetInfo and EMR config

-          If interFreqTargetInfo is not included, for frequencies included in EMR config

-          For inter-RAT related frequencies included in EMR config

How UE logs the measurements on EMR frequencies is left to the UE implementation.

3	For logging the measurements on EMR frequencies in logged MDT report, the qualityThreshold in measIdleConfig should not be applied.

- [AT117e][805][SON/MDT]  Not introducing SN configuration in DC scenarios (Ericsson)

Inform RAN3 on our agreement that that not introducing SN configuration in DC scenarios is applicable to all the DC scenarios such as EN-DC, NGEN-DC, NE-DC and NR-DC.

Intended outcome: Approved LS.

Deadline: 11:55 UTC, March, 2nd

R2-2204063 LS on logged MDT configuration at SN in DC scenario

=>	LS is approved

#### 8.13.4.2  Invited Input

Company input by tdocs

R2-2202733	Leftovers for on-demand SI	CMCC, Ericsson, Samsung, CATT, ZTE, Huawei	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202803	Discussion on MDT Related Open Issues	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202935	Support of MDT and QoE alignment	Qualcomm Incorporated	discussion	NR_QoE_enh

R2-2202974	Consideration on MDT open issues	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2203027	Discussion on MDT related open issues	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203329	Discussion on logged MDT open issues	Ericsson	discussion

R2-2203331	On Immediate MDT measurements	Ericsson, CMCC	discussion

R2-2203396	Early measurements logging in MDT	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

### 8.13.5	UE Capabilities

Initial discussion on Features / UE caps developed in RAN2, if any. Note that this AI is complementary to AI 8.0.2.

R2-2202804	UE Capabilities about SON and MDT Enhanced Features	CATT	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2202975	Consideration on UE capability	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2203028	Discussion on UE capabilities for R17 SON and MDT	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203427	SON MDT UE Capabilities	Qualcomm Incorporated	discussion	Rel-17

- [AT117e][877][SON/MDT] SONMDT related UE Capabilities (CATT, Intel)

Based on R2-2202804, R2-2202975, R2-2203028 and R2-2203427, build capability CR(s)

Intended outcome: Report and draft CR(s)

Deadline: 23:55 UTC, Feb, 25th

R2-2203831		[AT117e][877][SON/MDT] SONMDT related UE Capabilities (CATT, Intel)

Agreements:

1	Introduce an optional UE capability with signalling for Successful Handover Report.

2	Introduce an optional UE capability without signalling for SCG Failure Report for MRO.

3	Introduce an optional UE capability with signalling for Multiple CEF Report.

4	Introduce a conditionally mandatory feature for Logged MDT Measurement Suspension due to IDC Interference.

5	Introduce an optional UE capability with signalling for NR excess packet delay.

6 	R17 SON-MDT related capabilities are all defined per UE.

7	Introduce an optional UE capability with signalling for 2-step RACH Information Report.

8	Add “PCell” to the legacy MHI UE capability of “Mobility history information storage” only for R17

9	Introduce an optional UE capability with signalling for On-demand SI Report.

10	Introduce an optional UE capability with signalling for Signaling Based Logged MDT Override Protection.

11	Introduce optional UE capabilities with signalling for RLF-Report for CHO and DAPS HO, respectively.

12	Introduce an optional UE capability with signalling for PSCell MHI Report.

13	 Introduce an optional UE capability without signalling for SCell RACH Reporting.

14 No additional capability is required for SgNB RACH Report in NR.

- [AT117e][876][SON/MDT] SONMDT related UE Capabilities (CATT, Intel)

Based on the agreements of SONMDT related UE Capabilities, provide CR accordingly

Intended outcome: endorsed CR

Deadline: 23:55 UTC, March, 2nd

R2-2204089	38.306 CR for SONMDT UE capabilities	CATT	CR	Rel-17	38.306	16.7.0	-	-	B	NR_ENDC_SON_MDT_enh-Core

=>	Endorsed

R2-2204090	38.331 CR for SONMDT UE capabilities	CATT	CR	Rel-17	38.331	16.7.0	-	-	B	NR_ENDC_SON_MDT_enh-Core

=>	Endorsed and will be merged into SONMDT big CR through post meeting #888 step 2.

### 8.13.6	Others

R2-2202939	Discussion on PSCell MHI recording	SHARP Corporation	discussion

R2-2202940	Discussion on SHR in CHO recovery case	SHARP Corporation	discussion	R2-2201229

=>	WI is closed from RAN2 perspective.

# Summary

Agreed CR:

R2-2202706	Running 38.314 CR for R17 layer 2 measurements	CMCC	CR	Rel-17	38.314	16.4.0	0020	-	B	NR_ENDC_SON_MDT_enh-Core

=>	CR is agreed.

Post meeting email discussions:

- [Post117e][888][SON/MDT] 38.331 CR of introducing R17 SON/MDT (Ericsson, Huawei)

Step 1: 4 days discussion for endorsing SON CR, which based on R2-2203470 and capturing new agreements from this and previous meetings. (Ericsson)

Step 2: 3 days for merging MDT CR in and reviewing the big SON/MDT CR

All the related invited inputs on these proposals should be taken into account.

Intended outcome: Agreed SON/MDT RRC CR(s)

Deadline of Step1: 23:23 UTC, March 9th

Deadline of Step2: 23:23 UTC, March 11th

- [Post117e][898][SON/MDT] MDT satge-3 CR (Huawei)

Based on R2-2203025, capturing all the agreements of R17 MDT

Intended outcome: Agreeable draft CR, which will be merged into post meeting #888.

Deadline: 23:23 UTC, March 9th

- [Post117e][855][SON/MDT] 37.320 on introduction of Rel-17 MDT enhancements (Nokia)

Based on R2-2203394, capturing all the necessary agreements of R17 MDT

Intended outcome: Agreed 37.320 CR.

Deadline: 23:23 UTC, March 9th

Approved LS

R2-2204063 LS on logged MDT configuration at SN in DC scenario

=>	LS is approved