3GPP TSG-RAN WG2 Meeting #117-e	R2-2203832

# Electronic meeting, 21st Feb – 3rd Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | - | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.306 CR for SONMDT UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of R17 features on SON and MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | R17 UE capabilities for SON and MDT are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No R17 UE capabilities for SON and MDT are defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.17, 4.2.18, 5.7, 6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.331 CR TBD |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

4.2.17	SON parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| onDemandSI-Report-r17 Indicates whether the UE supports delivery of on-Demand SI information upon RA report request from the network. | UE | No | No | No |
| pscell-MHI-Report-r17 Indicates whether the UE supports the storage of PSCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. | UE | No | No | No |
| rach-Report-r16 Indicates whether the UE supports delivery of rachReport upon request from the network. | UE | No | No | No |
| rlfReportCHO-r17 Indicates whether the UE supports RLF-Report for conditional handover. | UE | No | No | No |
| rlfReportDAPS-r17 Indicates whether the UE supports RLF-Report for DAPS handover. | UE | No | No | No |
| success-HO-Report-r17 Indicates whether the UE supports the storage and delivery of Successful Handover Report. | UE | No | No | No |
| twoStepRACH-Report-r17 Indicates whether the UE supports the storage and delivery of 2-step RACH related information upon RA report request from the network. | UE | No | No | No |

4.2.18	UE-based performance measurement parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| barometerMeasReport-r16 Indicates whether UE supports uncompensated barometeric pressure measurement reporting upon request from the network. | UE | No | No | No |
| excessPacketDelay-r17 Indicates whether the UE supports the UL PDCP SDU exceeding the configured delay threshold ratio measurement per DRB (as specified in TS 38.314 [26]) and reporting in RRC_CONNECTED state. | UE | No | No | No |
| immMeasBT-r16 Indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state. | UE | No | No | No |
| immMeasWLAN-r16 Indicates whether the UE supports WLAN measurements in RRC_CONNECTED state. | UE | No | No | No |
| loggedMeasBT-r16 Indicates whether the UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. | UE | No | No | No |
| loggedMeasWLAN-r16 Indicates whether the UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| multipleCEF-Report-r17 Indicates whether the UE supports the storage and delivery of multiple CEF upon request from the network. | UE | No | No | No |
| orientationMeasReport-r16 Indicates whether the UE supports orientation information reporting upon request from the network. | UE | No | No | No |
| sigBasedLogMDT-OverrideProtect-r17 Indicates whether the UE supports the override protection of the signalling based Logged MDT configured in NR. | UE | No | No | No |
| speedMeasReport-r16 Indicates whether the UE supports speed information reporting upon request from the network. | UE | No | No | No |
| gnss-Location-r16 Indicates whether the UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. | UE | No | No | No |
| ulPDCP-Delay-r16 Indicates whether the UE supports UL PDCP Packet Average Delay measurement (as specified in TS 38.314 [26]) and reporting in RRC_CONNECTED state. | UE | No | No | No |

Next change

5.7	MDT and SON features

Definitions for feature

Mobility history information storage It is optional for UE to support the storage of PCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9].

Cross RAT RLF Report It is optional for UE to support the delivery of EUTRA RLF report to an NR node upon request from the network.

Radio Link Failure Report for inter-RAT MRO EUTRA It is optional for UE to support: - Inclusion of EUTRA CGI and associated TAC, if available, and otherwise to include the physical cell identity and carrier frequency of the target PCell of the failed handover as failedPCellId in RLF-Report upon request from the network as specified in TS 38.331 [9]. - Inclusion of EUTRA CGI and associated TAC as previousPCellId in RLF-Report as specified in TS 38.331 [9]. - Inclusion of eutraReconnectCellId in reconnectCellId in the RLF-Report as specified in TS 38.331 [9] upon UE has radio link failure or handover failure and successfully re-connected to an E-UTRA cell.

SCG Failure Report for MRO It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network.

SPCell ID indication It is optional for UE to support the delivery of the SPCell ID in the RA-Report, if the RA procedure is performed in a SCell of the MCG/SCG.

6	Conditionally mandatory features without UE radio access capability parameters

| Features | Condition |
| --- | --- |
| Skipping UL configured grant if no data to transmit. | Either configuredUL-GrantType1 or configuredUL-GrantType2 is supported. |
| Downlink SDAP header | Either NAS reflective QoS or as-ReflectiveQoS is supported. |
| IMS emergency call | It is mandatory to support IMS emergency call for UEs which are IMS voice capable in NR. |
| MAC subheaders with one-octet eLCID field | It is mandatory to support MAC subheaders with one-octet eLCID field for UEs/IAB-MTs supporting MAC CEs using extended LCID values as specified in TS 38.321 [8]. |
| Logged MDT measurement suspension due to IDC interference | It is mandatory to support Logged MDT measurement suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network and in-device coexistence indication as specified in TS 38.331 [9]. |

End of change

Annex:

The following table is R17 Layer-2 and Layer-3 feature list for NR_ENDC_SON_MDT_enh-Core for reference.

### 6.2.xx	NR_ENDC_SON_MDT_enh-Core

Table 6.2.xx-1: Layer-2 and Layer-3 feature list for NR_ENDC_SON_MDT_enh-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| xx. NR_ENDC_SON_MDT_enh-Core | xx-1 | RLF for CHO | Indicates whether the UE supports RLF-Report for conditional handover. |  | rlfReportCHO-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-2 | RLF for DAPS HO | Indicates whether the UE supports RLF-Report for DAPS handover. |  | rlfReportDAPS-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-3 | Report for SHR | Indicates whether the UE supports the storage and delivery of Successfu l Handover Report. |  | success-HO-Report-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-4 | RA report for 2-step RA | Indicates whether the UE supports the storage and delivery of 2-step RACH related information upon RA report request from the network. | 20-1 | twoStepRACH-Report-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-5 | SPCell ID indication | It is optional for UE to support the delivery of the SPCell ID in the RA-Report, if the RA procedure is performed in a SCell of the MCG/SCG. | 20-1 | N/A | N/A | No | No |  | Optional without capability signalling |
|  | xx-6 | PSCell MHI storage | t is optional for UE to support the storage of PSCell mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [2]. | 20-12 | pscell-MHI-Report-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-7 | SCG Failure Report for MRO | It is optional for UE to support the delivery of the SCG failure related parameters for MRO in SCGFailureInformation message to the network. |  | N/A | N/A | No | No |  | Optional without capability signalling |
|  | xx-8 | On demand SI report | Indicates whether the UE supports delivery of on-Demand SI information upon RA report request from the network. | 20-1 | onDemandSI-Report-r17 | SON-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-9 | Signaling Based Logged MDT Override Protection | Indicates whether the UE supports the override protection of the signalling based Logged MDT configured in NR. | 20-6 | sigBasedLogMDT-OverrideProtect-r17 | UE-BasedPerfMeas-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-10 | Multiple CEF report | Indicates whether the UE supports the storage and delivery of multiple CEF upon request from the network. |  | multipleCEF-Report-r17 | UE-BasedPerfMeas-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-11 | excess packet delay | Indicates whether the UE supports the UL PDCP SDU exceeding the configured delay threshold ratio measurement per DRB (as specified in TS 38.314 [26]) and reporting in RRC_CONNECTED state. |  | excessPacketDelay-r17 | UE-BasedPerfMeas-Parameters-r16 | No | No |  | Optional with capability signalling |
|  | xx-12 | Logged MDT Measurement Suspension due to IDC Interference | It is mandatory to support Logged MDT Measurement Suspension due to IDC Interference if both logged MDT and IDC are supported. | 20-6 and 24-7 | N/A | N/A | No | No |  | Conditional mandatory without capability signalling |