3GPP TSG-RAN WG2 #117-e	Tdoc R2-22xxxxx

Electronic meeting, 2022-02-21 - 2022-03-03

Agenda Item:	8.21.2

Source:	Ericsson

Title:	[AT117-e][050][NR17TEI] Explicit Indication of SI Scheduling start 	position (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

# 1	Introduction

This document is to collect comments for the CR:

- [AT117-e][050][NR17TEI] Explicit Indication of SI Scheduling start position (Ericsson)

Scope: Treat R2-2203365

Intended outcome: Agreed CR.

Deadline: W1 Friday (if possible)

Please provide your comments related to the CR

R2-2203365	Explicit Indication of SI Scheduling start position [SI-SCHEDULING]	Ericsson, Verizon, Softbank, Deutsche Telekom, vivo	CR	Rel-17	38.331	16.7.0	2953	-	B	TEI1

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Qualcomm Incorporated | Masato KITAZOE (mkitazoe@qti.qualcomm.com) |
| vivo | yangxiaodong5g@vivo.com |
| Samsung | June Hwang (june77.hwang@samsung.com) |
| Apple | Zhibin Wu (zhibin_wu@apple.com) |
| Huawei, HiSilicon | Dawid Koziol (dawid.koziol@huawei.com) |

# 3	Comments

Please provide the comments on the CR here:

| Company | Comments |
| --- | --- |
| Qualcomm Incorporated | The following text for Cond FIRST-SI is more about semantics (rather than presence condition) and should be moved to the field description. - If this field is absent for the subsequent SI messages, the field value is the value of the previous entry in the schedulingInfoList2 plus 1, i.e the SI messages are scheduled in consecutive SI window order (plus one) until the field is present again. |
| vivo | Agree with Qualcomm. Moreover, suggest the “SI window start position” is changed to “SI window position”. |
| Samsung | Agree with QC comment. |
| Apple | - We share the same view as Qualcomm that this needs to be moved to field description. - Also, regarding the same sentence, there is no real “field value” if the field is absent. So, we suggest to make the following change: If this field is absent for the subsequent SI message, the window position of the corresponding SI message is determined based on the field value of the most recent present entry in the schedulingInfoList2 by assumingthe SI message(s) after that entry are scheduled in consecutive SI window order (plus one) until the field is present again |
| Huawei, HiSilicon | - We agree with the comment from QCM. - We think it is better from overhead perspective to introduce this change via nonCriticalExtension of SIB1, i.e.: SIB1-v17xy-IEs ::= SEQUENCE { SI-SchedulingInfo-v17xy SEQUENCE { schedulingInfoList2-r17 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 OPTIONAL -– Need R } OPTIONAL, -- Need R nonCriticalExtension SEQUENCE {} OPTIONAL } - On the following field: si-WindowPosition-r17 INTEGER (1..256) We are wondering why such large values are required which wastes a lot of memory for the UE. Considering the maximum number of SI messages is 32, we think the maximum value for si-WindowPosition of 96 is enough. - On “-- Cond FIRST-SI” – we would prefer to make this field always present (i.e. make it non-optional). Current handling of its absence is unnecessarily complicated. - On the following line: valueTag-r17 INTEGER (0..31) OPTIONAL, -- Cond SIB-TYPE “Cond SIB-TYPE” cannot be reused here as: - SIB6, SIB7 or SIB8 cannot be scheduled in SchedulingInfo2 anyway. - Value tag is not used for posSIBs So we would need a code described as follows for example: “The field is mandatory present when sibType is set to type1. Otherwise, it is absent.” - For sib-MappingInfo field description, posSIBs should be mentioned as well. - In type1 and posSibType field descriptions, do we need to mention exact types that cannot be configured? Perhaps it is OK as a placeholder/reminder, but in the end the applicable posSIB and SIB types will be anyway part of ASN.1 signalling, right? |

# Conclusion

In the previous sections we made the following observations: