3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-22xxxxx

Elbonia, 21st of Feb – 3rd of Mar 2022

Agenda item:	8.10.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Report from [AT117-e][108][NTN] CHO open issues (Nokia)

WID/SID:	NR_NTN_solutions-Core - Rel-17

Document for:	Discussion and Decision

# 1	Introduction

The scope of this paper is as follows:

* [AT117-e][108][NTN] CHO open issues (Nokia)

Initial scope: Discuss open issues for CHO based on company contributions in AI 8.10.3.2.1

Initial intended outcome: Summary of the offline discussion with e.g.:

§  List of proposals for agreement (if any)

§  List of proposals that require online discussions

§  List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Tuesday 2022-02-22 0800 UTC

Initial deadline (for rapporteur's summary in R2-2203536): Tuesday 2022-02-22 1000 UTC

The CHO details are handled in the following sections.

# 2	Discussion

This section is divided topic-wise, based on what has been contributed by the companies.

## 2.1	On simultaneous configuration of time-based and location-based CHO execution conditions

This aspect has been discussed for multiple meetings already, but apparently no formal and final decision was taken. Several papers submitted to RAN2#117 try to address this issue, e.g. [4][5][6]. Rapporteur thinks there may not be a solid use case which would justify such combination, in addition to the measurement events Ax. However, companies are asked to clarify whether they see a need for such joint time-based and location-based triggering. Please provide a use case where this would be especially desirable.

| Question 1: Do you support joint time-based and location-based CHO triggering? If yes, please provide a use case, where this would be particularly helpful. |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

## 2.2	On the behavior at T2 if UE does not execute CHO

Another topic widely addressed in the papers to RAN2#117 concerns the UE behavior at T2, defining the end of the time window for CHO execution. In some papers (e.g. [5][10]) it is suggested those CHO configurations are released at T2, while in other papers ([1][7][8]) it is claimed those configurations can be kept and used for potential recovery. This area requires more clarity and at least the following issues should be resolved: is CHO Recovery supported for NTN UEs? What happens when the UE does not execute CHO at T2 (e.g. RLF?). What happens with the target cell’s CHO configurations at T2? Please answer those questions in the following tables.

| Question 2: What happens when the UE does not execute CHO at T2? Consider at least the following: - The UE encounters Radio Link Failure (RLF) - The UE continues the operation in the source cell/evaluates other possible CHO conditions - Other |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

Irrespective of the answer to Q2, please share your view what happens with the CHO configurations at T2.

| Question 3: What happens with the CHO configuration at T2? |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

And finally whether the NTN UE, supporting CHO, can be configured with CHO Recovery? If it can, then perhaps it makes sense not to delete the CHO configurations even at T2.

| Question 4: Can the NTN UE supporting CHO, be configured with CHO Recovery? Please share the details how this may work beyond time T2. |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

## 2.3	On evaluating the CHO conditions in NTN

Few papers (e.g. [3][14]) also discuss the UE’s behavior concerning the time/location-based triggering and events Ax. I.e. whether the UE shall evaluate them only within the time window [T1, T2] or when the location condition is met? Or should it be left up to the UE implementation?

| Question 5: What is the expected UE behavior when it is configured with time- or location-based condition jointly with the RRM event Ax: - UE is required to evaluate event Ax only when the time-based or location-based condition is met - It is up to the UE implementation as long as it has RRM measurements within the time window [T1, T2] or when the location condition is met |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

## 2.4	On the duration and granularity of T2

Some remaining Stage-3 details need to be resolved as well. One of them concerns the duration and granularity of T2. It has been decided that T1 is expressed as an absolute time value, while the T2 is a timer, started at T1. T2 should be long enough so that it covers large NTN footprints and allows the UE to be configured early. On the other hand, the rapporteur believes it does not have to be extremely accurate and the granularity of 10 or even 100 ms could be sufficient. Please share your opinion.

| Question 6: What is the preferred duration and granularity of timer T2? |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

## 2.5	On the number of MeasIDs for the CHO

E.g. in [4][12] it is discussed whether the number of MeasIDs to be used for CHO execution triggering shall be increased. [4] proposes to extend it to 3, while [12] states it is acceptable to keep the existing limit. Please share your view on the maximum number of configurable MeasIDs in NTN CHO.

| Question 7: What is the maximum value of MeasIDs for NTN CHO that should be supported in Rel-17? |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

## 2.6 Other

There are few other CHO-related proposals in the papers submitted to RAN2#117. E.g. [1][2] elaborate on the benefits of preparing multiple CHO candidates in advance and storing those CHO commands. Please kindly respond what other important aspects need to be addressed in Rel-17 NTN WI.

| Question 8: What other important CHO issues need to be discussed and decided in Rel-17 NTN? E.g. - Storing CHO commands for future candidate cells - Other |  |  |
| --- | --- | --- |
| Company | Answer | Comments |

# 3	Conclusion

The following proposals have been made in this document:

Proposals for agreement:

Proposal y:

Proposals for discussion:

Proposal x

# References

- R2-2202467	Remaining Rel-17 NTN open issues for CONNECTED mode	Nokia

- R2-2202565	Open issues in CHO	Qualcomm Incorporated

- R2-2202587	Consideration on open issues for CHO	Lenovo, Motorola Mobility

- R2-2202775	Open issues on CHO for R17 NR NTN	vivo

- R2-2202886	Remaining issues on CHO	Huawei, HiSilicon

- R2-2203005	Discussion on the RRC open issues in NTN	OPPO

- R2-2203051	Remaining NTN CHO issues	LG Electronics France

- R2-2203067	Discussion on RRC open issues for NTN	Xiaomi Communications

- R2-2203077	Further Discussion on the Open Issues of CHO	CATT	discussion	Rel-17

- R2-2203153	Remaining connected mode aspects for NTN	Ericsson	discussion

- R2-2203154	[Pre117-e][NTN][101] RRC open issues	Ericsson

- R2-2203236	Remaining open issues of CHO	NEC Telecom MODUS Ltd.

- R2-2203301	Open issues on RRC aspects	Samsung Research America

- R2-2203422	Remaining RRC open issues in NTN	InterDigital