3GPP TSG-RAN WG2 Meeting #117 electronic		draft R2-203632

Online, February 21 - March 3, 2022

Agenda Item:	9.4

Source: 	Vodafone (Rapporteur)

Title:  	Report of [AT117-e][203][UPIP] LTE UPIP configuration and capabilities (Vodafone)

Document for:   Discussion and Decision

# Introduction

This document aims to discuss the issues that have been raised by contributions submitted to AI 9.4 “User Plane Integrity Protection support for EPC connected architectures”.

1st phase comment deadline: Wednesday W1, 1000 UTC.

# Documents submitted, etc

(UPIP_EN-DC_UE; leading WG: RAN3; REL-17; WID: RP‑213669)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

Including discussion on SA3 LS R2-2200153

Including configuration and capability aspects of allowing full rate UPIP for EN-DC UEs connected to EPC

R2-2202145	Reply LS on LTE User Plane Integrity Protection (R3-221473; contact: Vodafone)	RAN3	LS in	Rel-17	To:SA3, SA2	Cc:CT4, CT1, RAN2

R2-2202717	Introducing support of UP IP for EPC connected architectures using NR PDCP	Huawei, HiSilicon, Vodafone, Ericsson	CR	Rel-17	36.331	16.7.0	4763	-	B	UPIP_SEC_LTE

R2-2202718	Introducing support of UP IP for EPC connected architectures using NR PDCP	Huawei, HiSilicon, Vodafone, Ericsson	CR	Rel-17	38.331	16.7.0	2904	-	B	UPIP_SEC_LTE

R2-2202719	Introducing support of UP IP for EPC connected architectures using NR PDCP	Huawei, HiSilicon, Vodafone, Ericsson	CR	Rel-17	36.300	16.7.0	1353	-	B	UPIP_SEC_LTE

R2-2202720	Introducing support of UP IP for EPC connected architectures using NR PDCP	Huawei, HiSilicon, Vodafone, Ericsson	CR	Rel-17	37.340	16.8.0	0294	-	B	UPIP_SEC_LTE

R2-2202721	Introducing support of UP IP for EPC connected architectures using NR PDCP	Huawei, HiSilicon, Vodafone, Ericsson	CR	Rel-17	38.323	16.6.0	0085	-	B	UPIP_SEC_LTE

R2-2202722	Discussion on LTE User Plane Integrity Protection (SA3 LS)	Huawei, HiSilicon	discussion	Rel-17	UPIP_SEC_LTE

R2-2203369	draft Reply LS on LTE User Plane Integrity Protection	Vodafone	LS out	Rel-17	To:SA3	Cc:RAN3, SA2

# Discussion

## 3.1	UE Capabilities for LTE UPIP are sent in NAS signalling

The RAN plenary WID in RP‑213669 states the following:

Note: For security related reasons, the UE’s support/non-support for EPS-UPIP is not sent in the UE Radio Access Capabilities, and instead, it is sent from the UE to the MME in NAS messages and then sent by the MME to the RAN in S1-AP signalling.

EN-DC capable devices need to support NR-PDCP, and the support of EN-DC is signalled to the eNB within existing R15 UE Radio Access Capabilities. Hence no changes to TS 36.306 are anticipated to be needed.

TS 24.301 provides some of the related procedures in sections 5.4.3.2 and 5.4.3.3, e.g.:

#### 5.4.3.3	NAS security mode command accepted by the UE

Upon receipt of the SECURITY MODE COMMAND message, the UE shall check whether the security mode command can be accepted or not. This is done by performing the integrity check of the message and by checking that the received replayed UE security capabilities, the received replayed UE additional security capabilities, if included in the SECURITY MODE COMMAND message, and the received nonceUE have not been altered compared to the latest values that the UE sent to the network.

Question 1: please indicate ONLY if you believe that changes to the UE capabilities in TS 36.306 are required for this work item.

| Company | comments |
| --- | --- |

## 3.2	Response to LS in LS S3-214462/R2-2200153

In this LS, SA3 requested RAN 2 “…to inform SA3 on their final decision with respect to which algorithm code points are to be used”.

The discussion document in R2-2202722 proposes that the Rel-15 principle of security configuration of ciphering for EN-DC capable UE should be reused, i.e. that:

Proposal 1: UPIP for the EPC connected architectures using NR PDCP is configured in following way:

- LTE algorithm code point is configured in SMC as legacy LTE UE, which is used to derive KUPint.

- NR algorithm code point indicated by integrityProtAlgorithm included in RadioBearerConfig is used to configure the UP IP algorithm applied by NR PDCP to perform integrity protection.

- The integrityProtection indicated in pdcp-Config is used to activated/deactivated the UP IP, which can be changed only by DRB release and add.

With small editorial changes, this proposal is captured in the draft response LS in R2-2203369.

Question 2: Do you agree that we should reply to SA3? Do you have comments to the above proposal and draft LS? Comments/updates on the LS should still be possible in a second round of discussion.

| Company | Reply to SA3 Yes/No | comments |
| --- | --- | --- |

## 3.3	Do the CRs submitted to this meeting cover all the changes needed in RAN 2’s specifications?

R2-2202717, 2718, 2719, 2720 and 2721 provide (respectively) CRs to TSs 36.331, 38.331, 36.300, 37.340 and 38.323.

Are changes to other specifications required?

Do the changes within these CRs cover all sections that need updating?

Question 3: Are changes to other specifications needed? Do other sections in these specifications need to be updated?

| Company | comments |
| --- | --- |

Question 4: Any comments to these CRs? Comments/updates on the CRs should still be possible in a second round of discussion.

| Company | comments |
| --- | --- |

## 3.4	LS dialogue between RAN3 and SA3 and SA2

RAN3 responded to the SA3 LS (S3-214462/R2-2200153) in R2-2202145/R3-221473 asking questions of SA3 and SA2.

SA2 have replied to SA3 and RAN3 in S2-2201518, and a reply from SA3 to RAN3 is anticipated soon.

It is believed that all these 3 LSs can be “noted” by RAN2.

Question 5: Please comment ONLY if you believe any of these LSs need handling by RAN2.

| Company | comments |
| --- | --- |

# 4. Summary

Based on the discussion we make the following proposals:

- To be added…

# 5. Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Vodafone (Rapporteur) | chris.pudney@vodafone.com |