3GPP TSG-RAN WG2 Meeting #117-e	 draft R2-22xxxxx

Online Meeting, Feb 21th – March 3rd, 2022

Agenda item:	8.11.6

Source:	Ericsson

Title:	[AT117-e][602][POS] NavIC running CRs (Ericsson/Huawei)

WID/SID:	NR_pos_enh-Core - Release 17

Document for:	Discussion and Agreement

# 1	Introduction

This document is to kick off the following email discussion:

- [AT117-e][602][POS] NavIC running CRs (Ericsson/Huawei)

Scope: Review the following CRs, collect comments, and update if necessary:

- R2-2202607 (NavIC introduction to 38.305)

- R2-2203710 (NavIC introduction to 38.331)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

In this email discussion the following contributions related with A-GNSS enhancements, posSIB additions RRC impacts and stage 2 TS 38.305 impacts,

- R2-2203710		Impacts of NavIC in NR RRC	Ericsson	discussion	Rel-17

- R2-2202607		Draft running CR for stage2 spec for NavIC in R17 positioning		Huawei, Hisilicon	draftCR	Rel-17		38.305		16.6.0	B	NR_pos_enh-Core

# 2	Contact Information

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| CATT | Jianxiang Li (lijianxiang@catt.cn) |
| Intel | Yi.guo@intel.com |

# 3	Discussion

## 3.1	Impacts of NavIC in NR RRC

R2-2203710 provides the solution related with the support of NavIC in NR RRC protocol. The following two SIBs are suggested to be added in the PosSystemInformation-r16-IEs, PosSI-SchedulingInfo and DedicatedSIBRequest:

| posSibType2-24 | NavIC-DifferentialCorrections |
| --- | --- |
| posSibType2-25 | NavIC-GridModelParameter |

The changes impacting RRC is primarily to add the NavIC SIBs for broadcast.

Question 1: Please provide your views on adding the above NavIC posSIBs in RRC protocol.

| Company | Comments |
| --- | --- |
| CATT | For IE DedicatedSIBRequest and PosSI-SchedulingInfo, the posSIB should be named as posSibType2-24 and posSibType2-25, not including ‘-v17xy’. After posSibType2-25-v17xy and before the bracket, there should only be one space. posSibType5-1, posSibType6-1, posSibType6-2, posSibType6-3,..., posSibType2-24-v17xy, posSibType2-25-v17xy } For IE PosSystemInformation-r16-IEs, the posSIB should be named as posSibType2-24-r17 and posSibType2-25-r17, not including ‘-v17xy’. [[ posSib2-24-v17xy SIBpos-r16, posSib2-25-v17xy SIBpos-r16 ]] |
| Intel | To CATT, the changes in endorsed CR are same as LTE RRC spec, i.e. with -v127xy. |

## 3.2	Impacts of NavIC in TS 38.305

R2-2202607 provides the solution related with the support of NavIC in TS 38.305. The contribution captures the following spec impacts:

- IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1 as the reference file is added into section 2 as reference.

- The abbreviation of NavIC is added into section 3.2.

- NavIC is included into Regional navigation satellite systems for GNSS positioning methods. The following sections that are affected by the introduction of NavIC are pointed out and the summarized modified part:

| Section name | Impact description |
| --- | --- |
| 4.3.2 Network-assisted GNSS methods | Navigation with Indian Constellation (NavIC) is added as one of the regional navigation satellite systems. |
| ### 8.1.1 General | NavIC is added as one of the supported GNSSs of the TS 38.305. |
| ##### 8.1.2.1.3 Ionospheric Models | NavIC is added as one of the GNSSs which need Ionospheric Model assistance. |
| ##### 8.1.2.1.5 GNSS-GNSS Time Offsets | NavIC is added as one of the GNSSs which need GNSS-GNSS Time Offsets assistance. |
| ##### 8.1.3.3.1 LMF initiated Location Information Transfer Procedure | NavIC is added as one of the positioning methods. |

The changes impacting stage 2 spec is primarily to support NavIC in Rel-17 positioning.

Question 2: Please provide your views on adding the above NavIC impacted corrections in TS 38.305.

| Company | Comments |
| --- | --- |
| CATT | The reference file for NavIC should not be inserted between [35] and [36]. It should be added as the last reference file after [41]. For clauses 8.1.2.1.3 and 8.1.2.1.4, the ‘and’ before BDS should be deleted. ##### 8.1.2.1.3 Ionospheric Models Ionospheric Model assistance provides the GNSS receiver with parameters to model the propagation delay of the GNSS signals through the ionosphere. Ionospheric Model parameters as specified by GPS [5], Galileo [8], QZSS [10], BDS [20] [34], and NavIC [xx] may be provided. ##### 8.1.2.1.5 GNSS-GNSS Time Offsets GNSS-GNSS Time Offsets assistance provides the GNSS receiver with parameters to correlate GNSS time (where the specific GNSS is indicated by a GNSS-1 ID) of one GNSS with other GNSS time (where the specific GNSS is indicated by a GNSS-2 ID). GNSS-GNSS Time Offsets parameters as specified by GPS [5], Galileo [8], GLONASS [9], QZSS [10], BDS [20] [34], and NavIC [xx] may be provided. |
| Intel | Do we need “NAVIC” in 8.1.2.1.1 reference time? |

## 3.3	Any other comments

Question 3: please provide any additional comment; e.g. any additional impacts foreseen

| Company | Comments |
| --- | --- |

# 4	Conclusion