3GPP TSG RAN WG2 Meeting #117-e                                          R2-220xxxx

# Electronic meeting, 21 Feb- 3 March, 2022

Agenda item:			8.11.1

Source:			Intel Corporation

Title:		Report of 	[AT117-e][632][POS] Merged CR to 38.305 (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the report of following offline discussion:

- [AT117-e][632][POS] Merged CR to 38.305 (Intel)

Scope: Merge the endorsed positioning CRs to 38.305.

Intended outcome: Agreeable CR

Deadline:  Wednesday 2022-03-02 1000 UTC

# Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |

# Discussion

## Summary

### 3.1.1 RAT dependent positioning

Additional changes:

On demand PRS:

1 Add a Stage 2 note clarifying the difference between index-based and explicit-based on-demand PRS requests. (based on RAN2 aqgreements)

2 The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated. (based on comments received in RAN2#117-e604)

TEG definition (capture RAN1 agreements):

“

- A “Rx TEG” is associated with one or more measurements obtained from one or multiple received RS resources. The Rx timing error differences between any pair of the measurements belonging to the same Rx TEG are within a certain margin.

- A “Tx TEG” is associated with one or more transmitted RS resources.  The Tx timing error differences between any pair of the RS resources belonging to the same Tx TEG are within a certain margin.

- The “group” means that for a set of multiple measurements or a set of multiple RS resources, if the error difference between any pair within the set is within the margin, the set is intuitively considered as timing error group, and is associated with a TEG ID.

- The definitions of the Tx/Rx timing delays/errors and Rx/Tx/RxTx TEGs in RAN2’s LS that RAN2 plans on using as a baseline are correct with the following changes.

  - UE RxTx ‘timing error group’ (UE RxTx TEG): Rx timing errors and Tx timing errors, associated with UE reporting of one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' differences within a certain margin

  - TRP RxTx ‘timing error group’ (TRP RxTx TEG): Rx timing errors and Tx timing errors, associated with TRP reporting of one or more gNB Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' differences within a certain margin

”

Merged endorsed CR

[AT117-e][604][POS] RAT-dependent positioning running CR to 38.305 (Intel)

Scope: Review and update the CR in R2-2202490.

Intended outcome: Endorsable CR in R2-2203605

Deadline:  Friday 2022-02-25 1000 UTC

R2-2203605	Running 38.305 CR for Positioning WI on RAT dependent positioning methods	Intel Corporation	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

- Endorsed

### 3.1.2 GNSS integrity

Merged endorsed CR

R2-2203604	Running CR of 38.305 for GNSS Positioning Integrity	InterDigital, Inc.	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

- Endorsed

[AT117-e][603][POS] Integrity stage 2 CRs (InterDigital)

Scope: Review and update the following CRs:

- R2-2202861 (integrity introduction to 36.305)

- R2-2202862 (integrity introduction to 38.305)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

### 3.1.3 A-GNSS enhancements

Merged endorsed CRs

R2-2203611	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilicon, Intel Corporation, ZTE Corporation, CBN, vivo, OPPO, Lenovo, MediaTek Inc, Spreadtrum Communications, Xiaomi.	CR	Rel-17	38.305	16.7.0	0084	1	B	NR_pos_enh-Core	R2-2109485

- Endorsed

[AT117-e][601][POS] BDS running CRs (CATT)

Scope: Review the following CRs, collect comments, and update if necessary:

- R2-2202402 (BDS introduction to 37.355)

- R2-2202403 (BDS introduction to 36.305)

- R2-2202404 (BDS introduction to 38.305)

Intended outcome: Endorsable CRs and report in R2-2203612

Deadline:  Friday 2022-02-25 1000 UTC

R2-2203615	Draft running CR for stage2 spec for NAVIC in R17 positioning	Huawei, HiSilicon	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

- Endorsed

[AT117-e][602][POS] NavIC running CRs (Ericsson/Huawei)

Scope: Review the following CRs, collect comments, and update if necessary:

- R2-2202607 (NavIC introduction to 38.305)

- R2-2203710 (NavIC introduction to 38.331)

Intended outcome: Endorsable CRs and report in R2-2203608

Deadline:  Friday 2022-02-25 1000 UTC

## 3.2 Comments on the merged CR

Discussion point : Companies are invited to provide view on the merged version?

| Company’s name | Section | Identified issues | Change suggestion |
| --- | --- | --- | --- |

# Summary report and proposals