3GPP TSG-RAN WG2 #118-e	R2-2206016

Electronic meeting, May 9 – 20, 2022

Agenda Item:	6.22.4

Source:	Ericsson

Title:	UE capabilities for MGE

Document for:	Discussion

# 1	Introduction

This contribution addresses the remaining details concerning UE capabilities for MGE. In particular, we come back to FFSs that were left, from RAN2#117-e’s email discussion (see R2-2203758).

# 2	Discussion

## 2.1	UE capability to support NCSG pattern

On RAN4’s latest LS on NCSG in R2-2204135 (R4-2206890), RAN4 informs RAN2 that:

- On top of agreed UE capability on NCSG support 19-1 as captured in R4-2202400, RAN4 further agreed on the following capabilities: Index; Feature group; Components; Prerequisite feature groups; Need for the gNB to know if the feature is supported; Applicable to the capability signalling exchange between Ues (V2X WI only)”.; Consequence if the feature is not configured by the UE; Type; Need of FDD/TDD differentiation; Need of FR1/FR2 differentiation; Capability interpretation for mixture of FDD/TDD and/or FR1/FR2; Note; Mandatory/Optional. 19-X; Network controlled small gap (NCSG); Support of per-FR NCSG; 19-1; yes; no; UE cannot be configured with per-FR NCSG; per-UE; No; No; Optional with capability signalling. 19-X+1; Network controlled small gap (NCSG); Supported NCSG patterns; 19-1; yes; no; Network does not know whether some NCSG patterns can be configured to UE; per-UE; No; No; Optional with capability signalling NCSG patterns #0, #1 are conditional mandatory if UE supports 19-1 NCSG patterns #13, #14 are conditional mandatory if UE supports 19-1 and 19-X. 19-X+2; Network controlled small gap (NCSG); Supported NR-only NCSG patterns; 19-1; yes; no; Network does not know whether some NR-only NCSG patterns can be configured to UE; per-UE; No; No; Optional with capability signalling NCSG patterns #2 and #3 are conditional mandatory if UE supports 19-1 NCSG patterns #17 and #18 are conditional mandatory if UE supports 19-1 and a FR2 band. Note: It may be a bit redundant for UE who already reports 19-X+1 to report 19-1 additionally. It is up to RAN2 for signaling overhead reduction.

While the following agreement was reached in RAN2#117-e:

introduce (A) UE capability to support NCSG as indicated in RAN4 feature list as a baseline. It can be removed if no longer needed after more input from RAN4 on (B) UE capability to support NCSG pattern.

On this matter, we understand RAN4’s note highlighted above, i.e., reporting 19-1 together with 19-X+1 is redundant.

In addition, we would rather prefer to have single capability linked to the feature support, as this reduces the spec complexity. However, we do not see a clear motivation to include the indication that “per-FR NCSG” is supported. Hence, perhaps adapting 19-1 to include the (other) related conditional mandatory patterns in the 19-X+1 could a priori be enough instead of indicating the supported NCSG pattern(s).

RAN2 to indicate a single NCSG-related capability to signal support of the feature.

To indicate support of NCSG, RAN2 to choose between: a) general UE capability (e.g., by modifying 19-1’s Mandatory/Optional description accordingly) or, b) the NCSG pattern(s) indication (bitmap).

## 2.2	Pre-configured MG

### 2.2.1	FR differentiation

It was proposed by one company in RAN2#117-e’s email discussion (see R2-2203758) to have the pre-configured MG capability to be “FR differentiated”. We however see no clear motivation to have it that way, nor we observe any suggestion from RAN4 to consider such differentiation (RAN4’s feature list can be found in R4-2206572/R2-2204132).

Pre-configured MG capability to remain as a perUE-type.

### 2.2.2	Separate UE capability for CA and non-CA case

As it is the case for the “per FR differentiation” for pre-configured MG. It was also proposed by (another) single company in RAN2#117-e’s email discussion (see R2-2203758) to introduce separate UE capabilities for the CA and non-CA case. Again, we see no clear motivation to have such indication(s), since defining separate capabilities for each scenarios adds complexity. Additionally, this have not been triggered by RAN4. Thus, the following.

There is no need to have a separate UE capability (for pre-configured MG) to indicate CA and non-CA scenarios.

## 2.3	Concurrent gaps: “Index 2” indication

It was also proposed by one company in RAN2#117-e’s email discussion (see R2-2203758) to introduce two separate capabilities for concurrent gaps:

- a “concurrent_perFR” to signal support of the entire table provided by RAN4, and

- a “concurrent_perUE” to indicate that only index-2 from the table is supported.

The table provided by RAN4 is depicted below (see related LS from RAN4 in R2-2202158/R4-2202604)

| Combinations of different gap types for per-FR gap capable UE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Index | # of simultaneous MG |  |  | RAN4 conclusion |
|  | Per-FR1 | Per-FR2 | Per-UE |  |
| 0 | 2 | 1 | 0 | Supported |
| 1 | 1 | 2 | 0 | Supported |
| 2 | 0 | 0 | 2 | Supported |
| 3 | 1 | 0 | 1 | Supported when per-UE gap is associated to PRS measurement |
| 4 | 0 | 1 | 1 |  |
| 5 | 1 | 1 | 1 |  |
| 6 | 0 | 0 | 1 | Supported |
| 7 | 1 | 1 | 0 | Supported |
| 8 | 1 | 0 | 0 | Supported |
| 9 | 0 | 1 | 0 | Supported |
| 10 | 2 | 0 | 0 | Supported |
| 11 | 0 | 2 | 0 | Supported |

However, RAN2 has already agreed on a single UE capability for concurrent measurement gap, and we see no real motivation have such differentiation, especially since RAN4 have not suggested the opposite. Hence, the following proposal.

RAN2 to stick to a single/general UE capability for concurrent measurement gaps.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to indicate a single NCSG-related capability to signal support of the feature.

Proposal 2	To indicate support of NCSG, RAN2 to choose between: a) general UE capability (e.g., by modifying 19-1’s Mandatory/Optional description accordingly) or, b) the NCSG pattern(s) indication (bitmap).

Proposal 3	Pre-configured MG capability to remain as a perUE-type.

Proposal 4	There is no need to have a separate UE capability (for pre-configured MG) to indicate CA and non-CA scenarios.

Proposal 5	RAN2 to stick to a single/general UE capability for concurrent measurement gaps.

# 4	References

R2-2204135 (R4-2206890), “LS on R17 MG enhancement - NCSG”, RAN4, RAN4#102-e, Feb. 21 – Mar. 3, 2022

R2-2203758, “[AT117-e][020][MGE] UE capabilites”, Intel Corporation, RAN2#117-e, Feb. 21 – Mar. 3, 2022

R4-2206572/R2-2204132, “Rel-17 RAN4 UE feature list for NR”, RAN4, RAN4#102-e, Feb. 21 – Mar. 3, 2022