3GPP TSG-RAN WG2#116-e	R2-2206141

Online, 9 - 20 May 2022

Title: 	[38.331 - H067] Update of candidate target cell and configuration

Source: 	Huawei, HiSilicon

Agenda item:	6.2.2

Document Type:	Discussion and Decision

# 1. Introduction

This documents considers issue H067 for CPAC: Remove the ToReleaseList. Discuss about the ToAddModList.

# 2 Discussion

RAN3 had the following agreements:

In CPA and MN/SN initiated inter-SN CPC, target SN can: Update/modify previous CPAC configurations provided in CPAC addition using SN initiated SN modification procedure Add prepared PSCells within the limit given by the MN or source SN using SN initiated SN modification procedure Cancel some of the prepared PSCells using SN initiated SN modification procedure. Cancel all prepared PSCells using SN initiated SN release procedure

RAN3 has agreed that the target SN can cancel some of the prepared PSCell using SN initiated SN modification procedure and also RAN3 has added the Conditional PSCell Change Information Required IE in the S-NODE CHANGE REQUIRED message to provide the configuration update for the list of PSCells prepared at the target SN, as captured in TS 38.323 clause 8.3.4.2:

If the Conditional PSCell Change Information Required IE is included in the S-NODE MODIFICATION REQUIRED message, the M-NG-RAN node shall, if supported, consider that the request provides the configuration update for the list of PSCells prepared at the target SN, as described in TS 37.340 [8].

RAN2 previously added the field cg-CandidateToReleaseList in the CG-CandidateList message to indicate the cancelled PSCell by the target SN, and changed cg-CandidateList to cg-CandidateToAddModList in order to allow indicating the configurations to be changed.

With such signalling, there would be no need for the RAN3 IE, hence we understand that the RAN3 IE indicates that the previous list of configurations should be completely overridden by the new list and there is no need to use the structures introduced by RAN2.

RAN3 also added the possibility for the MN to provide the information from the target SN to the source SN via the Conditional PSCell Change Information Update IE in the S-NODE MODIFICATION REQUEST message, as captured in TS 38.432 caluse 8.3.3.2:

If the Conditional PSCell Change Information Update IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request provides the list of PSCells prepared at the target S-NG-RAN node, as described in TS 37.340 [8].

Therefore, we see no need for the delta signalling in cg-CandidateList.

Proposal: Remove the ToAddModList/ToReleaseList structure in cg-CandidateList.

# 3	Conclusion

We propose the following:

Proposal: Remove the ToAddModList/ToReleaseList structure in cg-CandidateList.

# 4	TP for 38.331

### 11.2.2	Message definitions

#### –	CG-CandidateList

This message is used to transfer the SCG radio configuration for one or more candidate cells for Conditional PSCell Addition (CPA) or Conditional PSCell Change (CPC) as generated by the candidate target SgNB.

Direction: Secondary gNB to master gNB or eNB.

CG-CandidateList message

-- ASN1START

-- TAG-CG-CANDIDATELIST-START

CG-CandidateList ::=                SEQUENCE {

criticalExtensions                  CHOICE {

c1                                  CHOICE{

cg-CandidateList                    CG-CandidateList-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

CG-CandidateList-IEs ::=            SEQUENCE {

cg-CandidateList-r17        SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CG-CandidateInfo-r17    OPTIONAL,

nonCriticalExtension                SEQUENCE {}                                                          OPTIONAL

}

CG-CandidateInfo-r17 ::=            SEQUENCE {

candidateCG-Config-r17             OCTET STRING (CONTAINING CG-Config)

}

CG-CandidateInfoId-r17::=           SEQUENCE {

ssbFrequency-r17                    ARFCN-ValueNR,

physCellId-r17                      PhysCellId

}

-- TAG-CG-CANDIDATELIST-STOP

-- ASN1STOP

CG-CandidateList field descriptions

cg-CandidateList Contains information regarding candidate target cells for Conditional PSCell Addition (CPA) or Conditional PSCell Change (CPC) from the candidate target secondary node to the master node.