3GPP TSG RAN WG2#118-e	R2-2206810

e-Meeting, 9th - 20th May, 2022

3GPP TSG RAN WG1 #109-e	R1-2205619

e-Meeting, May 9th – 20th, 2022

Title:	Reply LS on expected AoA and AoD parameters

Response to:	R1-2205493 (R2-2206390)

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG1

To:	RAN WG2

Cc:	RAN WG3

Contact Person:

#### Name:	Ryan Keating

E-mail Address:	ryan.keating@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

1. Overall Description:

RAN1 would like to thank RAN2 for LS R1-2205493 (R2-2206390) on expected AoA and AoD parameters. RAN1 has discussed the questions asked by RAN2 in the LS. Note that RAN1 assumes RAN2 is referring only to expected DL AoA/AoD in their questions.

Question 1: Is the expected AoA/AoD information provided per TRP or per DL-PRS resource?

RAN1 Answer: The expected AoA/AoD information should be provided to the UE per TRP.

Question 2: Whether the uncertainty field for expected AoD (expected-DL-Azimuth-AoD-Unc and expected-DL-Zenith-AoD-Unc) and expected AoA (expected-DL-Azimuth-AoA-Unc and expected-DL-Zenith-AoA-Unc) can be optional?

RAN1 Answer: RAN1 assumes that the uncertainty field for the expected AoD (expected-DL-Azimuth-AoD-Unc and expected-DL-Zenith-AoD-Unc) and expected AoA (expected-DL-Azimuth-AoA-Unc and expected-DL-Zenith-AoA-Unc) can be optional under the condition that omitting the field means maximum uncertainty.

2. Actions:

To RAN2

ACTION:

- RAN1 respectfully askes RAN2 to take the provided answers into account during their work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110	22 Aug – 26 Aug 2022	Toulouse, France

TSG-RAN WG1 Meeting #110-e-Bis	10 Oct – 19 Oct 2022	E-Meeting