3GPP TSG-RAN WG2 Meeting #116 electronic						R2-2xxxxxx

Online, November 1-12, 2021

Agenda Item:	xx

Source: 			Session chair (CMCC)

Title: 			Report from SON/MDT session

Document for:	Approval

Organizational:

- LSs – contact companies should flag LSs that need presenting.  Otherwise we will directly note them

- Running CRs will be endorsed to be used as baseline and moved to email discussion.  Further agreements will be captured on that baseline CR.

- Only Email discussions and summary discussions will be treated during e-meetings (indicated clearly in the meeting notes)

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT116][800][SON/MDT] Organizational Hu

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions related to SON/MDT

    - Share meetings notes and agreements for review and endorsement

## 6.13	SON/MDT

(NR_ENDC_SON_MDT_enh-Core; leading WG: RAN3; REL-17; WID: RP-201281)

Tdoc Limitation: 5 tdocs

WI is declared 100% complete

### 6.13.1	Organizational

Tdoc Limitation: 0

LS in etc

R2-2204405	Reply LS on UE context keeping in the source cell (R3-212944; contact: Ericsson)	RAN3	LS in	Rel-17	To:RAN2

R2-2204406	LS Reply on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (R3-214429; contact: Ericsson)	RAN3	LS in	Rel-17	To:SA5, RAN2

R2-2204407	Reply LS on scenarios need to be supported for MRO in SCG Failure Report (R3-216159; contact: Samsung)	RAN3	LS in	Rel-17	To:RAN2

R2-2204408	Reply LS on Area scope configuration and Frequency band info in MDT configuration (R3-221178; contact: Huawei)	RAN3	LS in	Rel-17	To:RAN2

R2-2204409	Reply LS to SA5 on beam measurement reports (R3-221383; contact Ericsson)	RAN3	LS in	Rel-17	To:SA5	Cc:RAN2

R2-2204412	Reply LS on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (S5-213499; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2204413	Reply LS on Report Amount for M4, M5, M6, M7 measurements (S5-214523; contact: Nokia)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2204414	Reply LS on the details of logging forms reported by the gNB-CU-CP, gNB-CU-UP and gNB-DU under measurement pollution conditions (S5-215493; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2204415	Reply LS on the Beam measurement reports for the MDT measurements (S5-216628; contact: Ericsson)	SA5	LS in	Rel-17	To:RAN3	Cc:RAN2

R2-2204448	LS on UP measurements for Successful Handover Report (R3-212935; contact: Ericsson)	RAN3	LS in	Rel-17	To:RAN2

R2-2204498	Reply LS on MDT M6 calculation for split bearers in MR-DC (R3-222868; contact: Ericsson)	RAN3	LS in	Rel-17	To:RAN2, SA5

### 6.13.2	CRs and Rapporteur Resolutions

Tdoc Limitation: 0.

CR Rapporteurs to provide input CRs, and Provide resolution proposals for smaller and editorial corrections.

R2-2205903	Miscellaneous rapporteur corrections for SON-MDT	Ericsson	CR	Rel-17	38.331	17.0.0	3136	-	F	NR_ENDC_SON_MDT_enh-Core

### 6.13.3	SON Corrections

- [AT118e][801][SON/MDT] Summary of RILs discussion (Ericsson, Huawei)

For 1st round: Based on pre118 #801 email discussion, figure out the controversial RILs and RILs with easy agreeable proposals.

For 2nd round: Discuss NOKIA094, H073, Z422 and proposal 3-19 in R2-2206344 and try to get consensus on these issues.

For 3rd round: “To be discussed how/where information associated to the visited PSCells [ensuring the chronological order] should be captured in the spec”

1st round  Intended outcome: Report full of proposals…

1st round Deadline: 11:23 UTC, Tuesday May 10th

2nd round  Intended outcome: Report full of proposals…

2nd round Deadline: 11:23 UTC, Monday May 16th

3rd round  Intended outcome: Report with only one proposal…

3rd round Deadline: 11:23 UTC, Wed May 18th

R2-2206344	Summary of RAN2-118 #801 offline discussion	Ericsson  Rel-17	NR_ENDC_SON_MDT_enh-Core

The following RILs are FFS through email discussion: NOKIA094, H073, Z422.

Agreements:

1	The changes associated to the following RILs (as captured in the rapporteur CR):

•	E070, E071, E072, E142, E113, E075, E078, E115, E116, E117, E118, E119, E123, E137, E069

•	S712, S703, S706, S713, S714, S707, S709, S710, S716 (Z413, E123)

•	Z403, Z404, Z405, Z414, Z416, Z417, Z420, Z413, Z424, Z418, Z415, Z421

•	H069, H070, H075, H588, H077, H079, H070, H098, H101, H195, H100, H076

•	N025, N029, N030, N091, NOKIA095, NOKIA100,

•	C301, C310, C308, C303, C304, C305, C320, C311, C312, C321, C329

•	I044, I006

•	B181

2	RAN2 to reject the changes associated to the following RILs:

•	S701, S702, S704, S705, S708, S715

•	W006

•	N027, N025, N028, NOKIA093, N029, N092, NOKIA097

•	H071, H072, H104, H108

•	C316

•	O700, O701

•	B180

R2-2206464	Summary of RILs discussion	Ericsson, Huawei

Agreements:

1	The following options provides more information to protect the signalling based MDT configuration and result.

•	BOOLEAN OPTIONAL

2	For the Rel.17 PSCell MHI, the PSCells visited while connected to a certain PCell X are appended to the certain PCell X List in the MHI in the nested structure when the corresponding PCell X related entries are appended to the MHI. To be discussed how/where information associated to the visited PSCells [ensuring the chronological order] should be captured in the spec.

3	RAN2 agree to stick to the agreement reached in RAN2#117, on the inclusion of CHO configuration in the RLF report for the case of too late CHO, unless a new LS is received from RAN3.

4	RAN2 agree to keep the current implementation in the rapporteur CR i.e., UE logs the HO type at the time of handover failure.

5	[H588]  RAN2 postpone the inclusion of all the Rel-17 SIBs in the RA report for SI request to the next release.

FFS on “To be discussed how/where information associated to the visited PSCells [ensuring the chronological order] should be captured in the spec.” related to the above agreement 2 through email #801.

Agreements:

4	  No need to pursue [Z422], the msgA-SCS-From-prach-ConfigurationIndex is kept.

Proposal 5	  [E079] is not pursued in Rel-17, i.e. UE does not need to log the RA-InformationCommon of the RACH performed toward source cell during the DAPS HO, in the RLF report.

6	  [H099] is not pursued in Rel-17 i.e., RAN2 agree that separately handling the SN and MN related RACH report is not supported.

7	  [H073] UE checks the availability of the SHR, at the time of reporting.

8	  RAN2 postpone [C327] (i.e., the Inter-RAT mobility (MobilityFromNR) failure when the UE is configured with CHO configuration) to the next release.

9	  RAN2 agree that the UE logs the timeSinceFailure for each CEF report as a time elapsed between the point in time of the last CEF occurrence for a given entry in the CEF report list to the point in time that CEF report list is fetched by the network (i.e., until UE Information Response procedure)

10	  RAN2 agree to update the procedural text to include CHO and DAPS related RLF report contents in the RLF report based on UE capability

12	  [Z410] is not pursued i.e., no change to the current specification.

13	  [H074] RAN2 agree that UE includes the UPInterruptionTimeATHO only if a PDCP PDU was received from the source cell of the concerned HO, and a non-duplicated PDCP PDU was received from the target cell of the concerned HO.

14	  [NOKIA096] RAN2 agree that all the SHR triggering thresholds including T304, T310 and T312 thresholds are applicable to the DAPS HO, beside the sourceDAPS-FailureReporting (i.e., no changes to the current spec needed).

15	  [C326] not pursued. RAN2 agree to stick to the current agreement that the CHOCandidateCellList are not included in the SHR.

16	  [H106] RAN2 agree that timeSinceCHOReconfig will be logged when configuration of conditional handover is available in VarConditionalReconfig at the moment of declaring the radio link failure, or handover failure (i.e., no changes to the current spec).

17	  [C322 and E076] RAN agree that setting of the 2-step parameters can be moved before the setting of the 4-step RA parameters.

18	  [H099] RAN2 agree to postpone the issue related to the splitting the IF condition for the setting of the msg1-SubcarrierSpacing. RAN2 can revisit it if needed

R2-2205363	[Z408] Consideration on RLF-report for CHO-DAPS	ZTE Corporation, Sanechips	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2204942	[C328] Clarification on CHO Information Logging in Neighbour Cell Measurement in RLF Report	CATT	draftCR	Rel-17	38.331	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2205894	Corrections to mobility history information reporting [E120, E121, E122]	Ericsson	CR	Rel-17	38.331	17.0.0	3131	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2204880	Clarification on SHR for DAPS HO	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2204876	[N030] Correction to conditions determining successful handover report	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.0.0	3011	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2204877	[N028] Correction to SuccessHO-Config	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.0.0	3012	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2204878	Correction to Mobility History Information setting [N094][N095][E122][H072]	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	17.0.0	3013	-	F	NR_ENDC_SON_MDT_enh-Core	Late

R2-2204879	Corrections on Rel-17 RLFreport	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2204883	RIL: [S702] [S703] [S704] [S705] [S706] [S707] [S708]	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2204884	RIL: [S709][S710][S711][S712]	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2204885	RIL: [S713][S714][S715]Using ENUMERATED {true} instead of BOOLEAN	Samsung	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2204938	[C320] Add SgNB RA Report related Information	CATT	draftCR	Rel-17	38.331	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2204939	Addition of SON Features Enhancement in Stage 2	CATT	draftCR	Rel-17	38.300	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2204940	[C326] Clarification on CHO Candidate Cell List in SHR	CATT	draftCR	Rel-17	38.331	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2204941	[C327] Correction on CHO Information Logging for Mobility from NR Failure	CATT	draftCR	Rel-17	38.331	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2204966	[B180] TP for reporting CHO execution condition in RLF report	Lenovo	discussion	Rel-17

R2-2204967	[B181] TP for reporting time information for SCG failure	Lenovo	discussion	Rel-17

R2-2205046	[W006] Correction on the generation and discarding of SHR	NEC	CR	Rel-17	38.331	17.0.0	3035	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205072	Discussion on SON with DAPS [S704]	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2205074	Introduction of SHR in TS 38.300	Huawei, HiSilicon	CR	Rel-17	38.300	17.0.0	0453	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205075	Corrections on TS 37.320 for SON and MDT	Huawei, HiSilicon	CR	Rel-17	37.320	17.0.0	0118	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205361	[Z401][Z413] Consideration on multiple CEF report	ZTE Corporation, Sanechips	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2205362	[Z421] Consideration on RA report	ZTE Corporation, Sanechips	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2205364	Draft CR to 38331 on RLF-report	ZTE Corporation, Sanechips	draftCR	Rel-17	38.331	17.0.0	F	NR_ENDC_SON_MDT_enh-Core

R2-2205704	HO related SON corrections 	Qualcomm Incorporated 	discussion	Rel-17

R2-2205892	Corrections to 2-step RA Report [E076][E078]	Ericsson	CR	Rel-17	38.331	17.0.0	3130	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205893	Corrections to Mobility History Information [E120][E121][E122]	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2205901	Including source-related RA-Information in SHR [E079]	Ericsson	CR	Rel-17	38.331	17.0.0	3135	-	F	NR_ENDC_SON_MDT_enh-Core

### 6.13.4	MDT Corrections

R2-2204943	The Correction on TS37.320	CATT	discussion	Rel-17	37.320	NR_ENDC_SON_MDT_enh-Core

R2-2205076	Corrections on TS.38.314 for the delay measurement for split bearer	Huawei, HiSilicon	CR	Rel-17	38.314	17.0.0	0023	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205689	TP on L2 measurements for total RAN delay calculation	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2205686	TP on signalling based logged MDT override protection in inter-PLMN scenarios [RIL number E069]	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2205687	TP on IDC issues in logged MDT [RIL number E074]	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2205688	TP on multiple CEF reports [RIL number E075 and E123]	Ericsson	discussion	NR_ENDC_SON_MDT_enh-Core

R2-2205736	[S716] Remaining issues on multiple CEF reports	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2205738	[S701] Remaining issues on signalling based MDT protection	Samsung Electronics Co., Ltd	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

### 6.13.5	UE Capabilities

Initial discussion on Features / UE caps developed in RAN2, if any. Note that this AI is complementary to AI 6.0.2.

- [AT118e][802][SON/MDT] UE capabilities related to SONMDT (CATT)

Based on R2-2204944, R2-220494, R2-2205073 and R2-2205705, discuss the necessary changes to 38.306 and 38.331

Intended outcome: Report and agreeable CRs

Deadline: 11:23 UTC, Monday May 16th

R2-2204944	[C329] Add MDT related UE Capabilities of EMR	CATT	draftCR	Rel-17	38.331	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2204945	Add MDT related UE Capabilities of EMR	CATT	draftCR	Rel-17	38.306	17.0.0	NR_ENDC_SON_MDT_enh-Core

R2-2205073	Corrections on TS 38.306 for UE capabilities for SON and MDT	Huawei, HiSilicon	CR	Rel-17	38.306	17.0.0	0707	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205705	SON MDT UE Capabilities 	Qualcomm Incorporated	discussion	Rel-17

### 6.13.6	Other

R2-2204664	CR to 38314 on RA preamble measurement	ZTE Corporation, Sanechips, CMCC	CR	Rel-17	38.314	17.0.0	0022	-	F	NR_ENDC_SON_MDT_enh-Core

R2-2205222	Discussion on PSCell MHI recording for RRC inactive state	Sharp	discussion	R2-2202939

R2-2205567	Addition of Last Serving Beam in RLF Report	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2205566	Inclusion of the CSI reports in MDT framework	Nokia, Nokia Shanghai Bell	discussion	Rel-17	TEI17

## 5.4	SON/MDT support for NR

(NR_SON_MDT-Core; leading WG: RAN3; REL-16; started: Jun 19; Completed June 20; WID: RP-191776).

Documents in this agenda item will be handled in a break out session

Tdoc Limitation: See tdoc limitation for Agenda Item 5

### 5.4.1	General and stage-2 corrections

Including incoming LSs, TS 37.320 corrections

### 5.4.2	TS 38.314 corrections

### 5.4.3	RRC corrections

R2-2204548	Corrections to SON/MDT capabilities 	Lenovo	CR	Rel-16	38.306	16.8.0	0675	1	F	NR_SON_MDT-Core	R2-2202223

R2-2204549	Corrections to SON/MDT capabilities	Lenovo	CR	Rel-17	38.306	17.0.0	0699	-	A	NR_SON_MDT-Core

R2-2204589	Corrections on LTE UE RLF Report	China Telecom, CATT, Ericsson, ZTE	discussion

R2-2204594	Corrections on LTE UE RLF Report	China Telecom, CATT, Ericsson, ZTE	CR	Rel-16	38.331	16.8.0	2976	-	F	NR_SON_MDT-Core

R2-2204595	Corrections on LTE UE RLF Report	China Telecom, CATT, Ericsson, ZTE	CR	Rel-17	38.331	17.0.0	2977	-	A	NR_SON_MDT-Core

R2-2204672	[O700][O701] RRC corrections for MDT	OPPO	discussion	Rel-17	NR_SON_MDT-Core

R2-2204916	Correction on delay value configuration description	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3015	-	F	NR_SON_MDT-Core

R2-2204937	Add TAC into Previous Cell Information of RLF Report	CATT	draftCR	Rel-16	36.331	16.8.0	NR_SON_MDT-Core

R2-2205660	Addition of missing information into RA-InformationCommon-r16	Apple, Ericsson	CR	Rel-16	38.331	16.8.0	3108	-	F	NR_SON_MDT-Core

R2-2205661	Addition of missing information into RA-InformationCommon-r16	Apple, Ericsson	CR	Rel-17	38.331	17.0.0	3109	-	A	NR_SON_MDT-Core

R2-2205760	Discrepancy on inclusion of reconnectCellId	Samsung Electronics Co., Ltd	discussion	Rel-16	38.331	NR_SON_MDT-Core

R2-2205885	On DAPS handover failure handling	Ericsson	CR	Rel-16	38.331	16.8.0	3123	-	F	NR_SON_MDT-Core

R2-2205886	On sensor information configuration	Ericsson	CR	Rel-16	38.331	16.8.0	3124	-	F	NR_SON_MDT-Core

R2-2205887	On sensor information configuration	Ericsson	CR	Rel-17	38.331	17.0.0	3125	-	A	NR_SON_MDT-Core

R2-2205888	On including SSB and CSI-RS measurements in RLF report	Ericsson	CR	Rel-16	38.331	16.8.0	3126	-	F	NR_SON_MDT-Core

R2-2205889	On including SSB and CSI-RS measurements in RLF report	Ericsson	CR	Rel-17	38.331	17.0.0	3127	-	A	NR_SON_MDT-Core

R2-2205890	On ObtainCommonLocation related configuration	Ericsson	CR	Rel-16	38.331	16.8.0	3128	-	F	NR_SON_MDT-Core

R2-2205891	On ObtainCommonLocation related configuration	Ericsson	CR	Rel-17	38.331	17.0.0	3129	-	A	NR_SON_MDT-Core