3GPP TSG-RAN WG2 #118-e	R2-220xxxx

Electronic meeting, 9th May – 20th May 2022

Agenda Item:	5.1.4

Source:	Ericsson

Title:	[AT118-e][016][NR1516] Connection Control I (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

The following document is to provide and collect input about a way forward related to the following email discussion:

[AT118-e][016][NR1516] Connection Control I (Ericsson)

Scope: Treat R2-2205965, R2-2205966, R2-2205867, R2-2205406, R2-2205407, R2-2205868, R2-2205614, R2-2205586, R2-2205599

Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

A first round with Deadline for comments W1 Thursd May 12th 1200 UTC to settle scope what is agreeable etc
A Final round with Final deadline W2 Wednesd May 18th 1200 UTC to settle details / agree CRs etc.

# 2	Contact information

| - Company | - Name | - Email address |
| --- | --- | --- |
| - Samsung | - Seungri Jin | - seungri.jin@samsung.com |
| - Nokia |  | - amaanat.ali@nokia.com |
| - OPPO | - SHI Cong | - shicong@oppo.com |
| - Huawei, HiSilicon | - Zhenzhen Cao | - caozhenzhen@huawei.com |
| - ZTE | - LiuJing | - liu.jing30@zte.com.cn |
| - ZTE | - Fei Dong | - Dong.fei@zte.com.cn |
| - Apple | - Naveen Palle | - naveen.palle@apple.com |

# 3	Discussion

## 3.1	L1 parameters

R2-2205965	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-15	38.331	15.17.0	3140	-	F	NR_newRAT-Core, TEI16

R2-2205966	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-16	38.331	16.8.0	3141	-	A	NR_newRAT-Core, TEI16

R2-2205967	Correction of Need Code in IE SearchSpace	Ericsson	CR	Rel-17	38.331	17.0.0	3142	-	A	NR_newRAT-Core, TEI16

The CRs correct a conflict between Need Code and Field Description. 
Strictly, the proposed change is not backwards compatible.

Note there is a typo in the Rel-15 CR. CR missed to add the Need Code “S” that replaces the “R”.

Question 1: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - See the comments | - Either approach has no functional differences but this Need R with adding the description of absent condition violates the general guideline. - From our understanding, Need R without the description of absent condition is also possible (i.e. remove “If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0”) because this field is used for “Number of consecutive slots that a SearchSpace lasts in every occasion”. In other words, absent of this field, UE use the value 1 slot for monitoring of SearchSpace. - If we strictly apply the rule for handing need code, we share the view from this change but no strong view on this change. |
| - Nokia | - Yes | - This seems to have been missed and we are okay to correct this. |
| - OPPO | - Yes with comments | - It seems there is no issue if the need code for duration is Need R, because the value range starts from 2 which is the minimal value for consecutive slots, otherwise our understanding is the UE will use 1 slot. - But we also share the view that if following strictly the rule for the need code, it should be Need S. - For R15/R16 CR, are there BC issues? |
| - Huawei, HiSilicon | - Yes | - We would consider this as a typo (Agree with Samsung there is no functional difference, as Need R with the description means the same things as Need S with the description). - We suggest to not highlight this in a separate CR, i.e. can be merged into the rapporteur CR. - In the Rel-15 CR, the “S” is still missing? |
| - ZTE | - Yes | - The modification is correct and we assume all existing UEs already support the corresponding behaviour (no NBC issue). |
| - Apple | - No strong view on whether we need this change or not | - Agree with Samsung views. |

## 3.2	L2 parameters

R2-2205406	CR on 38.331 for sn-FieldLength	ZTE Corporation,Sanechips	CR	Rel-15	38.331	15.17.0	3079	-	F	NR_newRAT-Core

R2-2205407	CR on 38.331 for sn-FieldLength	ZTE Corporation,Sanechips	CR	Rel-16	38.331	16.8.0	3080	-	A	NR_newRAT-Core

The CRs proposes to correct the field description of sn-FieldLength as ‘The value of sn-FieldLength for a RLC shall be changed only using reconfiguration with sync’

Question 2: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes | - We think the proposed change is correct. |
| - Nokia | - Not sure | - We are not sure really we got the essence of the change. Is it editorial as there is no problem with interoperability but still some essential correction? - ZTE: Since this is a CR only regarding the restriction to the NW(e.g: Not allow NW to do sth), so there is no clear inter-operability, for example, if NW do something is allowed according to the change, the UE behavior is not predictable, that’s why there is no clear inter-operability. |
| - OPPO | - No | - We understand the issue is that for SN-fieldLength, the field descriptions says it can only be changed using reconfigure with sync. However, for RRC re-establishment case, the SN-FieldLength may also need to be configured by bearer type change which is not supposed to be the way of reconfiguration with sync. We share sympathy on this issue if our understanding is correct. - However, we don’t think by updating the “DRB” to “RLC” in the field description, the issue can be solved because the concerned part is the “reconfiguration with sync”. Or can the CR proponent further elaborate it? ZTE: Thanks for sympathies.firstly, yes, your understanding is correct. Not only for RRC re-establishment case, in most case, we may encounter the same issue once the bearer type change is performed. We mainly focus on resolving the issue raised in the CR (e.g the Bearer type change), in the procedure of bearer type change, the target CG shall establish a new RLC entity to associate with the DRB of the source CG. With the change ‘The value of sn-FieldLength for a RLC shall be changed only using reconfiguration with sync’, NW can set any one value of sn-FieldLength because the RLC entity in the target CG is newly established. On contrast, if the ‘DRB’ is kept as it is, the establishing a new RLC entity in the target CG is definitely not allowed according to the current wording ‘The value of sn-FieldLength for a DRB shall be changed only using reconfiguration with sync’ because the RLC entity is a part of a DRB. |
| - Huawei, HiSilicon | - No | - In the scenario mentioned in the CR, the SCG DRB is not valid which should be released, and a new MCG DRB should be added. No issue in this case we think. - ZTE: technically speaking, not only for RRC re-establishment procedure, in any case, once the barer type change is performed (e.g SCG bearer -> MCG bearer, or vice versa), the specified behavior defined in 37.340 is not allowed, please see below: - It will result that the bearer type change would be totally forbade because MN have no idea about the sn-FieldLength of the changed bearer on SN, the only thing NW can do is as you said, to release SCG DRB, and then re-establish MCG DRB. It will make the bearer type change procedure be useless. |
| - ZTE | - Yes, Proponent | - Not only for RRC re-establish procedure as mentioned in CR, in any case, once the bearer type change is performed (e.g SCG Bearer ->MCG Bearer as shown in the below table in 37.340),if NW strictly follow the current sentence ‘The value of sn-FieldLength for a DRB shall be changed only using reconfiguration with sync’ MN cannot promise to establish the RLC entity for bearer type change with a same value of sn-FieldLength because MN totally have no idea about the sn-FieldLength of the changed bearer on SN. - It will result that the bearer type change would be forbade by the current sentence. |
| - Apple | - No | - This can be handled by gNB implementation. The sn-FieldLength is a RLC parameter, so there does not seem to be much room for misinterpretation. Besides to replace “DRB” by just “RLC” is not correct, it should be “RLC entity”. However, there should be the same SN length for all RLC entities for a DRB, so we’d prefer not to change the current text. |

## 3.3	n77

R2-2205968	WF for NS_55 in NR CA	Ericsson	discussion	Rel-16	NR_RF_FR1-Core, TEI16

The document proposes to send LS to RAN4 to ask RAN4 to decide on solution for NS_55 in NR CA.

Question 3: Do companies agree with sending LS to RAN4 and await further RAN4 input.

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes | - We are fine to send LS to RAN4. |
| - Nokia | - See comment | - Our preference would be to have an explicit exception for this (for now) - otherwise we get very strange behaviour when C-band cells start using NS-55 and UEs do not camp on the cells because of that. |
| - Huawei, HiSilicon |  | - Our preference is also to have an exception for this. - If there is no consensus in RAN2, we are fine with an LS. |
| - Apple | - yes |  |

## 3.4	SMTC configuration

R2-2205614	SMTC configuration for target cell 	Lenovo	CR	Rel-16	38.331	16.8.0	3103	-	F	NR_newRAT-Core, TEI16

R2-2205586	SMTC configuration for target cell	Lenovo (Beijing) Ltd	CR	Rel-15	36.331	15.17.0	4804	-	F	NR_newRAT-Core

R2-2205599	SMTC configuration for target cell	Lenovo (Beijing) Ltd	CR	Rel-16	36.331	16.8.0	4805	-	F	NR_newRAT-Core

The CRs suggest to change ‘SN change’ to ‘PSCell change’ in the field description of targetCellSMTC-SCG-r16.

Question 4: Do companies agree with the changes proposed in CRs listed above?

| - Company | - Yes/No | - Comments |
| --- | --- | --- |
| - Samsung | - Yes |  |
| - Nokia | - No | - There is a potential misunderstanding of Lenovo. It was clarified - earlier already that when there is no SN change, the smtc is based - on the NR PSCell. This scenario for NR-DC has similar understanding. - So, we are not sure the change is really needed. |
| - OPPO | - Yes |  |
| - Huawei, HiSilicon | - No | - Similar understanding as Nokia. Not sure PSCell change without SN change requires SMTC configuration. |
| - ZTE | - See comments | - This field is configured by MN, included in MN RRC message. The motivation of introducing the field is to address SN addition and SN change cases. - For PSCell change without SN change, typically, the SN will include the smtc field in reconfigurationWithSync within SN RRCReconfiguration (this smtc is provided based on the timing of source PSCell). - However, for MN initiated intra-SN PSCell change, it is feasible for MN to also include the targetCellSMTC-SCG-r16 in MN generated RRC message, and this smtc is based on the timing of PCell. From UE’s perspective, the UE cannot differentiate whether SN is changed or not. So this change does not impact UE’s implementation. If both MN and SN provide smtc, it is up to the UE to decide which one to use, as specified in TS 37.340: “In (NG)EN-DC and NR-DC, SMTC can be used for PSCell addition/PSCell change to assist the UE in finding the SSB in the target PSCell. In case the SMTC of the target PSCell is provided by both MN and SN it is up to UE implementation which one to use.” - From network perspective, this CR provides another way to indicate smtc field, and it is only applicable to MN-initiated PSCell change procedure, but considering the SN will set the smtc field in reconfigurationWithSync. So this change cannot bring much benefit in practice. - So we are fine with current spec (without modification). |
| - Apple | - Not needed | - We share similar views as ZTE |

# 4	Conclusion

Tbd

# 5	References