3GPP TSG-RAN WG2 Meeting #118-e                                   R2-xxxxxxx

E-Meeting, May 9th – May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4809 | rev | 2 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CHO configuration with SCG release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, in RAN3 discussions (RAN3 #107-bis-e, RAN3 #108-e meetings), it was decided that the scenario where the SCG is released upon CHO execution, is supported, and changes to the Stage-2 (37.340, 36.300) and Stage-3 (36.423, 38.423) specifications were agreed. However, in the Rel-16 RRC specification 36.331, clarification seems to be needed to indicate that this scenario is supported, i.e., for a UE in MR-DC operation, upon CHO execution, the SCG is released by network signaling. The description of the field condReconfigurationToApply only indicates that the RRCConnectionReconfiguration message contained in it cannot contain the target SCG configuration for CHO. condReconfigurationToApply The RRCConnectionReconfiguration message to be applied when the condition(s) are fulfilled. The RRCConnectionReconfiguration in condReconfigurationToApply cannot contain a target node SCG configuration for CHO.. |
| Summary of change: |  | Provides text clarification in the description of the field cond ReconfigurationToApply to indicate that network signaling supports release of the SCG configuration upon CHO execution for a UE operating in MR-DC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is potential confusion regarding whether, for a UE operating in MR-DC, the SCG configuration is released upon CHO execution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.4, - CondReconfigurationToAddModList |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 2: Indicates that network signaling supports release of the SCG configuration upon CHO execution for a UE operating in MR-DC. Rev 1 and “-“: Provided specification changes indicating that UE autonomously releases SCG configuration upon CHO execution, if operating in MR-DC. This was not agreeable to majority of companies in the email discussion. |  |  |  |  |  |  |  |  |

## ----------Start of the change--------------

### 6.3.4	Mobility control information elements

## ----------Skipping unchanged parts--------------

#### –	CondReconfigurationToAddModList

The IE CondReconfigurationToAddModList concerns a list of conditional reconfigurations (i.e. conditional handover) to add or modify, for each entry the measId (associated to the triggering condition configuration) and the associated RRCConnectionReconfiguration.

CondReconfigurationToAddModList information element

-- ASN1START

CondReconfigurationToAddModList-r16 ::= SEQUENCE (SIZE (1.. maxCondConfig-r16)) OF CondReconfigurationAddMod-r16

CondReconfigurationAddMod-r16 ::= SEQUENCE {

condReconfigurationId-r16			CondReconfigurationId-r16,

triggerCondition-r16				SEQUENCE (SIZE (1..2)) OF MeasId

OPTIONAL,  -- Cond CondReconfigurationAdd

condReconfigurationToApply-r16	OCTET STRING (CONTAINING RRCConnectionReconfiguration)

OPTIONAL,-- Cond CondReconfigurationAdd

...

}

-- ASN1STOP

CondReconfigurationToAddMod field descriptions

condReconfigurationToApply The RRCConnectionReconfiguration message to be applied when the condition(s) are fulfilled. The RRCConnectionReconfiguration in condReconfigurationToApply cannot contain a target node SCG configuration for CHO. For CHO, if the UE is configured with MR-DC, the RRCConnectionReconfiguration message contained in condReconfigurationToApply includes nr-Config set to release.

triggerCondition The condition that needs to be fulfilled in order to trigger the execution of a conditional reconfiguration. When configuring two triggering events (MeasIds) for a candidate cell, the network ensures that both refer to the same measObject.

| Conditional presence | Explanation |
| --- | --- |
| CondReconfigurationAdd | The field is mandatory present if a condReconfigurationId is being added. Otherwise it is optional, need ON. |

## ----------End of the change--------------