3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-22XXXXX

Elbonia, 09 – 20 May 2022

Agenda item:	5.1.4.1.1	Connection control

Source:	Huawei (Rapporteur)

Title:	Report of [AT118-e][017][NR1516] Connection Control II (Huawei)

WID/SID:	NR Rel-15 and Rel-16

Document for:	Discussion and Decision

1	Introduction

This document is the report of the following email discussion:

[AT118-e][017][NR1516] Connection Control II (Huawei)

Scope: Treat R2-2204920, R2-2204921, R2-2206145, R2-2206146, R2-2204917, R2-2204918, R2-2204919, R2-2205251, R2-2205252, R2-2205617, R2-2205624

Ph1 Determine agreeable parts, Ph2 for agreeable parts agree CRs (offline agreement, CB online only if necessary).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

A first round with Deadline for comments W1 Thursday May 12th 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Wednesday May 18th 1200 UTC to settle details / agree CRs etc.

2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |

3	Discussion

3.1	CHO related discussions

CHO related papers

[1] R2-2204920	Correction on the RRC reestablishment in CHO	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3018	-	F	NR_Mob_enh-Core

[2] R2-2204921	Correction on the RRC reestablishment in CHO	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3019	-	A	NR_Mob_enh-Core

R2-2205850	CHO configuration with SCG release	Qualcomm Incorporated	CR	Rel-16	38.331	16.8.0	3120	-	F	NR_Mob_enh-Core

=> Revised in R2-2206145

[3] R2-2206145	CHO configuration with SCG release	Qualcomm Incorporated	CR	Rel-16	38.331	16.8.0	3120	1	F	NR_Mob_enh-Core

R2-2205858	CHO configuration with SCG release	Qualcomm Incorporated	CR	Rel-16	36.331	16.8.0	4809	-	F	LTE_feMob-Core

=> Revised in R2-2206146

[4] R2-2206146	CHO configuration with SCG release	Qualcomm Incorporated	CR	Rel-16	36.331	16.8.0	4809	1	F	LTE_feMob-Core

[1] and [2] are about corrections to CHO, and the reasons are as below:

- For CHO recovery, the UE can try CHO candidate cell and do the CHO handover if possible. The feature CHO recovery is only valid for CHO, but not CPC. However, the current spec only checks conditionalReconfiguration for CHO recovery, which covers both CHO and CPC cases.

In the CRs [1][2], for CHO recovery, it is clarified the UE only checks conditionalReconfiguration for CHO.

Question 1: Do companies agree with [1] and [2]?

| Company | Yes/No | Comments |
| --- | --- | --- |

[3] and [4] are about CHO configuration with SCG release. In R16, RAN3 agreed on the scenario where the SCG is released upon CHO execution, however the scenario does not seem to be supported. The CRs [3][4] are to introduce support for the scenario where a UE operating in MR-DC releases the SCG configuration upon CHO execution.

Question 2: Do companies agree with [3] and [4]?

| Company | Yes/No | Comments |
| --- | --- | --- |

3.2	DAPS related discussions

DAPS related papers

[5] R2-2204917	Discussion on RLC re-establishment issue upon DAPS fallback	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

[6] R2-2204918	Correction on UE behaviours for DAPS fallback_Alt1	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3016	-	F	NR_Mob_enh-Core

[7] R2-2204919	Correction on UE behaviours for DAPS fallback_Alt2	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3017	-	F	NR_Mob_enh-Core

[5] is about an issue during DAPS fallback procedure, and the contribution includes background, issues and some possible solutions. [6] and [7] are CRs for some solutions.

Question 3: Do companies agree with the following observation in [5]?

Observation: It is hard for the source gNB to handle the SRBs (including RLC state) from when the DAPS HO command is sent to when failure information message is sent.

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 4: for Q3, if the issue is confirmed, which of solutions do you prefer? E.g. Alt 1, Alt 2, Alt 3 (in [5]), and others if any.

| Company | Preferred solution | Comments |
| --- | --- | --- |

3.3	IAB related discussions

IAB related papers

[8] R2-2205251	Corrections on BAP entity release in MR DC release procedure in TS 38.331	Huawei, HiSilicon	CR	Rel-16	38.331	16.8.0	3060	-	F	NR_IAB-Core

[9] R2-2205252	Corrections on BAP entity release in MR DC release procedure in TS 38.331	Huawei, HiSilicon	CR	Rel-17	38.331	17.0.0	3061	-	A	NR_IAB_enh-Core

[10] R2-2205617	Correction to RRC reestablishment for IAB	Google Inc.	CR	Rel-16	38.331	16.8.0	3104	-	F	NR_IAB-Core

[11] R2-2205624	Correction to RRC reestablishment for IAB	Google Inc.	CR	Rel-17	38.331	17.0.0	3105	-	A	NR_IAB-Core

[8] and [9] are about an issue for IAB, and it is observed that the spec 38.331 does not clarify whether the IAB-MT releases the BAP entity even if the last and only configured bap-Config is released. The CRs [8][9] add the operation and the condition to release the BAP entity in IAB-MT’s MR DC release procedures

Question 5: Do companies agree with [8] and [9]?

| Company | Yes/No | Comments |
| --- | --- | --- |

[10] and [11] are about an issue for IAB, and the reasons are as below:

- If the IAB-MT is configured with the conditionalReconfiguration, the IAB-MT does not suspend BH RLC channels upon initiating the RRC reestablishment procedure. Upon initiating the RRC reestablishment procedure, the IAB-MT should suspend the BH RLC channels irrespective of whether the IAB-MT is configured with the conditionalReconfiguration.

In the CRs [10][11], it is clarified that if the IAB-MT is configured with the conditionalReconfiguration, the IAB-MT suspends BH RLC channels upon initiating the RRC reestablishment procedure.

Question 6: Do companies agree with [10] and [11]?

| Company | Yes/No | Comments |
| --- | --- | --- |

4	Conclusion

TBD.