3GPP TSG-RAN WG2 Meeting #118-e														R2-220wxyz

Electronic Meeting, May 9–20, 2022

Agenda item:	6.1.4

Source: 	MediaTek

Title: 		[AT118-e][033][MBS] UE capabilities (MediaTek)

Document for:		Discussion

# 1.	Introduction

This paper is to trigger the following email discussion of UE capabilities in MBS:

- [AT118-e][033][MBS] UE capabilites (MediaTek)

Scope: Treat R2-2204625, R2-2204907, R2-2205541, R2-2205746, R2-2205750, R2-2205855, R2-2205939, R2-2206114. Collect one round of comments, pave the way for on-line agreement (identify agreeable points, discussion points),

Intended outcome: Report

Deadline: For online CB W1 Thursday

## 1.1	Contacts

Contact person for each participating company:

| Company | Name | Email Address |
| --- | --- | --- |
| Ericsson | Martin van der Zee | martin.van.der.zee@ericsson.com |
| Ericsson | Henrik Enbuske | henrik.enbuske@ericsson.com |
| Huawei, HiSilicon | Dawid Koziol | dawid.koziol@huawei.com |

# 2.	Discussion

## 2.1	Mandatory UE capabilities for broadcast reception

In the last RAN2 meeting, the ROHC capability have been discussed for MBS broadcast and the following agreements are made：

P12: RoHC is mandatory for UEs supporting MBS broadcast: • At least profiles 0x0000, 0x0001, 0x0002 are supported. FFS other profiles. • FFS how many RoHC context sessions the UE has to mandatorily support. The number between 2 and 16 should be chosen. - RoHC profile 0x0006 is not used / configurable for broadcast MRB.

However, It is still FFS for the mandatory ROHC profiles and how many RoHC context sessions are mandatory for UE supporting broadcast.

### 2.1.1	ROHC context session

According to the contributions, the number of ROHC context sessions is proposed with the range of 2 to 16. Some companies prefer a default value of maxCID(i.e. 15) to be the mandatory capability of ROHC context sessions, while other companies indicated a smaller number (e.g. 2 or 8).

Meanwhile, the company tdoc in [7] indicates that the broadcast can reuse the current CONNECTED mode capability signaling for ROHC profile support, and the ROHC context session is considered as a limit across the total number of supported RBs.

#### Question 1: Do companies agree to introduce a capability for ROHC context session for MBS broadcast?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | No | As proponent of [7] we think that re-using the connected mode capability is sufficient. |
| Huawei, HiSilicon | Yes | We need to define a minimum capability for MBS broadcast UEs as agreed already: - P12: RoHC is mandatory for UEs supporting MBS broadcast: • At least profiles 0x0000, 0x0001, 0x0002 are supported. FFS other profiles. • FFS how many RoHC context sessions the UE has to mandatorily support. The number between 2 and 16 should be chosen. - RoHC profile 0x0006 is not used / configurable for broadcast MRB. The capability signalling does not help for MBS broadcast where the UEs are receiving the service in RRC ILDE and RRC INACTIVE. |

If the answer to Q1 is yes, please provide the further views on the number of ROHC context sessions .

#### Question 2: Please provide your views on the number of ROHC context sessions that should be supported for MBS broadcast.

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson |  | If a default value for the number of RoHC context sessions for MBS broadcast needs to be defined, we prefer a MaxCID of 15. |
| Huawei, HiSilicon | Up to 16 | Up to 16 is acceptable to us, but we are OK with lower value as well. |

### 2.1.2	ROHC profiles

According to the contributions submited, some companies suggest to keep the minimum set of ROHC profiles as agreed in the previous meeting(i.e. 0x0000, 0x0001 and 0x0002, which are mandatory at least for voice over IMS) for broadcast.

As proposed in R2-2205541, the profile 0x0004 should be supported for broadcast, since it is mainly a simplification of UDP/IP profile (0x0002), and it may not require much complexity while providing compression gain for services where only IP is used.

#### Question 3: Do you think the ROHC profile 0x0004 can be kept in the ROHC profile list for broadcast MRB? (please clarify if you think any additional profile is needed)

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | Yes | Agree with the motivation provided by Intel (R2-2205541). |
| Huawei, HiSilicon | No | We think there is no need to introduce IP only profile since it is not a common case in real deployments, e.g. due to limiatations for NAT usage (Network Address Translation). It is not widely deployed in network and UEs. We think the profiles agreed already are sufficient. |

### 2.1.3 Minimum number of broadcast MRBs

In the previous meeting, RAN2 has agreed the default number of multicast MRBs shares the common limitation with DRBs that MRBs+DRBs=16 without capability signaling. However, it is not concluded on whether broadcast can share the same limitation with unicast. Considering the use cases and limited UE performance in idle/inactive state, the capability for minimum number of MRBs supported for broadcast may be different.

Meanwhile, some companies in their contribution propose that there is no need to define new requirement for broadcast MRB, since maxNumberROHC-ContextSessions can be considered as a limit across the total number of supported RBs, and the gNB does not configure broadcast MRB for one particular UE.

#### Question 4: Do companies agree to introduce a UE capability for minimum number of MRBs supporting by broadcast UE? If yes, please provide the minimum number you preferred.

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | No | We do not see the need for new capability signalling, but if the UE supports MBS broadcast service, the UE should at least support one session e.g. at minimum 4 MBRs. But we are not sure if this is needed in addition to any agreement on the minimum number of RoHC context sessions that the UE is required to support for MBS broadcast. |
| Huawei, HiSilicon | Yes | It is beneficial to introduce a separate requirements for broadcast MRBs to avoid impact on unicast services when both unicast and MBS broadcast is used in RRC CONNETCED state. We think 4 broadcast MRBs is a reasonable number and no capability signalling is needed for this. We are not sure what is the linkage between this question and the number of ROHC sessions. |

## 2.2	Broadcast reception on non-serving cell

- According to the contributions submited, some companies suggest to introduce UE capability for Broadcast reception via non-serving cell. However, at meeting RAN2#117e, there was a discussion on whether it is optional (with UE capabilities) to support the broadcast reception on non-serving cell, and majorities agree that the reception in non-serving cell can be fully up to UE implementation without spec change. Rapporteur would like to check again if this is the majority views.

#### Question 5: Do companies agree to that the reception in non-serving cell can be fully up to UE implementation without spec change?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | Yes | Capability signalling can indicate if the UE is capable to receive MBS broadcast via non-serving cell, but does not indicate if the UE currently wants to receive MBS via non-serving cell, nor which service the UE is interested in. Furthermore the expected NW actions are not clear to us, i.e. in our understanding MII signalling + SCell capability can be used to configure SCell. What use case is missing? |
| Huawei, HiSilicon | No | Question is rather unclear. Obviously, the reception on non-serving cell is up to UE implementation. The main point for introducing the capability signalling is to let the network know that the UE can receive a service on a non-serving cell so that the UE does not have to be configured with a PCell or an SCell on this frequency. Without the capability signalling, even if the UE supports reception on non-serving cell, the network will have to configure a serving cell which makes the feature rather useless. We are not sure what the issue with introducing the capability signalling is. And of course the network needs to consider MII together with UE capabilities. @Ericsson: The case you are missing is, e.g. MII + non-serving cell reception capability means that the network does not have to configure SCell. |

## 2.3	Broadcast reception without capability signaling

According to R2-2205750, the proponet suggests to introduce MBS broadcast reception in idle, inactive and connected mode (FG33-1) to chapter 5 of 38.306 as an optional feature without capability signalling (since there is no UE capability signalling for FG33-1). Companies are invited to provided their views on whether such UE capability should be added to chapter 5 in 38.306.

#### Question 5: Do companies agree to introduce the UE capability for MBS broadcast reception in idle, inactive and connected mode (FG33-1) as an optional feature without capability signalling (to chapter 5 in 38.306)?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | Not sure | It would be good to describe the motivation for broadcast capability signalling. In our understanding the UE capability cannot be used in the transition from Idle to connected, but in case of resume the UE capability could be used in the initial configuration in the setup. But in case the UE is capable to support MBS broadcast, it does not mean that the UE is interested to receive MBS broadcast, nor it indicates which MBS broadcast service the UE is interested in. Also the signalling of both broadcast and SCell support should be clarified. Furthermore SCell continuity requires a reconfiguration. |
| Huawei, HiSilicon | No | We think the capability signalling is useful in this case so that the network can configure the UE with the ‘MBS broadcast compatible’ configuration as early as possible, i.e. even before MII is sent by the UE. |

## 2.4	Impact of MBS broadcast on paging and SIBs

In the contribution R2-2205746, the propent mentioned that UE may need to prioritize paging when the UE does not support the reception of Paging and group common PDSCH in the same slot. This enables the UE to receive paging and SI without any additional delay when paging and SI conflicts with broadcast.

However, it is rapporteur’s understanding that the current agreement from RAN1 has precluded this case:

RAN1#107bis-e Agreement For RRC_IDLE/INACTIVE UEs, a UE is not required to support reception of FDMed MCCH/MTCH PDSCH and SIB1 or Paging PDSCH in PCell.

In addition, in rapporteur understanding, the network can ensure that the paging/SIB information and group common PDSCH are not in the same slot.

#### Question 6: Do you think if RAN2 needs to discuss the potential issue for the reception of paging and group common PDSCH in the same slot?

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |
| Ericsson | Yes | The rapporteur is correct that RAN1 excluded the case, but we think that RAN1 did not discuss the scheduling impact. Furthermore the RAN1 optional UE capability for FDM/TDM-ed is also no solution for UEs in Idle/Inactive for Paging/SI and MBS broadcast service. In our understanding there is going to be more and more system information, and increased paging for MBS group paging and PEI transmissions. The NW would have to schedule MBS broadcast around SI transmissions, and only in case the scheduling is dynamic the Paging PO can be re-used for broadcast PTM transmissions when the UE is not paged. |
| Huawei, HiSilicon | Yes | The network may not always be able to prevent MBS broadcast from overlapping with Paging for each UE. It would be good to clarify that UE should prioritize Paging in this case. |

## 2.5	Simultaneous PDSCH processing capability

In the contribution R2-2206114, company mentioned several case for simultaneous PDSCH processing capability and suggest RAN2 to clarify the coverage of feature 33-3-2 and 33-3-3. Rapporteur thinks this is more suitable to be discussed in RAN1 and RAN2 can wait for RAN1’s conclusion. Therefore, no question is casted for this issue.

## 2.6 Other issues

#### Question 7: Companies are invited to comment if there are any other issues for MBS UE capabilities that needs to be discussed during this email discussion.

| Company | Answer (Yes/No) | Comments |
| --- | --- | --- |

# 3.	Final Summary and Proposal

Based on the email discussion, the following proposals are made for MBS UE capbility, with the easy proposals highlighted in green for online session:

TBD

# 4.	Reference

[1] R2-2204625	R17 MBS UE capabilities   	Qualcomm India Pvt Ltd	discussion	Rel-17	NR_MBS-Core

[2] R2-2204907	Discussion on mandatory ROHC support for MBS broadcast	MediaTek inc.	discussion

[3]R2-2205541 Remaining MBS UE capability open issues	Intel Corporation	discussion	Rel-17

[4]R2-2202786	Draft 306 CR for MBS UE capabilities	MediaTek Inc.	draftCR	Rel-17	38.306	16.7.0	B	NR_MBS-Core

[5] R2-2205746	Impact of MBS broadcast on paging and SIBs	Ericsson	discussion	Rel-17	NR_MBS-Core

[6] R2-2205750	UE capabilities for MBS	Ericsson	discussion	Rel-17	NR_MBS-Core

[7] R2-2205855	UE support for ROHC profiles and context sessions	Ericsson	discussion	Rel-17	NR_MBS-Core

[8] R2-2205939	Discussion on UE capabilities for MBS	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

[9] R2-2206114 UE capability discussion for MBS 	Xiaomi Communications	discussion	Rel-17	NR_MBS-Core

[10] R2-2205712	Discussion on MRB Configuration	Samsung 	discussion	Rel-17	NR_MBS-Core

[11] R2-2204626	R17 MBS UP remaining issues   	Qualcomm India Pvt Ltd	discussion	Rel-17	NR_MBS-Core

[12] R2-2203343 Report of: [Pre117-e][001][MBS] CP open Issues Input