3GPP TSG-RAN WG2 #118-e	Tdoc R2-220xxxx

Electronic meeting, 2022-05-09 – 2022-05-20

Agenda Item:	7.2.3

Source:	Ericsson

Title:	Report of [AT118-e][051][IoT NTN] Idle Inactive mode

Document for:	Discussion, Decision

# 1	Introduction

This is the report for the email discussion:

[AT118-e][051][IoT NTN] Idle Inactive Mode (Ericsson)

Scope: Treat R2-2204711, R2-2205250, R2-2205331, R2-2205861, R2-2204651

Ph1 Determine agreeable parts, Ph2, agree/endorse TP(s) if applicable.

Intended outcome: Report, endorsed TPs/Draft CRs

Deadline: Schedule 1 (CB online W2 if needed)

In this e-mail discussion we will discuss idle mode issues.

# 2	Discussion

## 2.1	t-Service corrections

Contribution R2-2204711 [1] and R2-2205250 [2] propose to introduce corrections to the text related to how t-service functions.

In [1] it is proposed to change the wording similar to NR NTN and it is also proposed that inter-frequency measurements are included in text that is related to LTE-M.

------------------ TP1 ------------------

#### 5.2.4.2	Measurement rules for cell re-selection

<Removed irrelevant parts>

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE should start to perform intra-frequency, inter-frequency or inter-RAT measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

#### 5.2.4.2a	Measurement rules for cell re-selection for NB-IoT

<Removed irrelevant parts>

If t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE should start to perform intra-frequency or inter-frequency measurements before the time t-Service regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the exact time to start measurement before t-Service is up to UE implementation.

------------------ TP1 ------------------

In [2], it is proposed to improve the text by using similar language and also removing the “should”, but simplified:

------------------ TP2 ------------------

#### 5.2.4.2	Measurement rules for cell re-selection

<Removed irrelevant parts>

Regardless of any procedure limiting measurements, if t-Service is present in SystemInformationBlockType3 of the serving cell, UE may start to perform intra-frequency or inter-frequency measurements, before the time t-Service. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

#### 5.2.4.2a	Measurement rules for cell re-selection for NB-IoT

<Removed irrelevant parts>

Regardless of any procedure limiting measurements, if t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE may start to perform intra-frequency or inter-frequency measurements, before the time t-Service.

------------------ TP2 ------------------

It is clear that “should” is not according to 3GPP terminology and rapporteur intends to change this.

The first question is whether t-Service actions should include inter-RAT measurements as this has yet to be agreed. Inter-RAT is likely to imply a TN RAT and while a UE should likely prioritize a terrestrial network over non-terrestrial network, it could be potentially be an energy consumption waste to perform inter-RAT due to t-Service, thus it is not entirely clear whether inter-RAT measurements should be included in t-Service actions.

Q1: Should t-Service actions include performing inter-RAT measurements as R2-2204711 [1]?

| Company | Y/N | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Y |  |
| OPPO | Y | The measurements before the stop time of the serving cell should also include inter-RAT frequency besides inter and intra-frequency. Otherwise, when the serving cell stops covering the current area, UE may lose the opportunity of camping in some cells belonging to other TN/NTN RAT. Note that this would be in line with the wording captured in TS 38.304 for NR-NTN. |
| InterDigital | Y |  |

<Rapporteur comment>

T-service includes/does not include inter-RAT measurements.

Rapporteur thinks that the text in [2] is more clean compared to the current text in 36.304-h00 [7], thus the question is whether to adopt the text in [1] or [2].

Q2: Adopt the text that:

- uses NR NTN text (R2-2204741) [1]

- uses simplified text (R2-2205250) [2]

- Other

| Company | 1/2/3 | Comment or further text |
| --- | --- | --- |
| Huawei, HiSilcion | 1) with comments | Fine to align with NR wording The text for NB-IoT is not correct as Squal is not used in the measurement rules If t-Service is present in SystemInformationBlockType3-NB of the serving cell, UE should start to perform intra-frequency or inter-frequency measurements before the time t-Service regardless of whether the serving cell fulfils Srxlev > SIntraSearchP or Srxlev > SnonIntraSearchP , and the exact time to start measurement before t-Service is up to UE implementation. About changing ‘should’ to ‘may’, we are not sure. we think it should be ‘shall’ for a UE supporting t-Service |
| OPPO | 1 | It would be simpler to capture this by the wording in TS 38.304 for NR-NTN. And agree with Huawei’s comments. |
| InterDigital | 1 with change suggested by Huawei | Agree with Huawei that Squal is not applicable for NB-IoT. |

<Rapporteur comment>

TP in R2-220xxxx is agreed .

## 2.2	NTN-only UE

In R2-2205331 [3] and R2-2205861 [4] the editor’s note was addressed (this was also addressed in contribution R2-2205594 [5] (Interdigital contribution)):

Editor’s Note: FFS how an NTN-only UE is barred from accessing TN cell.

It is argued in [3] and [4] that NTN-only UEs do not need to be specified and the editor’s note can be removed and in [5] it is mentioned that it should be discussed. Thus rapporteur think it can be discussed here as the main impact identified has to do with barring-mechanism.

Q3: Should there be any specification support for NTN-only UE in IoT NTN Rel-17?

| Company | Y/N | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No | Backward compatibility is a basic principle of 3GPP. This means a NTN UE shall support all mandatory features in the specification and this includes support of TN access. |
| OPPO | No | NTN UE must be capable to access TN since it has the mandatory capabilities for TN. This has just been agreed in NR NTN. |
| InterDigital | No |  |

Q4: If yes to Q3, should barring from accessing TN for NTN-only UE be introduced?

| Company | Y/N | Comment |
| --- | --- | --- |

<Rapporteur comment>

Barring for NTN-only is/is not introduced.

## 2.3	Barring mechanism

In R2-2205861 [4] the barring mechanism is discussed. It is explained that the current text in 36.304 reads:

-	cellBarred (IE type: "barred" or "not barred") 
This field indicates if the cell is barred for connectivity to EPC.
This field is ignored by the UEs supporting crs-IntfMitig while crs-IntfMitigEnabled is included in SIB1. 
This field is ignored by the BL UEs or UEs in CE supporting ce-CRS-IntfMitig while crs-IntfMigitNumPRBs is included in SIB1-BR.
This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1-BR or SIB1-NB.
In case of multiple EPC PLMNs indicated in SIB1/SIB1-BR, this field is common for all EPC PLMNs

And that in 36.331 [8], the cellBarred-NTN is mandatory present in cellAccessRelatedInfo-v1700 (cellAccessRelatedInfoNTN-r17 in latest agreed RILs), but cellAccessRelatedInfo-v1700 is optional. Thus there a slight discrepancy, where the presence of cellBarred-NTN may or may not be mandatory present for NTN, while the idle mode text implies that it does not always need to be present for NTN.

Q5: Which change to resolve barring discrepancy in RRC/idle mode?

- None

- Change the 36.304 text to “This field is ignored by UEs supporting NTN while cellBarred-NTN is included in SIB1-BR or SIB1-NB.” and make cellAccessRelatedInfo mandatory present whenever NTN is deployed. This can for instance be achieved in a loss way by renaming cellAccessRelatedInfo to something that makes it clear it is for NTN only.

- Keep 36.304 text and make cellBarred-NTN optional in RRC. Then the solution partly relies on the network configuring cellBarred-NTN correctly, by including cellBarred-NTN in scenarios when TN and NTN is deployed in the same bands and setting cellBarred to barred.

- Other

| Company | 1/2/3/4 | Comment or further text |
| --- | --- | --- |
| Huawei, HiSilicon | 1 | We do not see any issue with the current text. Although we assume that cellAccessRelatedInfo-NTN (according to rapporteur CR) will normally be present in a NTN cell, we see no problem with a NW barring all UEs (TN and NTN) by using the legacy bit. Option 2 is not enough because supporting NTN is not a reason to ignore the legacy bit in a TN cell |
| OPPO | 1 | We also do not see any issue with the current text. For NTN cell, since legacy UE and R17 non-NTN capable UE cannot access the NTN cell due to lack of pre-compensation capability, etc. Therefore, we think NTN cell should always set cellBarred as “barred” to prevent the access of legacy UE and R17 non-NTN capable UE. In this case, if cellBarred-NTN is not present in this NTN cell, UE supporting NTN could consider the cell to be barred according to cellBarred. |
| InterDigital | 1 | We see no problem with the current text. NW could bar all UEs using the legacy bit as others have stated. In addition, a TN cell won’t include this, so an NTN UE should ignore cellBarred only if cellBarred-NTN is present. |

<Rapporteur comment>

Change … to ….

## 2.4 Other open idle mode issues

Companies can bring up further idle mode issues or corrections below:

| Company | Issue |
| --- | --- |

# 3 Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	T-service includes/does not include inter-RAT measurements.

Proposal 2	TP in R2-220xxxx is agreed .

Proposal 3	Barring for TN-only is/is not introduced.

Proposal 4	Change … to ….

# 4 References

R2-2204711, Correction on Measurement rules for cell re-selection in IoT-NTN, OPPO, RAN2#118-e, May 2022

R2-2205250, 36.304 R17 editorial corrections, Nokia, Nokia Shanghai Bell, RAN2#118-e, May 2022

R2-2205331, Addressing 36.304 Editor’s note, Huawei, Hisilicon, RAN2#118-e, May 2022.

R2-2205861, IoT NTN idle mode issues, Ericsson, RAN2#118-e, May 2022.

R2-2204651, Clarification on TN NTN barring, Qualcomm Incorporated, RAN2#118-e, May 2022.

R2-2205594, IoT-NTN-only UE, Interdigital Inc., RAN2#118-e, May 2022.

3GPP TS 36.304, User Equipment (UE) procedures in idle mode, V17.0.0, Release 17, March 2022.

3GPP TS 36.331, Radio Resource Control (RRC), V17.0.0, Release 17, March 2022.