3GPP TSG-RAN WG2 Meeting #118-e     			          	                         R2-22XXXX

Electronic Meeting, 9 May – 20 May 2022

Agenda item:	6.14.4

Source:	CMCC

Title:	Report for [AT118-e][080][QoE] UE capabilities (CMCC)

WID/SID:	NR_QoE

Document for:	Discussion and Decision

Introduction

This document captures the following UE capabilities discussion:

[AT118-e][080][QoE] UE capabilities (CMCC)

Scope: Treat R2-2205944, R2-2204849. Determine agreeable parts. Update CR to reflect agreeable part and agree CR.

Intended outcome: Report, Endorsed CR(s) for merge

Deadline: CB W2 Wed (if needed), CR can be finally agreed in a post-meeting disc.

Contact List

| Company | Name | Email |
| --- | --- | --- |

Discussion

Minimum memory size for QoE paused measurements report

As suggested in [1], the agreement that minimal memory size of QoE paused measurements report is 64KB should be captured in TS 38.306, as a conditionally mandatory feature without UE radio access capabilities parameters. Therefore, the following change is proposed:

| Features | Condition |
| --- | --- |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurements reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |

Q1: Do you agree with the change above？

| Company | Yes or no | Comments |
| --- | --- | --- |

Summary:

Proposal:

Correction on RAN visible QoE

In [2], it is suggested that since RAN3 agreed the RAN visible QoE metrics is a subset of legacy QoE metrics, the dependency of RAN visible QoE and the corresponding legacy QoE should be clarified. And the following changes are proposed:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| qoe-Streaming-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for streaming services, see TS 26.247 [29]. | UE | No | No | No |
| qoe-MTSI-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for MTSI services, see TS 26.114 [30]. | UE | No | No | No |
| qoe-VR-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for VR services, see TS 26.118 [31]. | UE | No | No | No |
| ran-VisibleQoE-Streaming-MeasReport-r17 Indicates whether the UE supports RAN visible QoE Measurement Collection for streaming services. A UE supporting this feature shall also support qoe-Streaming-MeasReport-r17. | UE | No | No | No |
| ran-VisibleQoE-VR-MeasReport-r17 Indicates whether the UE supports RAN visible QoE Measurement Collection for VR services. A UE supporting this feature shall also support qoe-MTSI-MeasReport-r17. | UE | No | No | No |
| ul-MeasurementReportAppLayer-Seg-r17 Indicates whether the UE supports RRC segmentation of the MeasurementReportAppLayer message in UL, as specified in TS 38.331 [9]. | UE | No | No | No |

Q2: Do you agree with the change above?

| Company | Yes or no | Comments |
| --- | --- | --- |

Summary:

Proposal:

Summary

References

- R2-2204849	Introduction of AS layer memory size for QoE paused measurement reports, Lenovo

- R2-2205944	Correction on QoE capabilities dependencies, Huawei, HiSilicon