3GPP TSG-RAN WG2 Meeting #118-e     			          	                         R2-22XXXX

Electronic Meeting, 9 May – 20 May 2022

Agenda item:	6.14.4

Source:	CMCC

Title:	Report for [AT118-e][080][QoE] UE capabilities (CMCC)

WID/SID:	NR_QoE

Document for:	Discussion and Decision

Introduction

This document captures the following UE capabilities discussion:

[AT118-e][080][QoE] UE capabilities (CMCC)

Scope: Treat R2-2205944, R2-2204849. Determine agreeable parts. Update CR to reflect agreeable part and agree CR.

Intended outcome: Report, Endorsed CR(s) for merge

Deadline: CB W2 Wed (if needed), CR can be finally agreed in a post-meeting disc.

Contact List

| Company | Name | Email |
| --- | --- | --- |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| Apple | Ping-Heng Wallace Kuo | pingheng_kuo@apple.com |
| CATT | Ni Chunlin | nichunlin@catt.cn |

Discussion

Minimum memory size for QoE paused measurements report

As suggested in [1], the agreement that minimal memory size of QoE paused measurements report is 64KB should be captured in TS 38.306, as a conditionally mandatory feature without UE radio access capabilities parameters. Therefore, the following change is proposed:

| Features | Condition |
| --- | --- |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurements reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |

Q1: Do you agree with the change above？

| Company | Yes or no | Comments |
| --- | --- | --- |
| Lenovo | Yes | Proponent |
| Apple | Yes | We are fine to have this addition in order to capture the agreement. |
| CATT | Yes | We support this change |

Summary:

Proposal:

Correction on RAN visible QoE

In [2], it is suggested that since RAN3 agreed the RAN visible QoE metrics is a subset of legacy QoE metrics, the dependency of RAN visible QoE and the corresponding legacy QoE should be clarified. And the following changes are proposed:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| qoe-Streaming-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for streaming services, see TS 26.247 [29]. | UE | No | No | No |
| qoe-MTSI-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for MTSI services, see TS 26.114 [30]. | UE | No | No | No |
| qoe-VR-MeasReport-r17 Indicates whether the UE supports NR QoE Measurement Collection for VR services, see TS 26.118 [31]. | UE | No | No | No |
| ran-VisibleQoE-Streaming-MeasReport-r17 Indicates whether the UE supports RAN visible QoE Measurement Collection for streaming services. A UE supporting this feature shall also support qoe-Streaming-MeasReport-r17. | UE | No | No | No |
| ran-VisibleQoE-VR-MeasReport-r17 Indicates whether the UE supports RAN visible QoE Measurement Collection for VR services. A UE supporting this feature shall also support qoe-MTSI-MeasReport-r17. | UE | No | No | No |
| ul-MeasurementReportAppLayer-Seg-r17 Indicates whether the UE supports RRC segmentation of the MeasurementReportAppLayer message in UL, as specified in TS 38.331 [9]. | UE | No | No | No |

Q2: Do you agree with the change above?

| Company | Yes or no | Comments |
| --- | --- | --- |
| Lenovo | Yes but | In the description of ran-VisibleQoE-VR-MeasReport-r17 the reference to qoe-MTSI-MeasReport-r17 is wrong. On the actual changes we prefer to say: ran-VisibleQoE-Streaming-MeasReport-r17 “A UE indicating support of ran-VisibleQoE-Streaming-MeasReport-r17 shall indicate support of qoe-Streaming-MeasReport-r17.” ran-VisibleQoE-VR-MeasReport-r17 “A UE indicating support of ran-VisibleQoE-VR-MeasReport-r17 shall indicate support of qoe-VR-MeasReport-r17.” |
| Apple | Yes (with correction) | We prefer the original wordings in R2-2205944. However, there is a typo that should be corrected: ran-VisibleQoE-VR-MeasReport-r17 Indicates whether the UE supports RAN visible QoE Measurement Collection for VR services. A UE supporting this feature shall also support qoe-MTSIVR-MeasReport-r17. |
| CATT | Yes(with correction) | Share with Apple on this addition in 38.306. But in 38.331, we don’t reflect this dependency when specify this capability. Do we need to change RVQOE capability from optional to condition? |

Summary:

Proposal:

Summary

References

- R2-2204849	Introduction of AS layer memory size for QoE paused measurement reports, Lenovo

- R2-2205944	Correction on QoE capabilities dependencies, Huawei, HiSilicon