Agreements via email – from offline 110:

- The working assumption that new bit, e.g. cellBarred-NTN, is introduced for NR-NTN in SIB1 is confirmed

- NTN UE consider the cell to be barred for NTN access if cellBarredNTN is set to “barred”.

- NTN UE consider the cell to be allowed for NTN access if cellBarredNTN is set to “not barred”.

- NTN UE follows the legacy cellBarred for TN access and consider the cell is not allowed for NTN access if cellBarredNTN is not present.

Agreements:

- Location based cell reselection method is not introduced in NR NTN Rel-17