3GPP TSG-RAN WG2 #118-e	Tdoc R2-220XXXX

Electronic meeting, 9th - 20th May 2022

Agenda Item:	6.12.2.2

Source:	Ericsson (Rapporteur)

Title:	Report from [AT118-e][102][RedCap] RRC CR (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This is the report from the offline discussion below:

[AT118-e][102][RedCap] RRC CR (Ericsson)

Initial scope: continue the discussion on the RedCap WI-specific RILs, also considering the submitted contributions

Initial intended outcome: Summary of the offline discussion with e.g.:

·         List of resolved RILs

·         List of RILs for online discussion

·         List of RILs for further offline discussion

Deadline (for companies' feedback): Wednesday 2022-05-11 2000 UTC

Deadline (for rapporteur's summary in R2-2206192): Wednesday 2022-05-11 2200 UTC

Companies should consider the following Tdocs and the discussions therein in mind when providing feedback to the offline discussion:

R2-2206021	Miscellaneous corrections for RedCap WI	Ericsson	CR	Rel-17	38.331

R2-2206022	RedCap WI ASN1 RIL list	Ericsson	discussion	Rel-17

R2-2204725	[O374] correction on RedCap UE’s cell barring	OPPO	draftCR	38.331

R2-2204736	[O372] Discussion on prohibit timer for UAI for RRM relaxation fulfilment indication	OPPO

R2-2204737	[O377] Correction to 38.331 on UAI for RRM relaxation fulfilment indication	OPPO			draftCR	38.331

R2-2204813	[V166] Including RedCap Capability in the UERadioPagingInformation Inter-Node Message			vivo, Guangdong Genius	discussion

R2-2204814	[V170] Discussion on Inter-RAT Mobility from LTE to NR for RedCap	vivo, Guangdong Genius			discussion

R2-2204929	RRC open issues on Rel17 RedCap WI	Intel Corporation

R2-2206059	[X115]38.331 Corrections on UE's behaviour of getting SIB1 for Redcap	Xiaomi Communications	draftCR	38.331

R2-2206060	[X119][X114] Discussion on PDCCH-ConfigCommon for Redcap	Xiaomi Communications

R2-2206061	[X119][X114] 38.331 Corrections on PDCCH-ConfigCommon for Redcap	Xiaomi Communications	draftCR	38.331

R2-2206062	[X120] 38.331 Corrections on Need code of RedCap-specific initial DL BWP for handover			Xiaomi Communications	draftCR	38.331

R2-2204541	[S953] SI Request for RedCap UEs	Samsung Electronics Co., Ltd

R2-2204936	I051 support of RedCap based on intraFreqReselectionRedCap	Intel Corporation

R2-2204979	Cell reselection priority for RedCap (RIL#: S952)	Samsung

R2-2205523	SIB validity with eDRX	MediaTek Inc.

R2-2205783	Miscellaneous RedCap corrections	Nokia, Nokia Shanghai Bell	CR	38.331

R2-2205785	HD-FDD RedCap support in system information	Nokia, Nokia Shanghai Bell

R2-2206080	[H507] Corrections on cell re-selection measurements during RRC setup/resume	Huawei, HiSilicon	CR	38.331

R2-2206081	[H511] Corrections on redcapAccessRejected	Huawei, HiSilicon	CR	38.331

R2-2206082	[H513 H516 H520 H524 H525 H526 H527] Corrections on RedCap initial BWP	Huawei, HiSilicon	CR	38.331

R2-2204819	UE Capability and System Information for eDRX	vivo, Guangdong Genius

In this document, we continue the discussion based on the agreements above and the list of Tdocs provided above with the intention to formulate a list of proposals that are agreeable and a list of proposals that require further discussion during the related online session.

# Contact Information

Please fill in the following table for contact information:

| Company | Contact person - email@address.com |
| --- | --- |
| Ericsson | Emre A. Yavuz – emre.yavuz@ericsson.com |
| Intel | Yi.guo@intel.com |
| Samsung | s90.jeong@samsung.com |
| Xiaomi | Liyanhua1@xiaomi.com |

# 2	Discussion on RILs and open issues

## 2.1	RILs marked with “PropAgree”

Q 2.1 The following is a list of RILs which are marked as “PropAgree” in the latest version of the Excel document that contains RILs, i.e., R2-2206022:

H506, V163, H509, V168, V169, H514, H704, Z033, H515, M608, H517, V161, Z034, H522

The rapporteur has implemented those RILs in the 38.331 CR provided in R2-2206021, which is to be updated once RILs marked with “PropModifyAgree”, “PropDiscMeeting” or “PropReject” are concluded. The rapporteur proposes the following:

Proposal	The following RILs are agreed: H506, V163, H509, V168, V169, H514, H704, Z033, H515, M608, H517, V161, Z034, H522 (as captured in R2.2206021).

Do you agree with the proposal above? Please elaborate your reply, especially if you do not, and provide a resolution/text proposal that addresses your concerns considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | V168/V169 are related to the discussion on whether FD-FDD is mandatory or not. But it is ok for us for the changes. We did not find H704, is it typo? H509: We don’t agree to H509. We don’t see a benefit to move from field description to condition. In general, conditions should be used if the conditions is related to something in the same message and not something that was configured previously (though this may not be strictly followed). |
| Samsung | Yes, except for V168, V169 | V168/V169 can be discussed after having conclusion from FD-FDD discussion. H704, which is missing in R2-2206021, is to clarify 'If configured, the RedCap UE operating in this BWP uses…' from the field description of nonCellDefiningSSB-r17, and we are fine with the change. For H517, we are fine with the change itself, but want to clarify the comment from Huawei in their RIL: we understand that the RedCap specific initial DL BWP can be used for paging and OSI if the BWP includes CD-SSB and contains the entire CORESET#0. |
| Xiaomi | Yes except for V168 | H509: - Change ”Need R” to “Cond RAN-Paging” - Add Cond “RAN-Paging This field is optionally present, Need R, if the UE is configured with DILE eDRX, see TS 24.401 [23]; otherwise the field is not present.” It is better to change to “Cond IDLEeDRX”. Because the condition is the idle mode eDRX is used. V168 is about the HD-FDD flag while the FFS is about the eDRX flag. |

Summary – Q 2.1

???

Based on the observations above, the rapporteur proposes the following:

???

## 2.2	RILs marked with “PropModifyAgree”

Q 2.2 The following is a list of RILs which are marked as “PropModifyAgree” in the latest version of the Excel document that contains RILs, i.e., R2-2206022:

H520, H705

The rapporteur has implemented those RILs in the 38.331 CR provided in R2-2206021 with a modification on the text proposed by the source company. The rapporteur proposes the following:

Proposal	The following RILs are agreed: H520, H705 (as captured in R2.2206021).

Do you agree with the proposal above? Please elaborate your reply, especially if you do not, and provide a resolution/text proposal that addresses your concerns considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Comments | H520, is related to the discussion in Atmeeting 105, Proposal 9: Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. Would be good to wait a bit. |
| Samsung | Yes | We support the changes in the RILs. |
| Xiaomi |  | H520, the issue is relates to whether Redcap UE need to read the SS for paging, SI from PDCCH-ConfigCommon configuration from legacy initial BWP in case RedCap-specific initial DL BWP NOT contains CORESET#0. We have a paper R2-2206060. Can be discussed with X119-2. Or wait for AT105. H705, do not see the problem. |

Summary – Q 2.2

???

Based on the observations above, the rapporteur proposes the following:

???

## 2.3	RILs marked with “PropReject”

Q 2.3 The following is a list of RILs which are marked as “PropReject” in the latest version of the Excel document that contains RILs, i.e., R2-2206022:

X115, X110, X111, X112, V165, H525, H526

The rapporteur has indicated that there is no need to implement those RILs as argued in R2-2206022. The rapporteur proposes the following:

Proposal	The following RILs are not pursued: X115, X110, X111, X112, V165, H525, H526.

Do you agree with the proposal above? Please elaborate your reply, especially if you do not, and provide a resolution/text proposal that addresses your concerns considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Samsung | Yes | V165 can be superseded by H705 as rapporteur suggested. H525/526 seems obvious, so we are fine with rapporteur's suggestion. |
| Xiaomi | - | X115: we want to add: “To add: 6> if the UE is unable to acquire the SIB1: 7> perform the actions as specified in clause 5.2.2.5; 6> else: 7> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2. ” If people think it is already clear in the spec, we can follow the majority view. X110,and X111: “apply a supported uplink channel bandwidth with a maximum transmission bandwidth which is wider than or equal to the bandwidth of the initial BWP for the uplink“ then the qestion is which initial BWP, the one for Redcap or for eMBB UE? We would rather to reconsider this. X112, OK to withdraw it. H525/526, to“ Add “This parameter shall always be present if the initial UL BWP for non-RedCap UEs exceeds the RedCap UE maximum bandwidth.” We think that is already caputured in 38.213. |

Summary – Q 2.3

???

Based on the observations above, the rapporteur proposes the following:

???

## 2.4	RILs marked with “PropDiscMeeting”

The discussion in this section is on a selection of RILs from the following list which are marked as “PropDiscMeeting”:

I051, O374, V162, X116, H507, V170, H508, N016, H510, FW001, S952, H511, C271, H512, Z035, Z036, N107, X119, V164, H513, H516, H518, X114, H523, S953, H524

Q 2.4.1 This question is regarding RILs I051 and N016, which are related.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | As proponent. It would be good to follow legacy way, i.e. the UE only needs to check cell barring information, it can also save 1 bit in system information. |
| Samsung | Yes | Agree with N016. |
| Xiaomi | - | Can be discussed. The proposed change seems to treat cellBarredRedCap1Rx or cellBarredRedCap2Rx as optional which is conflict with what we have agreed: “For the cell barring in SIB1, RAN2 agree to use two mandatory sub-IEs with {barred, notBarred} values included in one optional parent IE cellBarredRedCap-r17.“ |

Summary – Q 2.4.1

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.2 This question is regarding RIL O374.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | Ok to us. |
| Xiaomi | - | Can discuss whether UE to follow the IFRI in SIB1 or treat the IFRI as allowed Seems the current spec is Ok. |

Summary – Q 2.4.2

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.3 This question is regarding RIL V162.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Samsung | Yes | To change to 'perform' would be fine. |
| Xiaomi | - | Can discuss. We also noticed that in 331, for eMBB UE, we also used this way. |

Summary – Q 2.4.3

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.4 This question is regarding RIL X116.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Do not see the problem to keep the sentence. |
| Samsung | No | To update the text like in MIB (as proposed by V162) would resolve the issue? |
| Xiaomi | - | The same issue with V162. |

Summary – Q 2.4.4

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.5 This question is regarding RIL H507.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Maybe | No strong opinion on whether the Note should be added. It would be good to capture something in RAN4 spec. |
| Xiaomi | - | No strong view. Maybe we can add it. |

Summary – Q 2.4.5

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.6 This question is regarding RIL H510.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Maybe | No strong opinion on whether “dB2” should be added as minimum value for stationary evluation. |
| Xiaomi | Yes | Reasonable. Is the value range decided by RAN4 or RAN2? |

Summary – Q 2.4.6

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.7 This question is regarding RIL FW001.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided, e.g., R2-2204353.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Do not see the problem since TS38.304 is clear. |
| Xiaomi | No | Do not see the problem. |

Summary – Q 2.4.7

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.8 This question is regarding RILs S952, H511, and C271.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | S952 cell reselection priority Cell level resele3ction priority for RedCap (1 rx, HDD) It has been excluded in last meeting. |
| Samsung | Yes (Proponent) | Regardless of 1RX and HD-FDD issues, the legacy structure can be considered for the RedCap itself. 1RX and HD-FDD issues can be discussed together with SIB1 indication (considering FD-FDD capability). |
| Xiaomi | - | S952: we have excluded this. H511,C271: OK |

Summary – Q 2.4.8

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.9 This question is regarding RIL H512.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Comments | It is related to At meeting discussion 105, should wait. |
| Xiaomi | - | Wait for AT105. |

Summary – Q 2.4.9

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.10 This question is regarding RIL H512.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel |  | Duplicated with Q 2.4.9 |

Summary – Q 2.4.10

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.11 This question is regarding RIL Z035.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | Looks good to us. |
| Samsung | Yes | - |
| Xiaomi | Yes |  |

Summary – Q 2.4.11

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.12 This question is regarding RILs Z036, N107, and H523.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Comments | H523, Need R should be correct since the field is put under extension and we need the way to delete it. Z036, we agreed in last meeting delta signalling is not used. So do not understand why the need code should be changed to Need M? - For RedCap-specific BWP, both common and dedicated configurations are provided using full configuration, i.e., delta configuration is not supported. N107, seems correct. |
| Xiaomi |  | Z036/ N107: can be discussed with X119-1 in Q 2.4.13. |

Summary – Q 2.4.12

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.13 This question is regarding RIL X119-1.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Comments | May discuss based on Vivo R2-2204817 on the UE behavior if the RedCap-specific initial BWP is not configured. |
| Xiaomi | Yes | Can be disussed with Z036. The purpose is to change the Need code of RedCap-specific initial DL BWP for handover. We have provides a paper R2-2206062: initialDownlinkBWP-RedCap-r17 BWP-DownlinkCommon OPTIONAL -- Cond ServCellAdd1 ServCellAdd1: For Redcap, this field is mandatory present upon handover from E-UTRA to NR. It is optionally present, Need M otherwise. |

Summary – Q 2.4.13

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.14 This question is regarding RIL V164.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Do not see the need to add every RAN1 details in RRC specification. |
| Samsung | Maybe no | We are not sure whether the referred RAN1 agreement is about this update. |
| Xiaomi | No | Do not see the problem. |

Summary – Q 2.4.14

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.15 This question is regarding RILs H513 and H516.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Editorial change, do not see the problem. |
| Samsung | No | The current wording looks okay to us. |
| Xiaomi | No | Do not see the problem. |

Summary – Q 2.4.15

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.16 This question is regarding RIL H518.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Do not see the problem, since we already mentioned “the UE uses this BWP also for receiving DL messages during initial access (Msg2, Msg4, ...) and after initial access .” |
| Samsung | No | Agree with Intel. |
| Xiaomi | - | No strong view. Can check with RAN1. |

Summary – Q 2.4.16

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.17 This question is regarding RIL X119-2.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | We think from signalling pov, all signalling in PDCCH-CommonConfig should be in RedCap specific initial BWP. Then whether further UE needs to get it from the RedCap specific initial BWP or from the legacy initial BWP is for signalling optimisation. |
| Xiaomi | Yes | Can discuss with H520 in Q2. The issue is relates to whether Redcap UE need to read the SS for paging, SI from PDCCH-ConfigCommon configuration from legacy initial BWP in case RedCap-specific initial DL BWP NOT contains CORESET#0. We have a paper R2-2206060. Or wait for AT105. |

Summary – Q 2.4.17

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.18 This question is regarding RIL X114.

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | Do not see the strong need on the suggested sentence. |
| Xiaomi | Yes | Need to discuss. If the RedCap specific initial DL BWP does NOT contain the entire CORESET#0, then the additional common control resource set configured for Redcap is not contained in the bandwidth of CORESET#0. The current spec says“ The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0.“ Obviously, it is not considering the BWP may not containing CORESET#0. We suggest it to be clarified. |

Summary – Q 2.4.18

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.19 This question is regarding RIL S953 (Tdoc R2-2204541).

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes (Proponent) | SI request configuration (for Msg1 based SI request) for the RedCap-specific initial UL BWP should be provided separately, as RedCap UEs have to use only the RedCap-specific initial UL BWP to perform RACH if configured. Note that RedCap-specific initial UL BWP has its own RACH configuration (preambles/ROs). The preambles/ROs available for SI request on RedCap-specific initial UL BWP are not same as the preambles/ROs available for SI request on legacy initial UL BWP (i.e. non RedCap-specific initial UL BWP). So current SI request configuration cannot be applied for both RedCap-specific initial UL BWP and legacy initial UL BWP (i.e. non RedCap-specific initial UL BWP). |
| Xiaomi | - | Can be discussed. The Redcap UE acquire the same SIB with eMBB UE, why the SI request configuration cannot be reused? |

Summary – Q 2.4.19

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.4.20 This question is regarding RIL V166

Do you agree with the issue(s) indicated? Please elaborate your reply, regardless of whether you do or not and provide a resolution/text proposal that addresses your concerns, if you agree with the intention considering the feedback from companies, if provided.

| Company | Yes/No | Comments |
| --- | --- | --- |

Summary – Q 2.4.20

???

Based on the observations above, the rapporteur proposes the following:

???

## 2.5	RRC related issues discussed separately

In this section, we discuss the open RRC related issues brought up in the contributions below:

R2-2204929 RRC open issues on Rel17 RedCap WI	Intel Corporation

R2-2204819 UE Capability and System Information for eDRX	vivo, Guangdong Genius

R2-2205523 SIB validity with eDRX	MediaTek Inc.

Q 2.5.1 In R2-2204929, it is proposed that supported number of Rx for RedCap UEs is provided in UERadioPagingInformation.

Do you agree with the proposal? Please elaborate your reply, especially if you do not.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes |  |
| Xiaomi | - | No strong view. The CN already can identify the Redcap UE from UE initial message. If the CN knows the gNB does not support Redcap, then the CN will not forward the paging to gNB. And for RAN paging, according the Xn AP, gNB can know which cell supports Redcap or not. |

Summary – Q 2.5.1

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.5.2 In R2-2204819 it is proposed that there should be two indications in SIB1, one that indicates whether IDLE eDRX is enabled in the serving cell, and one that indicates whether INACTIVE eDRX is enabled in the serving cell.

Do you agree with the proposal? Please elaborate your reply, especially if you do not.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Comments | It depends on the discussion in 110 on whether we have separate capability for IDLE and INACTIVE UE or not. It would be good to wait a bit. |
| Xiaomi | - | Same view with intel. |

Summary – Q 2.5.2

???

Based on the observations above, the rapporteur proposes the following:

???

Q 2.5.3 In R2-2205523 it is proposed that, by default, UEs configured with eDRX should consider stored system information to be invalid after 24 hours from the moment it was successfully confirmed as valid, which is currently specified as 3 hours, and suggested to introduce an optional parameter, i.e., si-ValidityTime, in case an operator prefers to configure it with 3 hours.

Do you agree with the proposal? Please elaborate your reply, especially if you do not.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Maybe | The intention seems reasonable however it seems more logical if this kind of config is defined as UE-specific (i.e. provided in RRCRelease) as the storage requirement may be very different for a UE config with eDRX of 2.56sec vs very long values, In addition, the procedural text needs to check whether eDRX-Allowed is set by the network. |
| Xiaomi | - | Can be discussed. Seems an optimization. Unlike in NB-Iot, the UEs may not be configured with such a large eDRX cycle( Note that we also have introduced eDRX of 2.56s) then seems 3 hrs in current spec is sufficient. |

Summary – Q 2.5.3

???

Based on the observations above, the rapporteur proposes the following:

???

# 3	Conclusion

Based on the discussion above rapporteur suggests a discussion on the following proposals:

Proposal 1	???

Proposal 2	???

Proposal 3	???

Proposal 4	???

Proposal 5	???

Proposal 6	???

Proposal 7	???

Proposal 8	???

Proposal 9	???

Proposal 10	???

Proposal 11	???

Proposal 12	???

Proposal 13	???

Proposal 14	???

Proposal 15	???

Proposal 16	???

Proposal 17	???

Proposal 18	???

Proposal 19	???

Proposal 20	???

Proposal 21	???

Proposal 22	???

Proposal 23	???

Proposal 24	???

Proposal 25	???

Proposal 26	???

# References

R2-2206021	Miscellaneous corrections for RedCap WI	Ericsson	CR	Rel-17	38.331

R2-2206022	RedCap WI ASN1 RIL list	Ericsson	discussion	Rel-17

R2-2204725	[O374] correction on RedCap UE’s cell barring	OPPO	draftCR	38.331

R2-2204736	[O372] Discussion on prohibit timer for UAI for RRM relaxation fulfilment indication OPPO

R2-2204737	[O377] Correction to 38.331 on UAI for RRM relaxation fulfilment indication	OPPO

R2-2204813	[V166] Including RedCap Capability in the UERadioPagingInformation Inter-Node Message vivo, Guangdong Genius

R2-2204814	[V170] Discussion on Inter-RAT Mobility from LTE to NR for RedCap	vivo, Guangdong Genius

R2-2204929	RRC open issues on Rel17 RedCap WI	Intel Corporation

R2-2206059	[X115]38.331 Corrections on UE's behaviour of getting SIB1 for Redcap	Xiaomi Communications

R2-2206060	[X119][X114] Discussion on PDCCH-ConfigCommon for Redcap	Xiaomi Communications

R2-2206061	[X119][X114] 38.331 Corrections on PDCCH-ConfigCommon for Redcap	Xiaomi Communications

R2-2206062	[X120] 38.331 Corrections on Need code of RedCap-specific initial DL BWP for handover			Xiaomi Communications

R2-2204541	[S953] SI Request for RedCap UEs	Samsung Electronics Co., Ltd

R2-2204936	I051 support of RedCap based on intraFreqReselectionRedCap	Intel Corporation

R2-2204979	Cell reselection priority for RedCap (RIL#: S952)	Samsung

R2-2205523	SIB validity with eDRX	MediaTek Inc.

R2-2205783	Miscellaneous RedCap corrections	Nokia, Nokia Shanghai Bell	CR	38.331

R2-2205785	HD-FDD RedCap support in system information	Nokia, Nokia Shanghai Bell

R2-2206080	[H507] Corrections on cell re-selection measurements during RRC setup/resume Huawei, HiSilicon

R2-2206081	[H511] Corrections on redcapAccessRejected	Huawei, HiSilicon	CR	38.331

R2-2206082	[H513 H516 H520 H524 H525 H526 H527] Corrections on RedCap initial BWP Huawei, HiSilicon

R2-2204819	UE Capability and System Information for eDRX	vivo, Guangdong Genius