3GPP TSG RAN WG2#119-e	R2-2209121

e-Meeting, 17th - 29th August, 2022

3GPP TSG RAN WG1#110	R1-2208040

Toulouse, France, August 22nd – 26th, 2022

Title:	LS to RAN2 on RRC parameter impact for NR up to 71GHz

Release:	Rel-17

Work Item:	NR_ext_to_71GHz-Core

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Huaming Wu

huaming.wu@vivo.com

# 1	Overall description

In RAN1 #110, RAN1 reached the following agreements with RRC parameter impact

Agreement

Support the value range of reportSlotOffsetList-r17, reportSlotOffsetListDCI-0-1-r17 and reportSlotOffsetListDCI-0-2-r17 for 480kHz and 960kHz SCS to INTEGER (0..128).

- Send an LS to RAN2

Agreement

Send an LS to RAN2 to request that 38.331 captures the following: If duration-r17 is absent, the UE assumes the duration in slots is equal to L (where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17).

An example of expected changes corresponding to the second agreement is provided in Appendix for reference.

# 2	Actions

To TSG RAN2

ACTION: 	RAN1 respectfully asks RAN2 to consider the above agreement and capture it in RAN2 spec.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #110-bis-e	10 - 19 October 2022	Online

TSG RAN WG1 Meeting #111	14 - 18 November 2022 	Europe

# Appendix: Example change to TS 38.331

6.3.2	Radio resource control information elements

duration Number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the UE applies the value 1 slot, except for DCI format 2_0. The UE ignores this field for DCI format 2_0. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset). For SCS 480 kHz and SCS 960 kHz, duration-r17 is used, and the configured duration is restricted to be an integer multiple of L slots and smaller than periodicity, where L is the configured length of the bitmap monitoringSlotsWithinSlotGroup-r17. If duration-r17 is absent, the UE assumes the duration in slots is equal to L. The maximum valid duration is periodicity-L. For IAB-MT, duration indicates number of consecutive slots that a SearchSpace lasts in every occasion, i.e., upon every period as given in the periodicityAndOffset. If the field is absent, the IAB-MT applies the value 1 slot, except for DCI format 2_0 and DCI format 2_5. The IAB-MT ignores this field for DCI format 2_0 and DCI format 2_5. The maximum valid duration is periodicity-1 (periodicity as given in the monitoringSlotPeriodicityAndOffset).