3GPP TSG-RAN WG2 Meeting #119 Electronic	R2-22XXXXX

Elbonia, 17 – 26 Aug 2022

Agenda item:	5.3.1.1.1

Source:	ZTE Corporation (Rapporteur)

Title:	Report of [AT119-e][008][NR1516] RRC Conn Control II (ZTE)

WID/SID:	RRC Conn Control II

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT119-e][008][NR1516] RRC Conn Control II (ZTE)

Scope: Treat R2-2208474, R2-2208476, R2-2208553, R2-2208550, R2-2208551, R2-2208552, R2-2208579, R2-2208580, R2-2208581, R2-2207400, R2-2207401, R2-2208402, R2-2208403, R2-2208691. Determine agreeable parts, For agreeable parts, agree CRs.

Intended outcome: Report, Agreed CRs, LS out if applicable

Deadline: Schedule 1

A first round with Deadline for comments W1 Friday August 19th 1900 UTC to settle scope what is agreeable etc

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei, HiSilicon | Zhenzhen Cao | caozhenzhen@huawei.com |
| OPPO | ShiCong | shicong@oppo.com lihaitao@oppo.com – 3.4 |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |

# 3	Discussion

## 3.1	P0-AlphaSets For Msg.A

R2-2208474	Correction for field description on PUSCH	MediaTek Beijing Inc.	CR	Rel-16	38.331	16.9.0	3423	-	F	NR_2step_RACH-Core

R2-2208476	Correction for field description on PUSCH	MediaTek Beijing Inc.	CR	Rel-17	38.331	17.1.0	3424	-	A	NR_2step_RACH-Core

Issue: Current field descripton for p0-AlphaSets only specify 4-step RACH when no set is configured, 2-step RACH is not specified.

Question 1: Do companies think the issue mentioned in R2-2208474/R2-2208476 is valid?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes/No | This doesn't seem to be a RAN2 issue. |
| OPPO | No | p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3 and msgA PUSCH), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH. As specified in field description, this filed is not used to configure p0-pusch and alpha for 4-step RACH or 2-step RACH as yellow highlighted. The last sentence intends to clarify how to handle the case when the set is absent, i.e., UE shall refer to P0-nominal for msg3 PUSCH. There is no motivation to also introduce MsgA PUSCH as a reference, so we think current spec is clear and no change is needed. |
| vivo | No | The correction is not so accurate. Firstly, P0-nominal is only used for PUCCH but not for MsgA PUSCH. Besides, msg3-Alpha will not be used for MsgA PUSCH if msgA-Alpha is provided. Anyway, all the details are given in the 38.213 spec, we don’t see the necessity to clarify everything clearly in the RRC field description. |

If the issue is valid, companies are invited to provide the comments on the change:

- Add more field description for p0-AlphaSets when no set is configured to cover power related parameter for 2-step RACH case

p0-AlphaSets configuration {p0-pusch, alpha} sets for PUSCH (except msg3 and msgA PUSCH), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3 PUSCH and msgA PUSCH, P0-UE is set to 0 and alpha is set according to msg3-Alpha configured for msg3 PUSCH and msgA PUSCH.

Question 2: If companies think the issue is valid, do companies agree with above change suggested in R2-2208474/R2-2208476?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | Should be discussed in RAN1 first, and even it is needed, most likely there should be separate configurations. |
| OPPO | No |  |

## 3.2	Bearer Type Change

R2-2208553	Considerations on sn-fieldlength change in the case of bearer type change	ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell, CATT	discussion	Rel-15	NR_newRAT-Core

Chair comment: Postponed last meeting

R2-2208550	CR on 38.331 for sn-FieldLength change for the case of bearer type change	ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell, CATT	CR	Rel-15	38.331	15.18.0	3436	-	F	NR_newRAT-Core

R2-2208551	CR on 38.331 for sn-FieldLength change for the case of bearer type change	ZTE Corporation, Sanechips,Nokia, Nokia Shanghai Bell, CATT	CR	Rel-16	38.331	16.9.0	3437	-	A	NR_newRAT-Core

R2-2208552	CR on 38.331 for sn-FieldLength change for the case of bearer type change	ZTE Corporation, Sanechips, Nokia, Nokia Shanghai Bell, CATT	CR	Rel-17	38.331	17.1.0	3438	-	A	NR_newRAT-Core

Description of the issue: Observation 1: According to the L2 action of the bearer type change defined in the TS 37.340, the RLC entity of a DRB shall be released in one node and newly established in the other node for bearer type change case (MCG to SCG, or SCG to MCG). Table A-1: L2 handling for bearer type change with and without a security key change due to a change of the termination point. Bearer type change from row to col; MCG; Split; SCG. no change of termination point (no key change); change of termination point (key change); no change of termination point (no key change); change of termination point (key change); no change of termination point (no key change); change of termination point (key change). MCG; N/A; PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action; PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure; PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure; PDCP: Recovery MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure; PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure. Split; PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: See Note 4 SCG MAC: Reconfigure; PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 4 SCG MAC: Reconfigure; N/A; PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 1 SCG MAC: See Note 1; PDCP: Recovery MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action; PDCP: Re-establish MCG RLC: See Note 3 MCG MAC: Reconfigure SCG RLC: See Note 1 SCG MAC: See Note 1. SCG; PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: Reconfigure; PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 4 SCG MAC: Reconfigure; PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action; PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 1 SCG MAC: See Note 1; N/A; PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 1 SCG MAC: See Note 1. NOTE 3: For EN-DC and NGEN-DC: Re-establishment and release. For NE-DC and NR-DC: Release. NOTE 4: For NE-DC: Re-establishment and release. For EN-DC, NGEN-DC and NR-DC: Release. Observation 2: Given the lack of the sn-Fieldlength information of DRBs in the inter-node RRC message, the L2 action to the bearer type change defined in TS 37.340 can not be implemented due to the restriction of ‘The value of sn-FieldLength for a DRB/multicast MRB shall be changed only using reconfiguration with sync’

Question 3: Do companies agree with the above issue observed in above?

| Company | Yes/No | Technical Arguments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | The intention of this CR should be aligned with the intention of the spec text, but the spec may be ambiguous. |
| OPPO | Yes | We think the potential misunderstanding is that the field description says the sn-Fieldlength can be changed only by using reconfiguration with Sync, however for the bearer type change defined in 37.340, the RLC will be release/established in MCG/SCG for MCG/SCG or SCG/MCG bearer type change case. Then, if the value of sn-Fieldlength can only be changed by reconfiguration with sync, it means for the MCG/SCG or SCG/MCG bearer type change cases, the sn-Fieldlength should be the same. However somehow it’s not feasible because there is no such info in the inter-node message. |
| vivo | Yes | We agree with the analysis mentioned. |
| Ericsson | Yes |  |

If companies think the issue is valid, please provide the comments on the below options for resolving above issue:

- Option 1: Implement the bearer type change via reconfigurationwithSynch or bearer add/release, some clarifications in the current TS 37.340 are needed.

- Option 2: Loose the restriction of the sn-fieldlength change defined in TS 38.331. the correction on TS 38.331 is needed.

- Option 2-1: Narrowing down the range of restriction of the sn-filedlength change from DRB level to RLC level such as ‘the value of sn-fieldlength of a RLC entity for a DRB shall be changed only using reconfiguration with sync’

- Option 2-2: Remove the restriction sentence ‘the value of sn-fieldlength for a DRB shall be changed only using reconfiguration with sync’ in the field description of sn-FieldLength

- Option 3: Include the information element sn-fieldlength in the inter-node message, the correction on TS 38.331 is needed, and need send an LS to RAN3.

Question 4: If the issue is confirmed, which option do you prefer?

| Company | Option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | 2-1 | Option 2-1 we think should be original intention of the spec text. |
| OPPO | Prefer Option1/3 |  |
| vivo | Option 2-1 | Option 2-1 resolves the issue with minimum restriction change. |
| Ericsson | 2-1 | This was the intention from the beginning |

For companies who prefer option 2-1, please comments on the corresponding change present in the CR R2-2208550, R2-2208551, R2-2208552:

R15/R16: sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength of a RLC entity for the DRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

R17: sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The value of sn-FieldLength of a RLC entity forthe DRB/multicast MRB shall be changed only using reconfiguration with sync. The network configures only value size12 in SN-FieldLengthAM for SRB.

Question 5: If the issue is confirmed, do companies agree with the change provided in R2-2208550/R2-2208551/R2-2208552?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | The changes look good. For the conditional presence for Reestab, “The field is mandatory present at bearer setup” should be “The field is mandatory present at RLC bearer setup.” |
| vivo | Yes |  |
| Ericsson | Yes |  |

For companies who prefer option 2-2, please comments on the corresponding change present in the CR R2-2208550, R2-2208551, R2-2208552:

R15/R16: sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The network configures only value size12 in SN-FieldLengthAM for SRB.

R17: sn-FieldLength Indicates the RLC SN field size, see TS 38.322 [4], in bits. Value size6 means 6 bits, value size12 means 12 bits, value size18 means 18 bits. The network configures only value size12 in SN-FieldLengthAM for SRB.

Question 6: If the issue is confirmed, do companies agree with the change provided in R2-2208550/R2-2208551/R2-2208552?

| Company | Yes/No | Comments |
| --- | --- | --- |

## 3.3	PDCP discardTimer

R2-2208579	38.331 cr(Rel-17) correction on the condition of configuring discardTimer	Xiaomi	CR	Rel-17	38.331	17.1.0	3447	-	F	NR_newRAT-Core

Moved from 6.0.3

R2-2208580	38.331 cr(Rel-16) correction on the condition of configuring discardTimer	Xiaomi	CR	Rel-16	38.331	16.9.0	3448	-	F	NR_newRAT-Core

Moved from 6.0.3

R2-2208581	38.331 cr(Rel-15) correction on the condition of configuring discardTimer	Xiaomi	CR	Rel-15	38.331	15.18.0	3449	-	F	NR_newRAT-Core

Moved from 6.0.3

Issue: According to 38.323, it says that: “a) discardTimer This timer is configured only for DRBs. The duration of the timer is configured by upper layers TS 38.331 [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.” However, in 38.331, the discardTimer IE uses condition setup, which has the following condition: “The field is mandatory present in case of SRB or DRB setup. Otherwise the field is optionally present, need M.” Thus, there is misalignment between 38.331 and 38.323.

Question 7: Do companies think the issue raised by R2-2208581 is valid?

| Company | Yes/No | Technical Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | The current text has no problem. The network will not signal it when it is not needed. |
| OPPO | Yes | Seems this is a typo in 331 the parent IE is drb, i.e., not related to SRB at all? |
| vivo | No | Combining SDAP and RRC specs, we can know that the NW would only configure this timer for DRBs, which is the intended behavior. There is no misalignment. |

If above issue is confirmed, companies are invited to provide the comments on the suggested solution in R2-2208581:

- To align with 38.323 that discardTimer is only applicable to DRB.

DCP-Config ::= SEQUENCE { drb SEQUENCE { discardTimer ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150, ms200, ms250, ms300, ms500, ms750, ms1500, infinity} OPTIONAL, -- Cond DRB pdcp-SN-SizeUL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup2 pdcp-SN-SizeDL ENUMERATED {len12bits, len18bits} OPTIONAL, -- Cond Setup2 headerCompression CHOICE { notUsed NULL, rohc SEQUENCE { maxCID INTEGER (1..16383) DEFAULT 15 /*omit for short*/ DRB This field is mandatory present when the corresponding DRB is being set up, absent for SRBs. Otherwise this field is optionally present, need M.

Question 8: If the issue is confirmed, do companies agree with above change provided in R2-2208581?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes with | It’s ok the update the spec, but it seems this is a typo. BTW, the format of the CR needs to be adjusted |

## 3.4	DAPS

R2-2207400	Correction to RLF configuration in case of DAPS HO	Fujitsu	CR	Rel-16	38.331	16.9.0	3255	-	F	NR_Mob_enh-Core

R2-2207401	Correction to RLF configuration in case of DAPS HO	Fujitsu	CR	Rel-17	38.331	17.1.0	3256	-	A	NR_Mob_enh-Core

Issue: According to current TS 38.331, if any DAPS bearer is configured, - In case that rlf-TimersAndConstants is not configured for a cell group, or the SpCellConfig contains the rlf-TimersAndConstants but the received rlf-TimersAndConstants is set to release, the UE shall use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1. However, regarding SIB1, UE implementation can be different, including SIB1 for source cell or SIB1 for target cell. Also, UE and gNB may have different understandings on the SIB1.

Question 9: Do companies think the issue raised by R2-2207400/R2-2207401 is valid?

| Company | Yes/No | Technical Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| OPPO | No | We think the current spec is correct because UE does not read target cell’s SIB1 during DAPS HO. |
| vivo | No | Obvious it is referred to as the SIB1 for target cell based on the procedural text. There is no room for misunderstanding. |

If above issue is confirmed, companies are invited to provide the comments on the suggested solution in R2-2205563:

The following changes are suggested:

- Add “for the target SpCell” to specify that the UE uses RLF parameters received from target cell for target cell group, in chapter 5.3.5.5.6, 5.3.5.5.7

The First change: ##### 5.3.5.5.6 RLF Timers & Constants configuration The UE shall: 1> if the received rlf-TimersAndConstants is set to release: 2> if any DAPS bearer is configured: 3> use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1 for the target SpCell; 2> else: 3> use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

The Second change: ##### 5.3.5.5.7 SpCell Configuration The UE shall: 1> if the SpCellConfig contains the rlf-TimersAndConstants: 2> configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6; 1> else if rlf-TimersAndConstants is not configured for this cell group: 2> if any DAPS bearer is configured: 3> use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1 for the target SpCell; 2> else 3> use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1;

Question 10: If the issue is confirmed, do companies agree with above change in R2-2207400/R2-2207401

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

R2-2208402	Clarification on headerCompression for DAPS bearer	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.9.0	3416	-	F	NR_Mob_enh-Core

R2-2208403	Clarification on headerCompression for DAPS bearer	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.1.0	3417	-	A	NR_Mob_enh-Core

Issue: For DAPS bearers, the PDCP entity is configured with two sets of security functions and keys and two sets of header compression protocols, associated with source cell and target cell, respectively. So the target cell can reconfigure headerCompression for PDCP entity associated with DAPS bearer. According to the field description for headerCompression, the network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment, and without any drb-ContinueROHC. However, for DAPS bearers, no PDCP re-establishment shall be performed. The network only reconfigures the PDCP entity to configure or release DAPS.

Question 11: Do companies think the issue raised by R2-2208402/R2-2208403 is valid?

| Company | Yes/No | Technical Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| OPPO |  | Not sure what is the case for NW configuring headerCompression when reconfiguring the PDCP entity to release DAPS. We understand it simply removes the source set. |
| vivo | Yes |  |

If above issue is confirmed, companies are invited to provide the comments on the suggested solution in R2-2208402/R2-2208403:

The following changes are suggested:

- Update the field description for headerCompression to clarify that the network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure or release DAPS, and without any drb-ContinueROHC.

#### – PDCP-Config headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configured ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC and EHC can be both configured simultaneously for a DRB. The network reconfigures headerCompression only upon reconfiguration involving PDCP re-establishment or involving PDCP entity reconfiguration to configure or release DAPS, and without any drb-ContinueROHC. Network configures headerCompression to notUsed when outOfOrderDelivery is configured..

Question 12: If the issue is confirmed, do companies agree with above change in R2-2208402/R2-2208403

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes,but | In the above change, the wording “to configure or release DAPS” has some ambiguities, e.g. it may be understood that non-DAPS bearer(s) will be configured with headerCompression. So we propose to improve the wording as below: …. or involving PDCP entity reconfiguration to configure DAPS bear(s) or to release DAPS bear(s) |
| vivo | Agree with intention | DAPS should be DAPS bearer. |

R2-2208691	Clarification on reestablishRLC for DAPS HO	ZTE Corporation, Sanechips Late

Issue: Observation 1: According to the current RRC specs, when the security key is changed for the target cell, the NW should set the reestablishRLC to true for the RLC entity associated with the target cell, regardless of whether the RLC bearer is associated with a DAPS bearer or not. reestablishRLC Indicates that RLC should be re-established. Network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2, multicast MRBs and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true.. Observation 2: According to the text procedure in 5.3.5.5.4 RLC bearer addition/modification, the UE will not use the reestablishRLC, if the RLC bearer is associated with a DAPS bearer, or if any DAPS bearer is configured and the RLC bearer is associated with an SRB. ##### 5.3.5.5.4 RLC bearer addition/modification For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall: 1> if the UE's current configuration contains an RLC bearer with the received logicalChannelIdentity/LogicalChannelIdentityExt within the same cell group: 2> if the RLC bearer is associated with an DAPS bearer, or 2> if any DAPS bearer is configured and the RLC bearer is associated with an SRB: 3> reconfigure the RLC entity or entities for the target cell group in accordance with the received rlc-Config; 3> reconfigure the logical channel for the target cell group in accordance with the received mac-LogicalChannelConfig; 2> else: 3> if reestablishRLC is received: 4> re-establish the RLC entity as specified in TS 38.322 [4]; 3> reconfigure the RLC entity or entities in accordance with the received rlc-Config; 3> reconfigure the logical channel in accordance with the received mac-LogicalChannelConfig; 3> if servedMBS-RadioBearer is received: 4> associate this logical channel with the PDCP entity identified by servedMBS-RadioBearer;. There are some misalignment between the field description of reestablishRLC and the text procedure on RLC bearer addition/modification. According to the field description, the NW must configure reestablishRLC when the security key used for the radio bearer associated with this RLC entity changes, even if the radio bearer is associated with the DAPS bearer or SRB in DAPS HO. But the UE will ignore this IE based on the text in RRC specs. So no need such strict restriction on reestablishRLC for DAPS bearer and SRB in DAPS HO.

Question 13: Do companies agree with the above issue observed in above?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | It was discussed at RAN2#112-e meeting. In the report below, section 2.3 is about RLC reestablishment discussion for DAPS (for discussing the CR R2-2010297). The majority of views is that in case of DAPS, a new RLC entity is established for the target, so it is not a reconfiguration of source RLC. In addition, lots of companies think if such changes are agreeable, we may have to specify alll cases for RLC re-establishment. In the end, the relevant CR was not agreed. R2-2010727 [AT112-e][213][MOB] DAPS RRC corrections Ericsson discussion Rel-16 NR_Mob_enh-Core, LTE_feMob-Core For the issue mentioned in Q13, we understand that RLC re-establishment is not needed, and there is no need to update the current spec. |
| OPPO | Yes |  |
| vivo | No | Agree with Huawei. |

If above issue is valid, companies are invited to provide the comments on the suggested proposal and change in R2-2208691:

The following proposal is suggested to remove the restriction on reestablishRLC for DAPS bearer and SRB in DAPS HO:

Proposal 1: The NW may or may not configure the reestablishRLC for a RLC bearer if the RLC bearer is associated with a DAPS bearer, or if any DAPS bearer is configured and the RLC bearer is associated with an SRB.

And the corresponding change is provided:

#### – RLC-BearerConfig reestablishRLC Indicates that RLC should be re-established. If the RLC bearer is associated with a DAPS bearer, or if any DAPS bearer is configured and the RLC bearer is associated with an SRB, network may or may not set this to true. Otherwise, network sets this to true at least whenever the security key used for the radio bearer associated with this RLC entity changes. For SRB2 and DRBs, unless full configuration is used, it is also set to true during the resumption of the RRC connection or the first reconfiguration after reestablishment. For SRB1, when resuming an RRC connection, or at the first reconfiguration after RRC connection reestablishment, the network does not set this field to true..

Question 14: If the issue is valid, do companies agree with above proposal and change in R2-2208691

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | See our comments for Q13. |
| OPPO | Yes |  |

# 4	Conclusion

TBD.