3GPP TSG-RAN WG2 Meeting #119 electronic	R2-220

# Electronic, 17th - 29th August, 2022

Source:	Huawei, HiSilicon

Title:          [Offline-406][POS] Rel-15 16 positioning RRC (Huawei)

Agenda Item:	5.3.2

Document for:	Discussion and Decision

- Introduction

This document provides a summary of the following contributions for R15/R16 RRC corrections for the following email discussion

[AT119-e][406][POS] Rel-15/16 positioning RRC (Huawei)

Scope: Evaluate the CRs in R2-2207408/R2-2207561/R2-2207873/R2-2207874/R2-2207875/R2-2207876 and check for agreeability.

Intended outcome: Agreed CRs

Deadline: Tuesday 2022-08-23 1200 UTC

The following has been submitted to AI 5.3.2

| R2-2207408 | Change request about Periodicity in SRSp configuration | vivo |
| --- | --- | --- |
| R2-2207561 | Change request about Periodicity in SRSp configuration | vivo |
| R2-2207873 | Correction for SRS-PeriodicityAndOffset-R16 | Huawei, HiSilicon |
| R2-2207874 | Correction for SRS-PeriodicityAndOffset-R17 | Huawei, HiSilicon |
| R2-2207875 | Correction for the capability of SRS-PeriodicityAndOffset-R16 | Huawei, HiSilicon |
| R2-2207876 | Correction for the capability of SRS-PeriodicityAndOffset-R17 | Huawei, HiSilicon |

  - Contacts

| Name | Company | Email |
| --- | --- | --- |

- Discussion on SRS periodicities

  - Correction to RRC spec

- 2.1.1	SRS periodicity and SCS

In R2-2207873/R2-2207874, it is also mentioned that certain SRS periodicity should only be restricted to certain SCS.

In R2-2207409/R2-2207561, it is mentioned that there are the following restrictions on the periodicity and SCS

for 30/60/120kHz only sl20480 INTEGER(0..20479) for 60/120kHz only sl40960 INTEGER(0..40959) for 120kHz only sl81920 INTEGER(0..81919)

Hence, the following change is proposed

Question1, do companies agree that restriction should be added for RRC spec for the configuration of SRS periodicity that

- Periodicity sl20480 is only applicable for scs of 30/60/120kHz

- Periodicity sl40960 is only applicable for scs of 60/120kHz

- Periodicity sl81920 is only applicable for scs of 120kHz

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel |  | Should not it be partially covered by the change in R2-2207873 as “For SRS-PosResource, sl20480, sl40960 and sl81920 cannot be configured for SCS=15kHz, sl40960 and sl81920 cannot be configured for SCS=30kHz, and sl81920 cannot be configured for SCS=60kHz. “? |
| ZTE | Yes |  |
| Qualcomm | Yes | Not essential, but O.K. to merge into another CR. |
| CATT | Yes |  |
| Ericsson | Yes | Same view as QC, can be merged to Huawei CR below |
| vivo | Yes | Agree to follow HW’s version to add the restrictions in the periodicityAndOffset fields. |
| Lenovo | Yes but | Is already covered by the CRs in R2-2207873/R2-2207874. |
| Samsung | Yes | Same view with QC and E//. This can be merged to R2-2207873/R2-2207874. |

- 2.2.2	Missing SRS periodicities,

In R2-2207873/R2-2207874, it is mentioned that the following agreements have been made regarding the periodicities of SRS configuration.

Agreement: The following periodicity values of DL PRS resource allocation are supported depending on SCS - 2μ{4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively

However, for the current list of periodicities, it is found that the values corresponding to 64*2μ for μ=1,2,3 are missing and the values corresponding to 10240*2μ for μ=1. Thus, it is proposed that these new values need to be added.

Question2, do companies agree that the new value for SRS periodicities sl128, sl256, sl512, and sl20480 should be added for periodic and semi-persistent SRS?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | Looks ok, the space should be removed between extension mark and comma. |
| ZTE | Yes |  |
| Qualcomm | Yes |  |
| CATT | Yes |  |
| Ericsson | Yes |  |
| vivo | Yes |  |
| Lenovo |  | We are bit confused here about the SRS periodicities sl128, sl256, sl512 and are wondering what takes precedence: the agreements which have been minuted in the RAN1#99 report or the official information given in the RAN1 consolidated parameter list, e.g. in R1-2003190 (received per LS R2-2004380 in RAN2#110-e). In that parameter list the periodicities sl128, sl256, sl512 are not mentioned (see tab “Positioning” and row #66). We agree that the SRS periodicity sl20480 is currently missing in RRC. Furthermore, we don’t see the stringent need to add the condition below in the field descriptions. Normally, we trust NW implementation to handle the signaling correctly. “When the field periodicityAndOffset-p/sp is present, the field periodicityAndOffset-p-Ext/sp-Ext shall be absent.” |
| Samsung | Yes | But we can’t understand why the new field periodicityAndOffset-sp-Ext-r16 should be newly defined. We can simply use the extension mark in SRS-PeriodicityAndOffset-r16 to add those values there. Also in R2-2207873, the below sentence in the field descriptions seems not needed. Since periodicityAndOffset-p/periodicityAndOffset-sp are mandatory fields, they should be always present. “When the field periodicityAndOffset-p/sp is present, the field periodicityAndOffset-p-Ext shall be absent.” |

  - Correction to UE capability spec

With the newly introduced SRS periodicity, there are backward compatibility issues that old UE may not support the newly added values.

Then, in R2-2207873/R2-2207874, it is proposed that a new capability is added for the newly added SRS periodicities. The field description for this new capability field is added in R2-2207875/R2-2207876

Question3: Do companies agree that new capability is needed for the newly added periodicities for SRS?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | Yes | Looks ok, for 306 CR, the suffix -r16 should be added for the field name srs-PeriodicityAndOffsetExt in the field description. |
| ZTE | Yes |  |
| Qualcomm | Yes |  |
| CATT | Yes | Mulit-RTT positioning will be added in the cover sheet. Impacted functionality: UL-TDOA positioning, and UL-AOA positioning |
| Ericsson | Yes |  |
| vivo | Yes |  |
| Lenovo |  | 38.331 CRs in R2-2207873/R2-2207874: The addition of UE capability srs-PeriodicityAndOffsetExt-r16 in IE Phy-ParametersCommon is NBC. Should discuss whether NBC change is acceptable or not. We prefer a backwards-compatible change. 38.306 CRs in R2-2207875/R2-2207876: - Rel-16 CR - Cover page: wrong meeting start date “15th”; wrong CR#0781 added (should be #0780). - Suffix “-r16” is missing for the UE capability and the description has not been added in alphabetical order. - Rel-17 CR - Cover page: wrong meeting start date “15th”. - The capability name srs-ExtendedPeriodicityAndOffset-v16xy is not aligned with the 38.331 CRs and the Rel-16 38.306 CR. - The description has not been added in alphabetical order. - For both CRs: To keep the context we think a condition should be added in the capability description, e.g. “The UE indicating support of this feature shall also indicate the support of srs-PosResources-r16 or srs-PosResourceSP-r16”. |
| Samsung | Yes |  |

- Conclusion

Based on the summary as above, we propose the following for discussion:

TBD