3GPP TSG RAN WG2 Meeting #119-e                                           Draft R2-220xxxx

# Electronic meeting, 17- 29 Aug, 2022

Agenda item:			6.11.1

Source:			Intel Corporation

Title:		Report of [AT119-e][409][POS] Rel-17 positioning capabilities (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the report of following offline discussion:

- [AT119-e][409][POS] Rel-17 positioning capabilities (Intel)

Scope: Check and update the rapporteur CR in R2-2207385 to take account of decisions of this meeting.  Evaluate the proposals in the following tdocs:

- R2-2208492

Intended outcome: Agreeable CR

Deadline: Tuesday 2022-08-23 1200 UTC

I would like to split the discussion into two phases:

Phase 1: to provide your view on issues;     Deadline:  Saturday 2022-08-20 1800 UTC

Phase 2: To check TPs; Deadline:  Tuesday 2022-08-23 1200 UTC

Following stage 2 changes are discussed in the offline discussion:

R2-2207385	Corrections on LPP capabilies	Intel Corporation

R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo

# Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |

# Discussion

## Changes from R2-2207385	Corrections on LPP capabilies	Intel Corporation

Reason for change:

Based on RAN1 latest feature list in R1-2205607 and RAN4 R4-2211189, the Prerequisite feature groups and some clarifications for following LPP capabilities are missing:

27-1-1 nr-UE-RxTEG-ID-MaxSupport-r17

27-1-2/27-1-2a  nr-UE-TxTEG-ID-MaxSupport-r17

27-1-3  nr-UE-RxTxTEG-ID-MaxSupport-r17

27-1-4   measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

27-1-4a   measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17

27-2-1 maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17 (in addition, restriction “The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2)” should be added;)

27-2-2 maxDL-PRS-RSRP-MeasurementFR1-r17, maxDL-PRS-RSRP-MeasurementFR2-r17

27-3-1 supportedDL-PRS-ProcessingSamples (In addition, a note should be added as

Note: this feature is supported for both UE-assisted and UE based positioning

)

27-3-2

prs-ProcessingWindowType1A-r17

prs-ProcessingWindowType1B-r17

prs-ProcessingWindowType2-r17

(A note should be added as Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP)

27-3-3 prs-ProcessingCapabilityOutsideMGinPPW (no change is needed)

27-4-1 nr-los-nlos-AssistanceDataSupport-r17 (in addition, a Note should be added as NOTE:	A single value is reported when both Multi-RTT and DL-TDOA are supported.)

27-6  (no change is needed)

dl-PRS-BufferType-RRC-Inactive-r17

durationOfPRS-Processing-RRC-Inactive-r17

maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17

27-7  (no change is needed)multiMeasInSameMeasReport-r17

27-8 nr-PosCalcAssistanceSupport-r17	(bit 3 DL-TDOA, bit 2 DL-AoD, )

27-9  (no change is needed) lowerRxBeamSweepingThan8-FR2-r17

27-10a mg-ActivationRequest-r17

27-12 nr-los-nlos-IndicatorSupport-r17(no change is needed)

27-13 additionalPathsExtSupport-r17(no change is needed)

27-13a supportOfDL-PRS-FirstPathRSRP-r17

27-14 additionalPathsExtSupport-r17(no change is needed)

27-14a supportOfDL-PRS-FirstPathRSRP-r17

27-15 posSRS-RRC-Inactive-InInitialUL-BWP-r17(no change is needed)

27-15b posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 (in addition, some notes should be added

Note 2: If component 9 is not signaled, the UE only supports same center frequency  between the SRS for positioning and initial UL BWP

Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP

Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.

)

27-15a maxNumOfSemiPeristentSRSposResources-r17										maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

27-15c maxNumOfSemiPeristentSRSposResources-r17										maxNumOfSemiPersistentSRSposResourcesPerSlot-r17

27-16a

olpc-SRS-PosRRC-Inactive-r17

27-18a/27-18b/27-18c dl-PRS-MeasRRC-Inactive-r17

27-19a

spatialRelationsSRS-PosRRC-Inactive-r17

27-20 dl-PRS-ResourcePrioritySubset-Sup-r17 (no change is needed)

27-21 nr-DL-PRS-BeamInfoSup-r17(no change is needed)

27-22 nr-PosCalcAssistanceSupport-r17, beamInfoSup(no change is needed)

14-2 supportedDL-PRS-ProcessingSamples-RRC-Inactive-r17

.

Summary of change::

To add the Prerequisite feature groups for LPP capabilities introduced in Rel-17 and corresponding clarifications based on RAN1/RAN4 latest feature list in R1-2205607 and R4-2211189

Consequences if not approved:

It is unclear on how to set the LPP capabilities.

Moderator’s comments:

- It is to align with latest RAN1/4 feature lists;

Moderator would like to check companies’ view:

Discussion point 3.1: Do you agree the changes in R2-2207385?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |

## Changes from R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo

Reason for change:

According to RAN4 LS R2-2206945, RAN2 needs to introduce the UE capability for support of Rx timing difference between the serving cell and non-serving cell for PRS measurement within a PPW in NR-DL-PRS-ProcessingCapability of TS 37.355. The capability type shall be per band.

- The applicability condition on Rx timing difference between the serving cell and a neighbor cell/TRP for PRS measurements within a PPW is max ∆T≤THR, where - ∆T is the time difference between the start of a slot containing PRS from the neighbor cell/TRP and the start of the closest slot from the serving cell, , and - the range of ∆T is determined by the expected RSTD and expected RSTD uncertainty in the assistance data. - THR is the threshold, which is up to UE capability (as already informed in R4-2206981) - The candidate values for THR include {CP length, ¼ symbol length, ½ symbol length, ½ slot length}

Summary of change::

Introduce a new UE capability for support of Rx timing difference between the serving cell and non-serving cell for PRS measurement within a PPW in NR-DL-PRS-ProcessingCapability;

Consequences if not approved:

Incomplete UE capability for PRS measurement.

Moderator’s comments:

The capability is indicated in RAN4 feature list as 14-3, and has been captured in RRC/306 based on latest RAN4 feature list. RAN4 did not agree to add this in LPP. Therefore I would suggest not to add it in LPP unless RAN4 requests us to do so.

| 14. NR_pos_enh | 14-3 | PRS measurement without MG | Capability for the threshold used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG. | 27-3-2 | yes |  |  | Per band | No | No | N/A | The candidate threshold values: CP length, 1/4 symbol, 1/2 symbol, half of slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Moderator would like to check companies’ view:

Discussion point 3.2: Do you agree the changes in R2-2208492?

| Company’s name | Yes or No? | Comments, if any |
| --- | --- | --- |

# Summary report and proposals

For agreement:

Online discussion:

# Reference

- R2-2207385	Corrections on LPP capabilies	Intel Corporation

- R2-2208492	Change request about UE capability for PRS measurement within a PPW	vivo