3GPP TSG-RAN WG2 Meeting #119-bis Electronic	R2-2210029

Online, Oct 10– 19, 2022

Agenda Item:	6.1.3

Source:	MediaTek Inc.

Title:	Correction on MBS capabilities

Document for:	Discussion, Decision

Introduction

In the last meeting of RAN2#119e, the issue for FG33-1-1 and FG33-1 is discussed. In the email discussion [AT119-e][602][MBS-R17] CP Other corrections (MediaTek), the following proposals are made by the rapporteur:

Proposal-9: FG 33-1-1 (slot-level repetition up to 16 for broadcast MTCH) should be defined as an optional feature with UE capability signalling (To be discussed online).

In the online meeting, the issue is discussed and the following agreement is made:

DISCUSSION P9: - QCM agrees with P9. - Ericsson thinks that in case it is supposed to work for IDLE/INACTIVE, this would have to be mandatory capability for MBS broadcast UEs. LGE agrees and is not sure why this was separated. Intel, Nokia, CATT agrees. - Huawei has not strong view on the capability signalling or not, but thinks the intention from RAN1 side to separate it was to make it optional. The feature can be used for specialized cases, e.g. Public Safety. Mediatek agrees and indicates that 8-times repetition is already supported anyway. QC, vivo agree. - Chair wonders if we need to ask RAN1 why they separated this? We keep FG 33-1-1 as a separate optional capability from FG-33-1 (follow RAN1). FFS whether there is capability bit.

In this contribution, we further the separate capability or the broadcast feature FG33-1-1, then provides the further corrections for the issue.

Discussion

In the spec, the broadcast feature is defined as the optional feature without signalling, which is supported by all UEs supporting broadcast. If a feature is defined as a separate optional capability from broadcast FG33-1, it should be defined as a capability with signaling. Otherwise, it will be added into the minimum capability for broadcast reception.

FG33-1-1 is defined as the 16 times dynamic slot-level repetition for broadcast MTCH. Meanwhile in FG33-1, the feature for broadcast is supported with 8-times repetition. If so, the UE which support 8-times repetition is not able to receive broadcast, which will increase the threshold of broadcast reception. If we support FG33-1-1 as a separate optional capability with capability signaling, it can be taken as an optional feature specific for CONNECTED UE with enhanced capability.

In the last meeting, it is confirmed by RAN2 that the FG33-1-1 is a separate optional capability from FG-33-1. Therefore, we think it is natural to add a capability bit for the FG-33-1-1.

Proposal 1: Introduce the UE capability with capability bit for FG33-1-1 and add to the specs.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: Introduce the UE capability with capability bit for FG33-1-1 and add to the specs.

References

R2_119-e MBS session notes (Dawid) 2022-08-28 EOM_rev1

R2-2208882 [AT119-e][602][MBS-R17] CP Other corrections (MediaTek)