3GPP TSG-RAN WG2 Meeting #119bis-e	R2-2210537

Electronic, 10th – 19th October, 2022

Agenda Item:		8.5.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on BSR enhancement for XR-specific capacity improvement

Document for:	Discussion and decision

1	Introduction

In RAN2#119 e-meeting [1], the following agreements for XR-specific capacity improvement were reached.

- 1: As starting point, RAN2 can further discuss the solutions in TR 38.838 that can impact on L2 operation (e.g., BSR, LCP, assistance information for scheduling, packet discarding, prioritization) for XR-specific capacity improvement. RAN2-specific solutions are not precluded (even if RAN1 hasn’t discussed them before). - 1: Enhancement to SPS/CG should be justified for XR scheduling and should be evaluated against dynamic grant (DG) scheduling - which should be considered as baseline. Should justify why enhancements are needed. - RAN2 considers SPS enhancements may not be needed in Rel-18 XR since PDCCH capacity is not assumed to be a problem for XR. FFS if SPS has some power consumption benefits.

In this contribution, we will discuss the following three issues related to the BSR enhancement:

- BSR content

- Trigger of BSR

- Granularity of buffer size

- Discussion

In 3GPP TR 38.838 [2], the UL traffic models for VR, Cloud gaming and AR are provided as shown in Table 1.

Table 1: Traffic models for UL XR applications

| Category | Packet Size (byte) | Periodicity (ms) | PDB (ms) | Jitter (ms) |
| --- | --- | --- | --- | --- |
| VR | 100 | 4 | 10 | No jitter |
| Cloud Gaming | 100 | 4 | 10 | No jitter |
| UL AR | variable (following truncated Gaussian distribution) | 16.67 | 30 | Optional [-4,4] |

According to Table 1, the UL XR applications can be divided into two categories.

UL VR/Cloud gaming traffic: integer periodicity without jitter and fixed packet size, CG can be used for this category.

UL AR traffic: non-integer periodicity with optional jitter and variable packet size, DG can be used for this category.

- CG can be used for UL VR/Cloud gaming traffic.

- DG can be used for UL AR traffic.

In the following section, we will discuss some potential BSR enhancements for UL AR traffic, so that the network can obtain more information to help with the scheduling of DG resources for UEs, thereby increasing the system capacity.

  - BSR content

According to the current specification [3], UE will report buffer size for each LCG.

Since the PDB is larger than the periodicity for the UL AR traffic, a LCH may buffer more than one data burst which have different remaining PDBs at the same time. If the UE reports BSR by using the current BSR MAC CE format, the network cannot identify the exact data volume for each data burst. The network cannot know the remaining PDB for each data burst either, since it is not aware of the exact arrival time of the first data burst due to jitter. As a result, the network may schedule DG resource for the UE too late or too early, which will affect both the service performance of the UE and the network capacity.

As shown in Figure 1, UE reports BSR by using the current BSR MAC CE format at time T1, and the buffer size includes the data volume of data burst 1 and data burst 2.

Case 1: network schedules resource for UE too late:

Since the network does not know the reaming PDB information, the network may schedule the resource for the UE after T2, and the PDB of the data burst 1 is already exceeded then.

Case 2: network schedules resource for UE too early:

Suppose the network schedules sufficient resources to accommodate both the data burst 1 and data burst 2 before T2, but in fact there is still a lot of delay budget left for data burst 2 which no need to be scheduled in hurry. However since the network does not know this information, it scheduled more resources than the UE really needs, it may not be able to schedule enough resources for other UEs.

Figure 1: Example of BSR

If the network can know the data volume and the associated remaining PDB for each data burst, the network can perform a better scheduling by considering all UEs’ requirements, and the system capacity can be improved.

Hence, we propose to enhance the BSR MAC CE format in order to include the data volume of each data burst and its associated remaining PDB.

- Enhance the BSR MAC CE to report the data volume of each data burst and its associated remaining PDB.

  - Trigger of BSR

According to the current specification [3], BSR will be triggered by some specific events, see below:

### 5.4.5 Buffer Status Reporting …… A BSR shall be triggered if any of the following events occur for activated cell group: - UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either - this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or - none of the logical channels which belong to an LCG contains any available UL data. in which case the BSR is referred below to as 'Regular BSR'; - UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC CE plus its subheader, in which case the BSR is referred below to as 'Padding BSR'; - retxBSR-Timer expires, and at least one of the logical channels which belong to an LCG contains UL data, in which case the BSR is referred below to as 'Regular BSR'; - periodicBSR-Timer expires, in which case the BSR is referred below to as 'Periodic BSR'. NOTE 1: When Regular BSR triggering events occur for multiple logical channels simultaneously, each logical channel triggers one separate Regular BSR.

Note that the PDB of UL AR traffic is larger than its periodicity, so when a new data burst arrives, the previous data burst may still exist in the LCH. As shown in Figure 2, when data burst 2 arrives, data burst 1 may still exist in the LCH. In this case, none of the trigger events listed above happens, there will be no BSR for data burst 2. The details are as follows:

- Regular BSR: since the previous data burst may still exist in the LCH, the Regular BSR will not be triggered.

- Periodic BSR: as shown in Figure 2, even if the timer length of periodicBSR-Timer is set to the periodicity and there is no jitter, Periodic BSR still does not work for data burst 2 since as long as there is a Padding BSR triggered between the two data bursts, the periodicBSR-Timer will be restarted.

- Padding BSR: since there are data in the LCH, the Padding BSR may not be triggered.

Figure 2: Example of data burst arrival

In order to let the network know the data volume of a data burst as soon as the data burst arrives, we propose RAN2 to enhance the trigger of BSR, for example, whenever a new data burst arrives. Alternatively, if there is no uplink jitter or the uplink jitter is negligible, the periodicBSR-Timer can be enhanced to avoid being restarted by the other triggered BSRs.

- RAN2 enhances the trigger of BSR, e.g., data burst arrives can be added as a new BSR trigger event, or the running periodicBSR-Timer is not restarted by other triggered BSRs.

  - Granularity of buffer size

According to the current specification [3], the BSR MAC CE includes a 5-bit or 8-bit Buffer Size field. The buffer Size field indicates the data volume by using an index. Table 6.1.3.1-1 and table 6.1.3.1-2 in TS 38.321 specify the mapping between the index and the data volume.

Table 6.1.3.1-1: Buffer size levels (in bytes) for 5-bit Buffer Size field Index; BS value; Index; BS value; Index; BS value; Index; BS value. 0; 0; 8; ≤ 102; 16; ≤ 1446; 24; ≤ 20516. 1; ≤ 10; 9; ≤ 142; 17; ≤ 2014; 25; ≤ 28581. 2; ≤ 14; 10; ≤ 198; 18; ≤ 2806; 26; ≤ 39818. 3; ≤ 20; 11; ≤ 276; 19; ≤ 3909; 27; ≤ 55474. 4; ≤ 28; 12; ≤ 384; 20; ≤ 5446; 28; ≤ 77284. 5; ≤ 38; 13; ≤ 535; 21; ≤ 7587; 29; ≤ 107669. 6; ≤ 53; 14; ≤ 745; 22; ≤ 10570; 30; ≤ 150000. 7; ≤ 74; 15; ≤ 1038; 23; ≤ 14726; 31; > 150000. Table 6.1.3.1-2: Buffer size levels (in bytes) for 8-bit Buffer Size field Index; BS value; Index; BS value; Index; BS value; Index; BS value. 0; 0; 64; ≤ 560; 128; ≤ 31342; 192; ≤ 1754595. 1; ≤ 10; 65; ≤ 597; 129; ≤ 33376; 193; ≤ 1868488. 2; ≤ 11; 66; ≤ 635; 130; ≤ 35543; 194; ≤ 1989774. 3; ≤ 12; 67; ≤ 677; 131; ≤ 37850; 195; ≤ 2118933. 4; ≤ 13; 68; ≤ 720; 132; ≤ 40307; 196; ≤ 2256475. 5; ≤ 14; 69; ≤ 767; 133; ≤ 42923; 197; ≤ 2402946. 6; ≤ 15; 70; ≤ 817; 134; ≤ 45709; 198; ≤ 2558924. 7; ≤ 16; 71; ≤ 870; 135; ≤ 48676; 199; ≤ 2725027. 8; ≤ 17; 72; ≤ 926; 136; ≤ 51836; 200; ≤ 2901912. 9; ≤ 18; 73; ≤ 987; 137; ≤ 55200; 201; ≤ 3090279. 10; ≤ 19; 74; ≤ 1051; 138; ≤ 58784; 202; ≤ 3290873. 11; ≤ 20; 75; ≤ 1119; 139; ≤ 62599; 203; ≤ 3504487. 12; ≤ 22; 76; ≤ 1191; 140; ≤ 66663; 204; ≤ 3731968. 13; ≤ 23; 77; ≤ 1269; 141; ≤ 70990; 205; ≤ 3974215. 14; ≤ 25; 78; ≤ 1351; 142; ≤ 75598; 206; ≤ 4232186. 15; ≤ 26; 79; ≤ 1439; 143; ≤ 80505; 207; ≤ 4506902. 16; ≤ 28; 80; ≤ 1532; 144; ≤ 85730; 208; ≤ 4799451. 17; ≤ 30; 81; ≤ 1631; 145; ≤ 91295; 209; ≤ 5110989. 18; ≤ 32; 82; ≤ 1737; 146; ≤ 97221; 210; ≤ 5442750. 19; ≤ 34; 83; ≤ 1850; 147; ≤ 103532; 211; ≤ 5796046. 20; ≤ 36; 84; ≤ 1970; 148; ≤ 110252; 212; ≤ 6172275. 21; ≤ 38; 85; ≤ 2098; 149; ≤ 117409; 213; ≤ 6572925. 22; ≤ 40; 86; ≤ 2234; 150; ≤ 125030; 214; ≤ 6999582. 23; ≤ 43; 87; ≤ 2379; 151; ≤ 133146; 215; ≤ 7453933. 24; ≤ 46; 88; ≤ 2533; 152; ≤ 141789; 216; ≤ 7937777. 25; ≤ 49; 89; ≤ 2698; 153; ≤ 150992; 217; ≤ 8453028. 26; ≤ 52; 90; ≤ 2873; 154; ≤ 160793; 218; ≤ 9001725. 27; ≤ 55; 91; ≤ 3059; 155; ≤ 171231; 219; ≤ 9586039. 28; ≤ 59; 92; ≤ 3258; 156; ≤ 182345; 220; ≤ 10208280. 29; ≤ 62; 93; ≤ 3469; 157; ≤ 194182; 221; ≤ 10870913. 30; ≤ 66; 94; ≤ 3694; 158; ≤ 206786; 222; ≤ 11576557. 31; ≤ 71; 95; ≤ 3934; 159; ≤ 220209; 223; ≤ 12328006. 32; ≤ 75; 96; ≤ 4189; 160; ≤ 234503; 224; ≤ 13128233. 33; ≤ 80; 97; ≤ 4461; 161; ≤ 249725; 225; ≤ 13980403. 34; ≤ 85; 98; ≤ 4751; 162; ≤ 265935; 226; ≤ 14887889. 35; ≤ 91; 99; ≤ 5059; 163; ≤ 283197; 227; ≤ 15854280. 36; ≤ 97; 100; ≤ 5387; 164; ≤ 301579; 228; ≤ 16883401. 37; ≤ 103; 101; ≤ 5737; 165; ≤ 321155; 229; ≤ 17979324. 38; ≤ 110; 102; ≤ 6109; 166; ≤ 342002; 230; ≤ 19146385. 39; ≤ 117; 103; ≤ 6506; 167; ≤ 364202; 231; ≤ 20389201. 40; ≤ 124; 104; ≤ 6928; 168; ≤ 387842; 232; ≤ 21712690. 41; ≤ 132; 105; ≤ 7378; 169; ≤ 413018; 233; ≤ 23122088. 42; ≤ 141; 106; ≤ 7857; 170; ≤ 439827; 234; ≤ 24622972. 43; ≤ 150; 107; ≤ 8367; 171; ≤ 468377; 235; ≤ 26221280. 44; ≤ 160; 108; ≤ 8910; 172; ≤ 498780; 236; ≤ 27923336. 45; ≤ 170; 109; ≤ 9488; 173; ≤ 531156; 237; ≤ 29735875. 46; ≤ 181; 110; ≤ 10104; 174; ≤ 565634; 238; ≤ 31666069. 47; ≤ 193; 111; ≤ 10760; 175; ≤ 602350; 239; ≤ 33721553. 48; ≤ 205; 112; ≤ 11458; 176; ≤ 641449; 240; ≤ 35910462. 49; ≤ 218; 113; ≤ 12202; 177; ≤ 683087; 241; ≤ 38241455. 50; ≤ 233; 114; ≤ 12994; 178; ≤ 727427; 242; ≤ 40723756. 51; ≤ 248; 115; ≤ 13838; 179; ≤ 774645; 243; ≤ 43367187. 52; ≤ 264; 116; ≤ 14736; 180; ≤ 824928; 244; ≤ 46182206. 53; ≤ 281; 117; ≤ 15692; 181; ≤ 878475; 245; ≤ 49179951. 54; ≤ 299; 118; ≤ 16711; 182; ≤ 935498; 246; ≤ 52372284. 55; ≤ 318; 119; ≤ 17795; 183; ≤ 996222; 247; ≤ 55771835. 56; ≤ 339; 120; ≤ 18951; 184; ≤ 1060888; 248; ≤ 59392055. 57; ≤ 361; 121; ≤ 20181; 185; ≤ 1129752; 249; ≤ 63247269. 58; ≤ 384; 122; ≤ 21491; 186; ≤ 1203085; 250; ≤ 67352729. 59; ≤ 409; 123; ≤ 22885; 187; ≤ 1281179; 251; ≤ 71724679. 60; ≤ 436; 124; ≤ 24371; 188; ≤ 1364342; 252; ≤ 76380419. 61; ≤ 464; 125; ≤ 25953; 189; ≤ 1452903; 253; ≤ 81338368. 62; ≤ 494; 126; ≤ 27638; 190; ≤ 1547213; 254; > 81338368. 63; ≤ 526; 127; ≤ 29431; 191; ≤ 1647644; 255; Reserved.

From the table 6.1.3.1-1 and table 6.1.3.1-2 we can see that the larger the index, the larger the range of data volume indicated by the index. We also can see that the range of data volume indicated by the index for 5-bit Buffer Size field is larger than the range of data volume indicated by the index for 8-bit Buffer Size field.

Considering the data volume of UL AR traffic is large, XR UEs are more likely to use the large indexes. In this case, the network can only know a very rough data volume of each data burst. Furthermore, UE is specified to report a short BSR with 5-bit Buffer Size field when there is only one LCG has data available for transmission, and the deviation will be even large than using the 8-bit Buffer Size field. In consequence, the network cannot accurately schedule resources for the UE. If the network schedules excessive resources for the UE, resources will be wasted and system capacity will be affected. Otherwise, the transmission performance of the UE is affected.

Hence, in order to improve system capacity, we propose RAN2 to refine the granularity of buffer size reporting.

- RAN2 to refine the granularity of buffer size reporting.

3	Conclusions

In this contribution, we discussed the three issues related to BSR for XR-specific capacity improvement. Based on our discussion, we conclude with the following observation and proposals:

- CG can be used for UL VR/Cloud gaming traffic.

- DG can be used for UL AR traffic.

- Enhance the BSR MAC CE to report the data volume of each data burst and its associated remaining PDB.

- RAN2 enhances the trigger of BSR, e.g., data burst arrives can be added as a new BSR trigger event, or the running periodicBSR-Timer is not restarted by other triggered BSRs.

- RAN2 to refine the granularity of buffer size reporting.

4	References

- R2_119-e_meeting_report_v1.

- 3GPP TR 38.838 V17.0.0, Study on XR (Extended Reality) Evaluations for NR (Release 17).

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification".