3GPP TSG-RAN WG2 Meeting #119bis-e	R2-22xxxxx

Electronic, October 10 – 19, 2022

Agenda item:	6.11.2.3

Source: 	Qualcomm Incorporated

Title: 	Summary of [AT119bis-e][416][POS] LPP CR (Qualcomm)

Document for:		Discussion

1.	Introduction

This document summarizes the following email discussion:

[AT119bis-e][416][POS] LPP CR (Qualcomm)

Scope: Merge the agreed LPP changes into a rapporteur CR.

Intended outcome: Agreeable CR

Deadline: Friday 2022-10-14 1000 UTC

1.1	References

[1]	R2-2209430, "Correction to UE capability for DL-AoD"	, Huawei, HiSilicon.

[2]	R2-2209431, "Correction to TEG margin reporting", Huawei, HiSilicon.

[3]	R2-2209434, "Corrections on the timing error margins", CATT.

[4]	R2-2209435, "Change Request of missing UE capabilities", CATT.

[5]	R2-2209436, "Corrections on the LPP capabilities", CATT.

[6]	R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell.

[7]	R2-2210199, "Correction on the maximum number of SRS and TxTEG association", ZTE, Sanechips.

[8]	R2-2210606, "Discussion on the provision of AL for achievable TIR calculation"	, vivo.

[9]	R2-2210784, "Summary of AI 6.11.2.3: LPP corrections", Qualcomm Incorporated.

[10]	(Draft) "Report from session on positioning and sidelink relay", Session Chair (MediaTek).

2.	Discussion

The following agreements were made [10].

Agreements:

Proposal 1:	The CR in 'R2-2209430, "Correction to UE capability for DL-AoD", Huawei, HiSilicon' is an essential correction. Agree a revision of the CR with the editorial issues fixed.

Proposal 3:	The changes related to capability indices 23-3-3, 27-12,  and 27-4-1 in 'R2-2209436, "Corrections on the LPP capabilities", CATT ' are essential corrections. Agree a revision of the CR with the change for 27-20 removed, and with the Note for 27-4-1 removed from DL-AoD.

Proposal 6:	The CR in 'R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell' is an essential correction. Revise the CR using the latest version of the specification.

Proposal 7:	The CR in 'R2-2210199, "Correction on the maximum number of SRS and TxTEG association", ZTE, Sanechips' is an essential correction. Convert the CR into a backwards compatible change by clarifying in an ASN.1 comment that the applicable value is 64. Add the "Isolated Impact" statement to the CR cover sheet.

Details of all proposals to be checked in email discussion [416].

The details of the Proposals are discussed in the following.

Issue #1: Proposal 1 [9]

Proposal 1:	The CR in 'R2-2209430, "Correction to UE capability for DL-AoD", Huawei, HiSilicon' is an essential correction. Agree a revision of the CR with the editorial issues fixed.

According to the email discussion scope, a "Rapporteur CR" with the agreed corrections will be prepared. However, for the CR in R2-2209430 [1], it was proposed to keep this CR separate [10]:

"On P1, Huawei indicate the proposal is BC, but they think there might be a need for a separate CR for visibility due to interoperability considerations."

The interoperability statement on the CR cover sheet [1] indicates the following:

"If the UE is implemented according to the CR while the network is not; or if the network is implemented according to the CR while the UE is not, the UE [added by Moderator] and the network would have different undestanding on the reported UE capabilities nr-DL-PRS-BeamInfoSup-r17 and dl-PRS-ResourcePrioritySubset-Sup-r17."

At previous meeting, we kept only the NBC CRs (ASN and/or functional) separate.

Question 1:	Do you agree that the content of the CR in 'R2-2209430, "Correction to UE capability for DL-AoD", Huawei, HiSilicon' [1] should be kept separate, and not merged into the "Rapporteur CR"?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | The CR is backward compatible in the ASN.1 level, but has larger functionality impacts than the other CRs. It is better to highlight the change in the CR by an independent CR. |

If you have any comments on the content of the CR in 'R2-2209430, "Correction to UE capability for DL-AoD", Huawei, HiSilicon' [1] please provide them in the Table below.

| Company | Comments |
| --- | --- |

Issue #2: Proposal 3 [9]

Proposal 3:	The changes related to capability indices 23-3-3, 27-12,  and 27-4-1 in 'R2-2209436, "Corrections on the LPP capabilities", CATT ' are essential corrections. Agree a revision of the CR with the change for 27-20 removed, and with the Note for 27-4-1 removed from DL-AoD.

On Proposal 3, there were no concerns raised online.

However, given that a "Rapporteur's CR" will be prepared, Proposal 3 is rephrased below.

The content of this CR [5] according to the Proposal 3 above is included in the first version of the LPP CR, which is available in the same folder as this discussion document.

Question 2:	Do you agree with the following:

The changes related to capability indices 23-3-3, 27-12,  and 27-4-1 in 'R2-2209436, "Corrections on the LPP capabilities", CATT' [5] will be merged into the Rapporteur CR but with the additional Note for 27-4-1 removed from DL-AoD.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei,HiSilicon | Yes |  |

Issue #3: Proposal 6 [9]

Proposal 6:	The CR in 'R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell' is an essential correction. Revise the CR using the latest version of the specification.

Concerns were raised that an essential correction may also be needed for Rel-16 [10]:

"On P6, Huawei wonder if there is Rel-16 impact; they are OK with a Rel-17 CR but think we could consider a Rel-16 version next meeting."

Concerns were raised that change #3 [6]:

"(3.)	nr-DL-PRS-RxBeamIndex indication is used for DL-PRS measurements only when additional DL-PRS measurements are also included and all these DL-PRS measurements are associated with a single TRP (up to 8 measurements in Rel-16 or 24 measurements in Rel-17).

(3.)		In 6.5.11.4, NR-DL-AoD-SignalMeasurementInformation, clarify the field description for nr-DL-PRS-RxBeamIndex that it is used for DL-PRS measurements only when additional DL-PRS measurements are also included and all DL-PRS measurements are associated with a single TRP."

is not essential [10]:

"CATT think on P6, “associated with a single TRP” should be deleted in the description, because the IE is already per-TRP.  Nokia think this may be clear from the ASN.1, but an explicit clarification is useful."

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements associated with a single TRP in nr-DL-AoD-MeasList-r16 when additional DL-PRS measurements are also included in either nr-DL-AoD-AdditionalMeasurements-r16 or nr-DL-AoD-AdditionalMeasurementsExt-r17. If the value of the receive beam index for two or more DL-PRS measurements is the same, it indicates that the target device receive beam for the two or more DL-PRS measurements associated with a TRP were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements and/or DL-PRS RSRPP measurements from the same DL-PRS Resource Set associated with a TRP have been made with the same RX beam by the target device; otherwise it is not present.

The content of this CR [6] according to the Proposal 6 above is included in the first version of the LPP CR, which is available in the same folder as this discussion document.

Question 3:	For the CR in R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell' [6] do you agree that for the change #3, the phrase "associated with a (single) TRP" is not needed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | It is useful to clarify that the field is included only when two Rx beams are the same for the reception of the resources from the same TRP. But this has R16 impacts. It is also beneficial to clarify in the R16 version. |

If you have any comments on the content of the CR in ' R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell' [6] please provide them in the Table below.

| Company | Comments |
| --- | --- |

Issue #4: Proposal 7 [9]

Proposal 7:	The CR in 'R2-2210199, "Correction on the maximum number of SRS and TxTEG association", ZTE, Sanechips' is an essential correction. Convert the CR into a backwards compatible change by clarifying in an ASN.1 comment that the applicable value is 64. Add the "Isolated Impact" statement to the CR cover sheet.

Comments were made that the correction may be better made in the field description [10]:

"Lenovo think on P7, the change as proposed is one way to do it, but it could also be in the field description.  Intel think we should take a BC change and are OK with the ASN.1 comment. Huawei agree with Lenovo that the field description would be a better place."

Therefore, there appears to be two options:

Option 1:

–	Multiplicity and type constraint definitions

-- ASN1START

[parts omitted]

maxNumOfRxTEGs-r17						INTEGER ::= 32

maxNumOfRxTEGs-1-r17					INTEGER ::= 31

maxNumOfTxTEGs-1-r17					INTEGER ::= 7

maxTxTEG-Sets-r17						INTEGER ::= 256		-- Maximum applicable number is 64

maxNumOfRxTxTEGs-1-r17					INTEGER ::= 255

maxNumOfTRP-TxTEGs-1-r17				INTEGER ::= 7

maxNumOfSRS-PosResources-r17            INTEGER ::= 64

maxNumOfSRS-PosResources-1-r17          INTEGER ::= 63

[parts omitted]

-- ASN1STOP

Option 2:

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]],

[[

nr-UE-RxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase3

nr-UE-TxTEG-TimingErrorMargin-r17	TEG-TimingErrorMargin-r17		OPTIONAL,-- Cond TEGCase2-3

nr-UE-RxTxTEG-TimingErrorMargin-r17	RxTxTEG-TimingErrorMargin-r17	OPTIONAL -- Cond TEGCase1-2

]]

}

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. - nr-UE-Tx-TEG-ID specifies the ID of this UE Tx TEG. - carrierFreq specifies the frequency of the SRS for positioning resources. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG. For each UE Tx TEG, there may be up to 8 changes (different nr-TimeStamp) of the TEG-SRS association information provided in nr-SRS-TxTEG-Set, i.e., the maximum value for maxTxTEG-Sets is 64.

Question 4:	For the Issue#4 above, which Option do you prefer?

| Company | Option 1/ Option 2/ Both/ None | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option2 |  |

Other

Any other comments (e.g., on the first draft LPP CR) can be provided in the table below.

| Company | Comments |
| --- | --- |

Annex: Agreements [10]

Agreements:

Proposal 1:	The CR in 'R2-2209430, "Correction to UE capability for DL-AoD", Huawei, HiSilicon' is an essential correction. Agree a revision of the CR with the editorial issues fixed.

Proposal 3:	The changes related to capability indices 23-3-3, 27-12,  and 27-4-1 in 'R2-2209436, "Corrections on the LPP capabilities", CATT ' are essential corrections. Agree a revision of the CR with the change for 27-20 removed, and with the Note for 27-4-1 removed from DL-AoD.

Proposal 6:	The CR in 'R2-2209683, "NR-DL-AoD-SignalMeasurementInformation corrections", Nokia, Nokia Shanghai Bell' is an essential correction. Revise the CR using the latest version of the specification.

Proposal 7:	The CR in 'R2-2210199, "Correction on the maximum number of SRS and TxTEG association", ZTE, Sanechips' is an essential correction. Convert the CR into a backwards compatible change by clarifying in an ASN.1 comment that the applicable value is 64. Add the "Isolated Impact" statement to the CR cover sheet.

Details of all proposals to be checked in email discussion [416].

Agreement:

Proposal 2:	The CR in 'R2-2209435, "Change Request of missing UE capabilities", CATT ' is not an essential correction.

Agreement:

RAN2 confirm that for the field nr-UE-RxTEG-TimingErrorMargin in IE NR-DL-TDOA-SignalMeasurementInformation, and for the fields nr-UE-TxTEG-TimingErrorMargin and nr-UE-RxTxTEG-TimingErrorMargin in IE NR-Multi-RTT-SignalMeasurementInformation, absence of the fields indicates the maximum value and the fields do not need to be made mandatory.