3GPP TSG-RAN WG2 Meeting #120	R2-2211183

Toulouse France, November 14~18, 2022

Agenda item:	8.5.4.2

Source:	Qualcomm Incorporated

Title:	Scheduling enhancements for capacity improvement

WID/SID:	FS_NR_XR_enh

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss two scheduling enhancements for capacity improvement: a new type of CG to efficiently support bursty XR traffic flows and a new way to dynamically schedule UL grants in a periodic manner. We also discuss the benefits of dynamically de-/activated measurement gaps, as a means to reduce interruptions caused by measurement gaps and thus improve UE’s throughput and system’s capacity.

# Discussion

## New type of CG configuration for XR

Configured grant (CG) is a good feature for flows which (i) are periodic; (ii) have tight delay requirements. Most flows generated by XR applications meet these two criteria. However, some of XR flows have more complicated traffic patterns than the one(s) that CG is designed for. For example, video streams may consist of periodic burst of PDUs instead of single PDUs. Such new traffic patterns may require enhancements to the legacy CG configuration and procedure, for the following reasons.

First, it is not sufficient to have a single Tx occasion per traffic cycle. It may be possible to use a CG occasion to transmit the first PDU in a burst and then rely on dynamic grants to schedule the rest of the burst. Then PDUs in the later part of the burst may experience more delay. This is not desirable if the bursts consist of PDU sets, because PDUs in the same PDU set need to be sent as closely together as possible.

On the other hand, if a CG is configured to have a very short periodicity, i.e. matching the arrival times of PDUs within a burst, each PDU is guaranteed to have short and similar latency. However, it is very resource expensive to have such a configuration, because transmission occasions between bursts would be wasted.

Observation 1. 	A single CG with legacy configuration is not able to efficiently support XR traffic.

The aforementioned shortcomings with using a single legacy CG may be overcome by using multiple CG configurations. The concern with this approach is that the number of CGs may be required. For example, typically there can be >10 PDUs in a burst for high encoding rates. That means UE needs at least that number of CGs just for video flows, if we don’t consider CGs for other flows such as audio, pose update and control messages. Moreover, if network use multiple sets of such CGs to implement adaptations between different frame rates or transmission parameters, the total number of CGs would further multiplicate, e.g. tens of CGs.

Having too many CGs is too complex for UE to implement. In addition, signaling can also be an issue. For example, if type-1 CG is used, its Tx parameters can only be changed by RRC reconfiguration. Or if type-2 CG is used, RAN1 may have to introduce group activation DCI to support adaptation between two large sets of CGs.

Observation 2. 	If multiple legacy CGs with periodicity matching that of XR traffic are used, a very large number of them (e.g. 10s) may be needed.

One possible enhancement to overcome shortcomings with legacy CG configurations is to introduce a new configuration pattern which matches XR’s traffic pattern. More specifically, in this new configuration,

- There are a cluster of PUSCH occasions in each cycle.

- The periodicity of PUSCH occasions within a cluster can be configured based on the interarrival times of PDUs in a burst or some delay requirements.

- The number of Tx occasion within a cluster, which is up to network configuration, can be comparable to the number of PDUs in a data burst.

- The periodicity between clusters can be configured based on the periodicity of data bursts.

An example of such configuration is illustrated in Figure 1. The advantages of this new configuration are the following:

- It can ensure very low access latency for uplink traffic and yet is still resource efficient as no PUSCH occasions are wasted between bursts.

- Only a single configuration is needed for a flow. For type-2 CGs, legacy de-/activation DCI can be used to enable adaptation of Tx parameters in the same way as legacy.

Multiple PUSCH occasions in a CG period is beneficial to reduce the latency, which can improve the performance. The single CG-PUSCH configuration can define multiple PUSCH occasions has already been supported in R16 NR-U.

Figure 1. An example of enhanced configuration for CG.

Observation 3. 	Multiple PUSCH occasions in a CG period is useful to reduce latency for XR. R16 NR-U already supports multiple PUSCH occasions in a CG period.

Proposal 1.	Multiple PUSCH occasions in a CG period already supported in R16 NR-U can be extended for XR.

## Non-Integer CG Periodicity

R16/17 CG periodicity values mismatch with the periodicity of XR UL traffic arrivals. This implies that there is a tempo mismatch between them. The typical XR traffic periodicities are 60, 120 frames per seconds (fps) or Hz, of which frame periodicities are 16.67ms, 8.33ms. Since CG periodicity values support only integer multiples of slot, no matter which periodicity is chosen from currently available values from TS38.331, it cannot be exactly aligned with UL traffic arrival timing. Figure 3 illustrates the case of XR periodicity mismatch between 60fps XR traffic and CG periodicity of 16 slots and 17 slots in 15kHz SCS. This would lead to XR capacity loss due to the packet delay caused by the timing difference between CG resources and actual XR traffic. In TDD system, UL resource pattern also needs to be considered for CG resources.

Observation 4. 	There is a time mismatch between the periodic XR UL traffic and R16/R17 CG configuration. This will lead to capacity loss due to the packet delay caused by the timing difference between CG resources and actual XR traffic.

Figure 3. Mismatch between XR UL traffic (60fps) and R16/17 CG periodicity (16 slots or 17 slots)

The similar mismatch issue is being under discussion in CDRX enhancements. By adopting the solution of non-integer DRX cycle, the non-integer value for CG periodicity can also resolve the time mismatch between XR traffic and CG resources. Figure 4 illustrates the case of CG resources when the periodicity is set to 50/3ms. Especially for CG Type-2, the non-integer periodicity can activate/deactivate all the CG resources for XR traffic with a single DCI, so the network does not need to set multiple CG configurations with separate DCIs.

Observation 5.	The non-integer periodicity can address the mismatch issue between XR traffic and CG resources.

Observation 6. 	For CG Type-2, the non-integer periodicity can activate/deactivate all the CG resources for XR traffic with a single DCI. However, addressing the mismatch with multiple CG configurations would require separate DCIs.

Figure 4. Enhanced CG periodicity addressing mismatch problem (50/3 slots)

Proposal 2.	RAN2 study options to support non-integer periodicities for CG.

Proposal 3.	RAN2 adopt the same solution to support non-integer periodicities for CG and DRX.

## Periodic DGs

One limitation of CG has always been that it is not very adaptive. Type-1 CGs have to rely on RRC reconfiguration, which is not fast enough to match the timeline of XR traffic (see analysis in [1]). Although type-2 CGs can use activation DCI or multiple configurations to adapt to variations in traffic, it is expected to happen too often. Otherwise, it depletes the original purpose of using CG.

However, XR applications require close match between scheduling and short-term variations in link quality and traffic load. That is necessary in order to guarantee XR’s stringent QoS requirements in a resource efficient way (i.e. maximize system capacity).

Observation 7. 	XR applications require close adaptation between scheduling and short-term variations in link quality and traffic load. But legacy CG is not flexible enough for such adaptations.

One enhancement is to combine the best of DG and CG in a single configuration, i.e.

- With this enhancement, network periodically provides dynamic grants to UE. This deterministic availability of PUSCH resources ensures short access latency, just as legacy CGs do. The pattern of Tx occasions can be either the same as legacy (i.e. one occasion per cycle) or the same as the one proposed in Section 2.1 to best match traffic pattern of a XR flow.

- However, for each individual Tx occasion, PUSCH transmission is dynamically scheduled by a DCI. This enables network to schedule each TB with the best Tx parameters possible, e.g. based on the most recent link state feedback and buffer status report, etc.

- UE can apply the legacy UL skipping procedure to skip any occasions in which it does not have data. On network side, network can use either PDCCH skipping or cancelation DCI to cancel any Tx occasions that it does not want to schedule. As a result, no resources are wasted.

Proposal 4.	Introduce an enhancement in which UE is pre-configured with a sequence of periodic PUSCH occasions but each occasion is dynamically scheduled by DCI.

## Enhanced measurement gaps

In NR, measurement gaps (MG) can be configured for UE to perform inter-frequency or inter-RAT measurements, for the purpose of mobility, positioning, or SFTD. If a MG occasion overlaps with PDCCH/PDSCH/PUSCH reception or transmission, it always has higher priority except Msg2/3/4/A/B in RACH procedures. For this reason, MGs have negative impacts on performance because they interrupt normal data transfers.

We observe that due to non-integer valued periodicities of XR traffic, it is more likely for data to overlap with MGs, which have integer-valued periods such as 20ms, 40ms, etc. That is because it is impossible for network to align MGs in a way that they do not overlap with data bursts. Figure 2 illustrates such an example. In this example, UE is configured with MGs with a periodicity of 20ms while XR bursts arrive at 60 frame per second. As XR data bursts have a shorter periodicity than MGs, they always “catch up” with MGs and then collide into one.

Whenever data transfer is interrupted by a MG, extra delay is added. Given that common duration of a MG is several msec (e.g. 6msec) and XR flows typically have end-to-end delay requirement of 10~20ms, each interruption by MG reduce the delay budget of XR traffic by at least 30%. That is very undesirable, because it can considerably reduce system capacity.

Observation 8. 	Due to non-integer valued periodicities, XR traffic is more likely to overlap with measurement gaps, which can increase delay and reduce system capacity.

Figure 2. XR DL traffic interrupted by measurement gaps

In addition, short periodicities of XR traffic require use of short DRX inactivity timers (e.g. <10ms). That implies that it becomes more likely for DRX inactivity timer to expire in the middle of a MG. That will also cause extra delay if UE still has data to send or receive. This issue is specific to XR, because in legacy DRX inactivity timer typically is much longer than duration of a MG and hence would not expire in the middle of a MG.

Observation 9.	With shorter DRX inactivity timer required for XR, it is more likely for DRX inactivity timer to expire in the middle of a measurement gap, which creates extra delays to data.

As delay requirements and system capacity are two of the most important performance objectives for studies for XR, we think it is necessary for RAN2 to study enhancements that can minimize the aforementioned impacts by MGs. We think several options are possible, for example:

- Introduce signalling enhancements that allows network to dynamically de-/activate a MG configuration. Network can decide whether to temporarily deactivate a MG based on relative importance between ongoing data and a specific MG occasion(s).

- Introduce criteria for dynamic priority between data and MG. For example, if UE meets the R17 RRM measurement relaxation criteria and network permits, then delay-critical data is allowed to have higher priority than MG.

Proposal 5. 	RAN2 study enhancements (e.g. dynamic de-/activation or dynamic priority) that can mitigate impacts of measurement gaps on delay performance of XR traffic.

# Conclusion

Based on the above analysis, we’d recommend RAN2 to discuss and adopt the following proposals:

New type of CG configuration

Observation 1. 	A single CG with legacy configuration is not able to efficiently support XR traffic.

Observation 2. 	If multiple legacy CGs with periodicity matching that of XR traffic are used, a very large number of them (e.g. 10s) may be needed.

Observation 3. 	Multiple PUSCH occasions in a CG period is useful to reduce latency for XR. R16 NR-U already supports multiple PUSCH occasions in a CG period.

Proposal 1.	Introduce a new type of CG configuration which can have multiple transmission occasions within one cycle and the cycle length matches that of XR traffic.

Non-Integer CG Periodicity

Observation 4. 	There is a time mismatch between the periodic XR UL traffic and R16/R17 CG configuration. This will lead to capacity loss due to the packet delay caused by the timing difference between CG resources and actual XR traffic.

Observation 5. 	The non-integer periodicity can address the mismatch issue between XR traffic and CG resources.

Observation 6. 	For CG Type-2, the non-integer periodicity can activate/deactivate all the CG resources for XR traffic with a single DCI. However, addressing the mismatch with multiple CG configurations would require separate DCIs.

Proposal 2.	RAN2 study options to support non-integer periodicities for CG.

Proposal 3.	RAN2 adopt the same solution to support non-integer periodicities for CG and DRX.

Periodic DGs

Observation 7. 	XR applications require close adaptation between scheduling and short-term variations in link quality and traffic load. But legacy CG is not flexible enough for such adaptations.

Proposal 4.	Introduce an enhancement in which UE is pre-configured with a sequence of periodic PUSCH occasions but each occasion is dynamically scheduled by DCI.

Enhanced measurement gaps

Observation 8. 	Due to non-integer valued periodicities, XR traffic is more likely to overlap with measurement gaps, which can increase delay and reduce system capacity.

Observation 9.	With short DRX inactivity timer required for XR, it is more likely for DRX inactivity timer to expire in the middle of a measurement gap, which creates extra delays to data.

Proposal 5. 	RAN2 study enhancements (e.g. dynamic de-/activation or dynamic priority) that can mitigate impacts of measurement gaps on delay performance of XR traffic.