3GPP TSG-RAN WG2 Meeting #120	                R2-2211201

# Toulouse, France, November 14-18, 2022

Agenda Item:	8.4.1

Source: 	CATT, Fujitsu

Title: 	Discussion on RAN1 LS on measurement and configurations for L1L2-based inter-cell mobility

Document for:	Discussion

1. Introduction

A RAN1 LS [1] on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility was sent to RAN2. In the LS, RAN1 provides the latest RAN1 agreements and also ask questions to other WGs.

In this contribution, we discuss the question related to RAN2 and provide our view on RAN2 feedback to RAN1.

2. Discussion

In RAN LS [1], a question (i.e. question 3) on L1 measurement and TCI state configurations was raised to RAN2 and RAN3.

- L1 measurement and TCI state configurations RAN1 has started the discussion on the configuration for L1 measurement and TCI states for candidate cells. Regarding the following RAN2 agreements captured in RAN2 LS (R1-2208331/R2-2209257), it is not clear for RAN1 which kind of information/configuration for candidate cell(s) are available at a serving cell for inter-DU case for Rel-18 L1/L2 mobility. Thus, companies have different understanding on the implication of the sentence “as much commonality as reasonable” in the LS. - The design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable. FFS which aspects need to be different. Question 3 (to RAN2 and RAN3): RAN1 respectfully asks RAN2 and RAN3 if the serving DU knows the measurement RS configuration and TCI state configuration of cells served by another DU.

From UE point of view, if there are inter-DU candidate cells for LTM, UE is supposed to be configured with the related measurement RS configuration and TCI state configuration of candidate cells served by another DU, i.e. same as that for intra-DU case. This principle aligns well with the previous RAN2 agreement that the design for intra-DU and inter-DU L1/L2-based mobility should share as much commonality as reasonable.

From serving DU point of view, if there are inter-DU candidate cells for LTM, the measurement RS configuration and TCI state configuration of these candidate cells served by another DU are useful to serving DU, i.e., serving DU needs to handle the L1 measurements report of candidate cells from UE (i.e. based on the measurement RS configuration of the candidate cell), and during the HO decision, serving DU selects the target cell from the candidate cells and may also select a beam of the selected target cell(i.e. based on the TCI state configuration of candidate cells).

From network interface point of view, it is up to RAN3 on whether it is feasible for serving DU to know the measurement RS configuration and TCI state configuration of cells served by another DU.

Therefore, it is proposed,

Proposal 1: Reply to RAN1 that “If there are inter-DU candidate cells for LTM, RAN2 assumes serving DU can know the measurement RS configuration and TCI state configuration of these candidate cells served by another DU”.

3. Conclusion

In this contribution, we provide our analyses on the question in RAN1 LS, and proposal is as follows:

Proposal 1: Reply to RAN1 that “If there are inter-DU candidate cells for LTM, RAN2 assumes serving DU can know the measurement RS configuration and TCI state configuration of these candidate cells served by another DU”.

4. Reference

- R2-2211154, LS on L1 intra- and inter- frequency measurement and configurations for L1/L2-based inter-cell mobility (R1-2210727).