3GPP TSG-RAN WG2 Meeting #120                                                                 R2-2211202

Toulouse, France, Nov 14th- Nov 18th, 2022

Source:	CATT

Title:	On Procedure Description

Agenda Item:	8.4.2.1.2

Document for:	Discussion and Decision

Introduction

This contribution will focus on the discussion of the signalling procedure of LTM, and gives the suggestion on other groups.

Discussion

2.1 Discussion on overview signaling procedure

Based on current discussion, one possible overview signaling is as following.

Figure 1signalling procedure for Uu interface

In general we can divide the full procedure into 4 separate parts as following:

- LTM preparation, initiation of the LTM procedure, prepare the LTM candidate cell configuration from NW side

- LTM configuration, provide the LTM configuration to the UE which includes the candidate cell configuration, and may also include the L1 measurement configuration.

- LTM decision, NW make the decision on the target cell

- LTM execution, UE perform the access to the indicated target cell, and possible indication to NW

Proposal 1: The baseline LTM procedure should include the following four parts: LTM preparation, LTM configuration, LTM decision, LTM execution.

The further discussion will be carried for each part.

- LTM preparation

For the LTM preparation part, the gNB should prepare the LTM candidate cell for the UE, the preparation procedure mainly focus on inter-node message which is RAN3 scope.

For CU/DU split case, from RAN2 perspective, it should be the CU initiates the LTM preparation procedure. The CU could determine the initiation of LTM preparation based on the RRM measurement results or based on the deployment. And the CU could provide T-DU the suggested candidate cells to T-DU. These issues also have been discussed in RAN3 and following agreements were made in RAN3:

During L1/L2 handover configuration, the gNB-CU sends the suggested candidate cell(s) to the gNB-DU in UE Context Modification Request procedure, FFS in one message or multiple messages. The gNB-DU may accept the target cells of L1/L2 handover and responds to the gNB-CU with the access control result in UE Context Modification Response message(s). gNB-DU may accept all or part of the target candidate cells. gNB-DU initiated L1/L2 handover configuration is not allowed.

Observation 1: RAN3 have agreed CU to initiate the LTM preparation procedure and CU provide suggested candidate cells to T-DU. T-DU may accept all or part of the target candidate cells, and respond to CU.

- LTM configuration

For the LTM configuration part, source cell will sends the LTM configuration to the UE via RRC message, and the UE will respond the RRC complete message to the gNB. It seems the RRC reconfiguration message is suitable to encapsulate the LTM candidate cell configuration.

Proposal 2: RRC reconfiguration message is used to configure the LTM configuration for the UE, which including at least one LTM candidate cell configuration.

Proposal 3: Upon reception of the RRC reconfiguration message including LTM configuration, UE sends the RRC reconfiguration complete message to respond the LTM configuration.

As for the contents and signaling format of the LTM configuration, it will be discussed in other agenda in RAN2; this contribution will not further discuss the details.

As discussed above, the LTM configuration for UE is generated as RRC signaling, for CU-DU split gNB, it is obvious it is the CU to generate the RRC reconfiguration including LTM configuration. CU sends the RRC reconfiguration message to UE via S-DU. And upon reception of the RRC reconfiguration including LTM configuration UE responds with RRC reconfiguration complete message to CU via S-DU. It seems legacy procedure can be reused.

Observation 2: Legacy procedure between CU and DU can be reused to transmit the RRC reconfiguration including LTM configuration for CU-DU split gNB.

- LTM decision

RAN2 has assumed that we rely on L1 measurements to trigger L1L2 mobility. As for what to measure and report is dependent on RAN1 decision. And considering more than one candidate cells are configured for UE, it is possible to support dynamic L1 measurement and report, i.e. the NW may indicate the UE which candidate cell should be measured and reported. But it also depends on RAN1 decision. The TA acquisition procedure is under RAN1 discussion, RAN1 will determine the applicable scenario and how to pre-acquire the TA for target cell.

Observation 3: The following issues are RAN1 scope, which help the NW to make the LTM execution decision:

- L1 measurement and report

- Whether dynamic L1 measurement and report is supported

- TA pre-acquisition method and applicable scenarios

Even though how to acquire the TA is under RAN1 discussion which is not belongs to RAN2 scope. But RAN2 could discuss the trigger node and possible coordination between CU-DU. From RAN2 perspective, the procedure is better to be initiated by NW instead of UE autonomously. Considering more than on candidate cells can be configured for the UE, it is not necessary to acquire TA for all candidates, the NW could determine when and which candidate cell that the UE to acquire the TA based on the L1 measurement results. For CU-DU split gNB, the TA acquisition should be initiated by S-DU.

Proposal 4: From RAN2 perspective, the TA pre-acquisition is initiated by gNB. gNB or S-DU sends command to indicate the UE to perform TA acquisition for indicated candidate cell, which RAN1 could take into consideration.

Considering the TA procedure needs the interaction between the UE and the T-cell, for inter-DU case, the coordinate between the S-DU and T-DU is needed, e.g. when S-DU determine to trigger UE to perform TA acquisition procedure for one candidate cell in another DU, the S-DU should indicate the T-DU to perform TA measurement procedure, meanwhile, the T-DU may provide the related resource for TA acquisition to UE via S-DU. But detail for the content of the TA acquisition coordination between S-DU and T-DU depends on the TA acquisition method adopted by RAN1.

Proposal 5: For inter-DU case, coordination between S-DU and T-DU may need for following issues, which RAN3/RAN1 could take into consideration.

- the trigger of the TA acquisition for indicated candidate cell

- resource for TA acquisition for indicated candidate cell

As for how to provide the TA of the candidate cell to UE, it could be postponed until RAN1 make further discussion on the TA acquisition mechanism.

Proposal 6: Discussion on provision the TA of the candidate cell to UE could be postponed until RAN1 make further discussion on the TA acquisition mechanism.

- LTM execution

NW will send LTM command which is one MAC CE or MAC CE+DCI to UE to inform the UE perform the LTM execution. The LTM command was discussed last meeting in RAN2, and the following agreements about the LTM command were made:

RAN2 assumes L1/2 mobility trigger information is conveyed in a MAC CE, FFS if the MAC CE or a DCI is used for the actual triggering. RAN2 assumes the MAC CE for L1/2 mobility trigger contains at least a candidate configuration index. RAN2 assumes RACH resource for CFRA for L1 L2 dynamic switch may be provided in RRC configuration (or potentially by MAC CE FFS). FFS if the MAC CE can indicate TCI state(s) (or other beam info) to activate for the target Cell(s), dep on RAN1 progress.

Considering the topic of details of LTM command (e.g. LTM command format MAC CE or MAC CE+DCI, content of the LTM command) will be discussed in 8.4.2.3, so this contribution will not further discuss this topic.

Upon reception of the LTM command, the UE will trigger the LTM execution procedure and access to the target cell. Due to the TA for the target cell may be acquired before/with the LTM command, so RA procedure could be skipped for this case. And it was agreed in last meeting “both RACH-based (CFRA, CBRA) and RACH-less procedures for L1 L2 mobility switch may be supported.” So whether step 6 is executed could depend on whether TA is acquired.

Observation 4: Upon reception of LTM command, UE may skip the RA procedure when the TA of the target cell is acquired.

After complete the sync with the target cell, UE needs to inform the target cell the completion of the cell switch. It could be one explicit signaling sent to the target cell which is better to be one L1L2 indication. Another way is implicitly inform the target cell, e.g. one UL transmission scramble by C-RNTI of the UE. RAN2 should further discuss how to indicate target cell the completion of the LTM cell switch.

Proposal 7: During the LTM execution, UE should indicate the target cell the completion of the LTM cell switch, RAN2 further discuss the following option:

- Explicit way, e.g. explicit L1/L2 indication sent to target cell

- Implicit way, e.g. based on UL transmission scramble by C-RNTI of the UE

For CU/DU split case, the decision is made by S-DU. Upon S-DU make the decision on LTM execution, for inter-DU case, the T-DU may need to prepare for UE access, especially for RACH-Less case, the T-DU needs to prepare to receive UE transmission or schedule for UE transmission. Hence take these into consideration, S-DU needs to inform T-DU upon S-DU determine to trigger the LTM execution for inter-DU case.

Proposal 8: For inter-DU case, S-DU needs to inform T-DU the trigger of the LTM execution on target cell, upon S-DU make the decision on LTM execution, which RAN3 could take into consideration.

For CU/DU split case, upon LTM execution, S-DU may send the Downlink Data Delivery Status to inform the CU about the unsuccessfully transmitted downlink data to the UE, which is legacy procedure for intra-DU mobility. But this step is also should be taken into consideration by RAN3.

Proposal 9: For inter-DU case, upon LTM execution, S-DU may send the Downlink Data Delivery Status to inform the CU about the unsuccessfully transmitted downlink data to the UE, which RAN3 could take into consideration.

As discussed above, UE should inform target cell the completion of the LTM, upon get the indication from UE whether T-DU should indicate CU the completion of the LTM has discussed on RAN3, and WA was reached as following:

WA: The gNB-DU indicates the gNB-CU about the UE successful access to the target cell by Access Success message.

Considering some CU configuration should be applied after LTM execution e.g. RRM measurement, no matter it is intra-DU or inter-DU case, so the CU should be informed after LTM execution, so from RAN2 perspective, the WA made by RAN3 can be confirmed.

Proposal 10: From RAN2 perspective, DU shall indicate CU about the UE successful access to the target cell, RAN3 WA could be confirmed.

2.2 Description on signaling procedure for CU/DU split case

Combine the above discussion, we propose one possible signaling procedure taking CU/DU into consideration, i.e. take the inter-node message into consideration. Considering the intra-DU case is a special case of inter-DU HO, so we first take the inter-DU cell switch as example, a feasible signaling procedure is as following:

Figure 2 signaling procedure for inter-DU LTM

Step 0: UE report the RRM measurement report to the CU via S-DU, CU makes the decision on triggering LTM preparation.

Step 1-2: CU initiates the LTM preparation procedure to the candidate target DU (T-DU), in the LTM request message, the CU may include the suggested candidate Cell list to T-DU to assistant the T-DU determine the candidate cell. T-DU may accept the request or reject it. And the T-DU respond with LTM request ACK message, the DU will provide the configuration for the accepted candidate cells. The decision on the determined candidate cell should leave to the T-DU. The T-DU may accept all or part of the suggested candidate cells.

Step 3-4: CU generates a RRC reconfiguration message including the LTM configuration and sends it to UE via S-DU.

Step 5: UE sends the RRC reconfiguration complete message to the CU via S-DU to respond the LTM configuration.

Step 6: Based on the configuration, UE will perform the L1 measurement and report the L1 measurement results for LTM candidate cells. FFS on whether dynamic L1 measurement and report is supported

Step 7: TA acquisition may be performed by UE for the LTM candidates, coordination between S-DU and T-DU may needed, S-DU sends the indication to T-DU to indicate the trigger on TA acquisition for target cell, and the T-DU may respond with S-DU with the ACK message which may include the TA acquisition resource. S-DU sends the TA acquisition trigger command to UE, which may include the related resource, UE and the T-cell perform the TA acquisition procedure. FFS on how to perform TA acquisition procedure between UE and target cell;  FFS on how to inform UE the TA for the target cell.

Step 8: based on the L1 measurement report and/or the TA, the S-DU could make the LTM execution decision. And generates the LTM command (FFS on MAC CE or MAC CE and DCI) and sends the LTM command to the UE, and indicates the T-DU via CU.

Step 9: if needed, The S-DU also sends a Downlink Data Delivery Status frame to inform the CU about the unsuccessfully transmitted downlink data to the UE.

Step 10: if needed, UE will trigger the random access procedure to the target cell to complete the sync to the target cell.

Step 11: UE sends the LTM complete indication to the target DU.

Step 12: T-DU sends the LTM complete indication to the CU.

Proposal 11: Take the figure 2 and the procedure above as baseline for inter-DU LTM procedure.

For intra-DU case, the coordination between S-DU and T-DU could be omitted, so one possible signaling procedure is as following:

Figure 3signalling procedure for intra-DU LTM

Step 0: UE report the RRM measurement report to the CU, CU makes the decision on triggering LTM preparation.

Step 1-2: CU initiates the LTM preparation procedure to the DU, in the LTM request message, the CU may include the suggested candidate Cell list to DU to assistant the DU determine the candidate cell. DU may accept the request or reject it. And the DU respond with LTM request ACK message, the DU will provide the configuration for the accepted candidate cells. The decision on the determined candidate cell should leave to the DU. The DU may accept all or part of the suggested candidate cells.

Step 3-4: CU generates a RRC reconfiguration message including the LTM configuration and sends it to UE via DU.

Step 5: UE sends the RRC reconfiguration complete message to the CU via DU to respond the LTM configuration.

Step 6: Based on the configuration, UE will perform the L1 measurement and report the L1 measurement results for LTM candidate cells. FFS on whether dynamic L1 measurement and report is supported

Step 7: TA acquisition may be performed by UE for the LTM candidates, DU sends the TA acquisition trigger command to UE, which may include the related resource, UE and the T-cell perform the TA acquisition procedure. FFS on how to perform TA acquisition procedure between UE and target cell;  FFS on how to inform UE the TA for the target cell.

Step 8: based on the L1 measurement report and/or the TA, the DU could make the LTM execution decision. And generates the LTM command (FFS on MAC CE or MAC CE and DCI) and sends the LTM command to the UE.

Step 9: if needed, UE will trigger the random access procedure to the target cell to complete the sync to the target cell.

Step 10: UE sends the LTM complete indication to the target cell.

Step 11: DU sends the LTM complete indication to CU.

Proposal 12: Take the figure 3 and the procedure above as baseline for intra-DU LTM procedure.

Conclusion

Based on the previous analysis in section 2, our main contributions are summarized as follows:

Observation 1: RAN3 have agreed CU to initiate the LTM preparation procedure and CU provide suggested candidate cells to T-DU. T-DU may accept all or part of the target candidate cells, and respond to CU.

Observation 2: Legacy procedure between CU and DU can be reused to transmit the RRC reconfiguration including LTM configuration for CU-DU split gNB.

Observation 3: The following issues are RAN1 scope, which help the NW to make the LTM execution decision:

- L1 measurement and report

- Whether dynamic L1 measurement and report is supported

- TA pre-acquisition method and applicable scenarios

Observation 4: Upon reception of LTM command, UE may skip the RA procedure when the TA of the target cell is acquired.

Proposal 1: The baseline LTM procedure should include the following four parts: LTM preparation, LTM configuration, LTM decision, LTM execution.

Proposal 2: RRC reconfiguration message is used to configure the LTM configuration for the UE, which including at least one LTM candidate cell configuration.

Proposal 3: Upon reception of the RRC reconfiguration message including LTM configuration, UE sends the RRC reconfiguration complete message to respond the LTM configuration.

Proposal 4: From RAN2 perspective, the TA pre-acquisition is initiated by gNB. gNB or S-DU sends command to indicate the UE to perform TA acquisition for indicated candidate cell, which RAN1 could take into consideration.

Proposal 5: For inter-DU case, coordination between S-DU and T-DU may need for following issues, which RAN3/RAN1 could take into consideration.

- the trigger of the TA acquisition for indicated candidate cell

- resource for TA acquisition for indicated candidate cell

Proposal 6: Discussion on provision the TA of the candidate cell to UE could be postponed until RAN1 make further discussion on the TA acquisition mechanism.

Proposal 7: During the LTM execution, UE should indicate the target cell the completion of the LTM cell switch, RAN2 further discuss the following option:

- Explicit way, e.g. explicit L1/L2 indication sent to target cell

- Implicit way, e.g. based on UL transmission scramble by C-RNTI of the UE

Proposal 8: For inter-DU case, S-DU needs to inform T-DU the trigger of the LTM execution on target cell, upon S-DU make the decision on LTM execution, which RAN3 could take into consideration.

Proposal 9: For inter-DU case, upon LTM execution, S-DU may send the Downlink Data Delivery Status to inform the CU about the unsuccessfully transmitted downlink data to the UE, which RAN3 could take into consideration.

Proposal 10: From RAN2 perspective, DU shall indicate CU about the UE successful access to the target cell, RAN3 WA could be confirmed.

Proposal 11: Take the figure 2 and the procedure above as baseline for inter-DU LTM procedure.

Proposal 12: Take the figure 3 and the procedure above as baseline for intra-DU LTM procedure.

Reference

[1] Draft_R2_119bis-e_meeting_report_v1