3GPP TSG-RAN WG2 Meeting #120                                                              R2-2211205

Toulouse, Nov 14th – Nov 18th, 2022

Source:	CATT

Title:	Discussion on Selective Activation of Cell Groups in NR-DC

Agenda Item:	8.4.3

Document for:	Discussion and Decision

Introduction

In RAN2#119bis-e meeting, NR-DC with selective activation cell of groups related issues were discussed and the following agreements were achieved [1].

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - a. Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. - c. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell. Confirm that “CPA” selective activation of cell groups will be supported for this WI objective Confirm that we aim to support delta configuration, i.e. that there need to be a known reference. RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

In this contribution, in section 2, we continue to discuss scenarios and basic solutions related to the selective activation of cell groups in NR-DC. The proposals are summarized in Section 3.

Discussion

Scenarios for selective activation of cell groups

MN-initiated CPC/ SN-initiated CPC

In RAN2#119e meeting, we had reached a consensus to support of subsequent CPC after normal PSCell change or CPC execution. Furthermore, we need to discuss which CPC type of the legacy CPC cases should be considered in R18 i.e. which node could initiate the procedure.

The intention of this objective is to allow the UE perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network. From this point view, the scenario is applicable for MN kept with SCG change. So the CPC/CPA configuration kept by UE should correspond to the same MN. It is obvious that the candidate SNs of the stored CPC and CPA should be the neighbor cells that have overlapping coverage with the MN. Distinctly, it is MN instead of SN who is the suitable node to make the decision to choose the candidate SNs. Due to that the MN have the neighbouring cells information of MN based measurement report according to the MN measConfig or deployment, but the SN only have the neighbouring cells information of SN. It is not ensured that the candidate cells selected by source SN can be under the coverage of MN i.e. could be configured with the MN as DC.

Figure 1 MN-initiated CPC and SN- initiated CPC

In the figure above, candidate SN 1 ~ SN4 and source SN are the candidate SN/PSCell which are selected by MN based on MN RRM measurement results. Candidate SN5~SN7 are the candidate SN/PSCell which are selected by SN based on SN RRM measurement results. Assuming the UE is configured with candidate SN 5~7, if the UE performs SCG change from source SN to candidate SN 6, it is obvious that the SN6 is out of the coverage of MN which is not applicable for the second cell group selective for the source MN. Therefore, the candidate PSCells selected by source SN become useless as before or after UE perform SCG change to the SN5~7, UE may trigger the MN change procedure.

Observation 1: It is more suitable for MN to decide the candidate SNs for the subsequent SCG changes since SN does not have the information of the neighbour cells which may have the overlapping coverage with MN.

Besides, taking the execution condition update into consideration, Only MN initiated CPC is feasible if no enhancement is applied to the legacy execution condition. For MN-initiated inter-SN CPC, candidate PSCell and execution conditions are selected and generated by MN based on MN configured measConfig. The execution conditions of candidate PSCell is still applicable for using in subsequent SCG change unless MN updates the MN measConfig.

However, for SN-initiated inter-SN CPC, candidate PSCell and execution conditions are generated by SN based on SN configured measConfig. After the first PSCell change, the configured execution conditions of candidate PSCell will not be valid anymore as the measurement configuration in the new serving PSCell is different from that in old one. So the candidate PSCell execution conditions need to be updated after each SCG change. The update of execution condition after each SCG change eliminates the benefit of supporting selective activation of SCGs (i.e. to allow subsequent cell group change after changing CG without reconfiguration).

Observation 2: SN-initiated inter-SN CPC will introduce complexity as it requires execution condition update after each subsequent PSCell change.

Based on the above analysis, for the objective of selective activation of cell groups, MN-initiated CPC should be taken into consideration with high priority. For SN-initiated CPC, it could be considered after the discussion of MN-initiated CPC (if time allowed).

Proposal 1: For the cases of CPC, support MN-initiated CPC with high priority.

How to trigger the subsequent cell group change

During the WID discussion of RAN meeting, it seems that companies have different understanding on how to trigger the subsequent cell group change. Some companies think it is triggered by UE, based on pre-configured CPAC execution conditions (i.e. UE triggered solution), Some other companies think that the SCG change is performed based on NW command (i.e. NW triggered solution), i.e. the network configures the candidate cell groups configuration to UE in advance, the network makes the decision of SCG change and sends the SCG change command to inform the UE to perform the SCG change procedure.

In RAN2#119e meeting, it was agreed that supporting of subsequent conditional changes (CPC) after a cell group change (normal or conditional). It seems to imply that UE triggered solution is supported, i.e. SCG change is triggered by UE based on execution condition(s) evaluation. However, it is necessary to confirm that we should focus on the UE triggered solution.

Proposal 2: Focus on UE triggered solution (i.e. selective activation of cell groups is triggered when the execution condition(s) of at least one candidate PSCell is fulfilled).

Term for selective activation of cell groups

During the discussion in previous meeting, the term of CPA/CPC was used for selective activation of cell groups. However, using the term “CPA/CPC” in R18 will introduce some troubles, e.g. we need to distinguish whether the configuration is CPA or CPC configuration which depends on whether the UE had been configured a SCG before sending the conditional reconfiguration. Further, the CPA configuration will change to CPC configuration after CPA execution or legacy SCG addition. And also we may need to discuss various cases for CPA and CPC. For selective activation of cell groups, it could be simply understood as the network can configure multiple candidate target PSCell(s) configuration with execution conditions to UE for both case of standalone and DC, and the UE will not release the multiple candidate target PSCell(s) configuration after successful PSCell addition (from standalone to DC) or PSCell change (from DC to DC).So, to facilitate the discussion, we think it is better to use a more general term(e.g. conditional selective cell groups(CSCG) which is similar as “LTM” used for “L1/L2-triggered mobility” to distinguish from CPAC.

Therefore, it is proposed,

Proposal 3: For selective activation of cell groups, the term “conditional selective cell groups (CSCG)” is used to distinguish from legacy CPAC.

Security issue

In last meeting, security related issue was discussed. It was concluded that RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security.

RAN2 aim to support selective activation of cell groups without RRC reconfiguration with respect to security (FFS, need to consult with SA3 at some point in time).

We understand that as a general principle it is natural that the security key should be updated when UE accesses a new SCG. The key issue is whether the security key should be updated when UE accesses a previous cell from other SN, which be mentioned in RAN2#119.

If security key are re-used when UE goes back to a previous cell from other SN, it seems to be simpler but may cause security issue, and it may introduce complexity for NW implementation to avoid security issue. Considering the input parameters for encryption algorithm (e.g. PDCP counter, S-KgNB etc.) could be same for the 2 times UE access the same SN. Furthermore the SN of PDCP is not consecutive due to the RB is used for data transmission in other SN. In order to follow the principle of avoiding reuse of the COUNT with the same RB identity (COUNT wrap around), NW may need more complex implementation. Therefore, to avoid security issue, sk-counter should not be reused when UE goes back to a previous cell from other SN.

Proposal 4: The sk-counter cannot be reused when UE goes back to a previous cell from other SN.

Based on the discussion above, no matter whether the target PSCell is a new PSCell or a previous PSCell from other SN, the security key should be updated. In order to support selective activation of cell groups without RRC reconfiguration with respect to security, new security key update mechanism should be introduced.

We can focus on the discussion of the following security key update mechanism during this stage:

- Option 1: NW provides a common sk-counter for all candidate target SCGs to UE, the sk-counter will be incremented by specified step after each SCG change;

- Option 2:  NW provides a list of sk-counter for each candidate target SCGs to UE, the sk-counter in the list will be used after each SCG change to the candidate target SCG(no repeat);

- Option3: Introduce new SN security key derivation method.

Among the three options, Option 1 provides a simpler solution, the UE only keeps one common sk-counter for all candidate target SCGs, and the UE will increment sk-counter autonomously while each SCG change based on a fixed rule (e.g. add by specified step autonomously e.g. plus one each time). Similarly, the network also increment sk-counter while each SCG change based on the same rule. For Option 2, the key question is the sk-counter may not be incrementally no matter which SCG the UE selective to access which needs to be further checked with SA3 whether it is allowed or not. For option3, more complexity and spec impacts will be introduced which is not what we expected. Therefore, we prefer Option 1.

Proposal 5: For security key update, NW provides a common sk-counter for all candidate target SCGs to UE, the sk-counter will be incremented after each SCG change.

Configuration for selective activation of cell groups

Delta configuration

In last meeting, delta configuration related issue was discussed and concluded that we aim to support delta configuration, i.e. that there need to be a known reference.

Confirm that we aim to support delta configuration, i.e. that there need to be a known reference.

However, how to determine the known reference configuration has not decided yet. Obviously, the reference configuration should be determined and configured by the network. The network can configure the reference configuration independent of the candidate SCG configurations, all the candidate SCG configurations are configured based on the reference configuration following the delta configuration principle. Alternatively, the network also can take one of the candidate SCG configurations as reference configuration, other candidate SCG configurations will be configured based on the reference candidate configuration, for example, take the first candidate SCG configuration in the candidate SCG configuration list as reference configuration and indicate it to UE for getting the full configuration of each candidate SCG. The two options are listed below.

- Option1: Network configures a reference configuration independent of the candidate SCG configurations;

- Option2: Network takes one of the candidate SCG configurations as reference configuration and indicates it to UE.

For option 1, it needs extra signalling to send the reference configuration to UE, the UE always needs to keep the reference configuration during the procedure of selective activation of SCG, which will introduce extra signalling overhead and UE storage. But it is independent from the candidate configuration, any candidate SCG configuration update will not impact other candidate SCG configuration. For option2, it could save the signalling overhead, due to the reference configuration is also one of the candidate configurations. But if NW updates the candidate configuration which is specified as reference configuration, other candidate configuration may need to be updated consequently. Both options have pros and cons, so RAN2 should discuss which option is better and do the down selection.

Proposal 6: Down select between the following options for reference configuration:

- Option1: Network configures a reference configuration independent of the candidate SCG configurations;

- Option2: Network takes one of the candidate SCG configurations as reference configuration and indicates it to UE.

Using initial Source PSCell for subsequent SCG change

In legacy CPC, all configured PSCell configurations and the source PSCell configuration will be removed after successful CPC execution. For selective activation of cell groups, one issue is whether the initial source cell can be used for the subsequent SCG change, similar as the configured candidate SCG configurations. Simply using the source PSCell for subsequent SCG change may be not workable. The reason is that the source PSCell may be added by a normal PSCell addition/change, so there is no execution condition configured for the source cell. Besides, the source cell configuration is not the delta configuration based on the reference configuration. Therefore, it is not feasible to use the initial source PSCell for subsequent SCG change if network do not to configure source cell as a candidate PSCell in candidate SCG configuration.

Considering above, we propose that it is up to network on whether to use the initial source SCG for subsequent SCG change. If the network wants to use the source SCG for subsequent SCG change, the network should configure it as one of the candidate SCG configurations.

Proposal 7: NW configures the source SCG as one of the candidate SCG configurations if the NW wants to use it for subsequent selective SCG change.

How to keep/release the candidate SCG configurations

In RAN2#119bis-e meeting, whether UE keeps all candidate SCG configurations or if those are indicated by the network was discussed but is still FFS now.

Baseline procedure to support subsequent secondary cell group change (FFS if UE keeps all configurations or if those are indicated by the network, FFS support of nested configs): - a. Step 1: when the execution condition of a CPC candidate PScell is met, a UE performs the execution of CPC towards this candidate PScell. - b. Step 2: After finishing the PSCell addition or change, the UE doesn’t release conditional configuration of other candidate PSCells for subsequent CPC, the UE continues evaluating the execution conditions of other candidate PScells. - c. Step 3: When the execution condition of a candidate PScell is met, the UE performs the execution of CPC towards this candidate PSCell.

We understand it should depend on network to judge whether one candidate SCG configuration should be kept or released based on the RRM measurement results or based on some actual deployment situations. This gives the network more flexibility; network can dynamically indicate UE to keep/release the candidate SCG configuration in different cases. And when there is a need to update the candidate SCG configuration (e.g. network detects that some SCGs have good or poor radio link quality based on RRM measurement), the network can re-configure the candidate SCG configuration by adding/modifying/releasing some candidate SCGs. Furthermore, the network can release all candidate SCG configuration based on some actual conditions and considerations.

From the perspective of UE, the UE will keep/add/modify/release the candidate SCG configuration based on the network configuration/indication, if no network configuration/indication is received by UE, UE will keep all stored candidate SCG configurations for subsequent selective activation of cell groups after each SCG change.

Proposal 8: Whether UE keeps/releases any of the configured candidate SCG configurations for subsequent SCG change is based on indication from network.

Execution conditions

As described in section 2.1.1, for R17 MN-initiated inter-SN CPC and CPA, execution conditions for candidate PSCell are configured and generated by MN based on MN configured measConfig. If the same principle is followed, for the selective cell group case, when the procedure is initiated by MN, i.e. the execution condition is configured by MN, the execution conditions of candidate PSCell could still be applicable for subsequent SCG change due to the MN measConfig doesn’t need to updated with the SCG change procedrure. However, in R17 for SN-initiated inter-SN CPC and intra-SN CPC, execution conditions for candidate PSCells are generated by SN based on SN configured measConfig. If the same principle is followed, for the selective cell group case, when the procedure is initiated by SN, i.e. the execution condition is configured by SN, which based on the SCG measConfig. After the SCG change performed, the SCG measConfig will be updated with the new serving SCG, the configured execution conditions of other candidate PSCell which is based on the measConfig configured by the old serving SCG, will not be valid anymore. So the candidate PSCell execution conditions need to be updated after each SCG change.

For selective activation of cell groups, if SN-initiated selective activation of cell groups is supported, considering that the intention of this objective is to allow the UE perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network, execution conditions update after each SCG change should also be avoided.

Proposal 9: RAN2 aim to support selective activation of cell groups without RRC reconfiguration for execution conditions update.

Signaling structure design for selective activation of cell groups

Considering of signalling structure design for selective activation of cell groups, there are the following two options:

Option 1: Configurations of all candidate SCGs are in parallel in RRC message (similar as legacy CPC configuration signalling structure)

Option 2: Candidate SCG Configurations can be included the candidate SCG configuration (i.e. similar as CPC configuration in CPC configuration)

For option 1, it requires less spec effort and it may be sufficient for the targeted scenarios. For option 2, it may be useful in some specific case (e.g. some PSCells can only be the candidate for next SCG change when UE camp on a specific PSCell). So RAN2 need to discuss which option to support,

Therefore, it is proposed.

Proposal 10: Discuss the signalling structure for the candidate SCG configurations.

- Option 1: Configurations of all candidate SCGs are in parallel in RRC message (similar as legacy CPC configuration signalling structure)

- Option 2: Candidate SCG Configurations can be included the candidate SCG configuration (i.e. similar as CPC configuration in CPC configuration)

Conclusion

Based on the previous analysis in section 2, our main conclusions are summarized as follows:

MN-initiated CPC/ SN-initiated CPC

Observation 1: It is more suitable for MN to decide the candidate SNs for the subsequent SCG changes since SN does not have the information of the neighbour cells which may have the overlapping coverage with MN.

Observation 2: SN-initiated inter-SN CPC will introduce complexity as it requires execution condition update after each subsequent PSCell change.

Proposal 1: For the cases of CPC, support MN-initiated CPC with high priority.

How to trigger the subsequent cell group change

Proposal 2: Focus on UE triggered solution (i.e. selective activation of cell groups is triggered when the execution condition(s) of at least one candidate PSCell is fulfilled).

Term for selective activation of cell groups

Proposal 3: For selective activation of cell groups, the term “conditional selective cell groups (CSCG)” is used to distinguish from legacy CPAC.

Security issue

Proposal 4: The sk-counter cannot be reused when UE goes back to a previous cell from other SN.

Proposal 5: For security key update, NW provides a common sk-counter for all candidate target SCGs to UE, the sk-counter will be incremented after each SCG change.

Delta configuration

Proposal 6: Down select between the following options for reference configuration:

- Option1: Network configures a reference configuration independent of the candidate SCG configurations;

- Option2: Network takes one of the candidate SCG configurations as reference configuration and indicates it to UE.

Using initial Source PSCell for subsequent SCG change

Proposal 7: NW configures the source SCG as one of the candidate SCG configurations if the NW wants to use it for subsequent selective SCG change.

How to keep/release the candidate SCG configurations

Proposal 8: Whether UE keeps/releases any of the configured candidate SCG configurations for subsequent SCG change is based on indication from network.

Execution conditions

Proposal 9: RAN2 aim to support selective activation of cell groups without RRC reconfiguration for execution conditions update.

Signaling structure design for selective activation of cell groups

Proposal 10: Discuss the signalling structure for the candidate SCG configurations.

- Option 1: Configurations of all candidate SCGs are in parallel in RRC message (similar as legacy CPC configuration signalling structure)

- Option 2: Candidate SCG Configurations can be included the candidate SCG configuration (i.e. similar as CPC configuration in CPC configuration)

Reference

[1] R2 119bis-e Chair Notes EOM rev1