3GPP TSG-RAN WG2 #120	R2-2211212

Toulouse, France, November 14-18, 2022

Agenda Item:	6.15.1

Source:	OPPO

Title:	Discussion on R1-2210492

Document for:	Discussion, Decision

Introduction

This paper will discuss some left issues on R1-2210492.

Discussion

Based on R1 answer

RAN1’s reply:

In case a band combination supporting both NR Sidelink communication and NR Sidelink discovery, the UE capability of these per-FS L1 feature would be the same for NR Sidelink communication and NR Sidelink discovery.

The source of this question (Q2) was the discussion as follows in

<= Start of R2-2208855, [AT119-e][509] =>

Q5-1b: If one answers agree to Q5-1a, please share your view / preference on the Alt-1 and Alt-2 of CR provided in 8901, e.g., which alternative to adopt

- Alt-1 of Annex in 8901 (also pasted into Annex of this document)

- Alt-2 of Annex in 8901 (also pasted into Annex of this document)

- Another alternative (if this option is selected, please indicate the detailed CR solution)

| Company | Alternative | Comment |
| --- | --- | --- |
| OPPO | 1 or 2 | Can follow majority view |
| Ericsson | Comments | See reply to Q5-1a |
| vivo | Prefer Alt-2 | We assume that a BC supporting both communication/discovery always has the same L1 capabilty. Therefore, Alt-2 is better for reducing signalling overhead. |
| MediaTek | 1 or 2 |  |
| Huawei, HiSilicon | 2 | Alt-2 needs RAN1/4 confirm. |
| Intel |  | We think both options can work |
| InterDigital | 1 or 2 |  |

Rapp Comment: No much reply to this issue, so Rapp assume there is no clear objection in R2 to have this. Yet 4/9 company propose to go for LS. Rapp is OK to go for the LS, yet instead of leaving all question to RAN1, RAN2 can make the conclusion/assumption (considering 6/9 support for Q5-1a), that these per-FS L1 features also apply to discovery BC-list, and ask for RAN1 concern if any. Furthermore, it can include question of Alt-1 and Alt-2 above (5/9 support option-2).

Proposal 9	(6/9) RAN2 assume the sidelink per-FS L1 features also apply to and thus should be reported in discovery BC-list. Send LS to RAN1 to ask if any concern (4/9). In the LS, also ask RAN1 for a same BC supporting both communication and discovery, whether these per-FS L1 features can be different depending on the usage for Communication or for Discovery.

<= Start of R2-2208855, [AT119-e][509] =>

Based on R1 reply, it is clear that for a same BC supporting both communication and discovery, there would be some unnecessary signaling overhead if reporting per-FS capability in both

1/ BC list for communication: BandCombinationListSidelinkEUTRA-NR

2/ BC list for discovery: supportedBandCombinationListSL-NonRelayDiscovery, supportedBandCombinationListSL-RelayDiscovery

Then given in legacy spec, if a BC supports communication (regardless of whether it support discovery or not), it would be included in the BC list for communication, seems we should rely on 1 above for the per-FS capability reporting of BC supporting both communication and discovery.

Then the problem is how for network to differentiate, for BCs in BandCombinationListSidelinkEUTRA-NR, between the BC supporting

A/ communication only

B/ both communication and discovery.

One solution is to introduce additional flag for the communication BC-list

BandCombinationParametersSidelinkEUTRA-NR-v17xy ::= SEQUENCE {

supportNonRelayDiscovery-r17           ENUMERATED {supported}                          OPTIONAL,

supportRelayDiscovery-r17              ENUMERATED {supported}                          OPTIONAL

}

Report additional indication for the BC in BandCombinationListSidelinkEUTRA-NR supporting both communication and non-relay/relay discovery, to differentiate BC supporting both communication and non-relay/relay discovery and discovery only.

Conclusion

We have the following proposals:

Proposal 1	Report additional indication for the BC in BandCombinationListSidelinkEUTRA-NR supporting both communication and non-relay/relay discovery, to differentiate BC supporting both communication and non-relay/relay discovery and discovery only.

Reference

[1] R1-2210492, Reply LS to RAN2 on Per-FS L1 feature for NR sidelink discovery BC-list