3GPP TSG RAN WG2#120		R2-2211235

Toulouse, France, 14th - 18th November, 2022

Source:	vivo

Title:          Consideration of use case specific aspects

Agenda Item:	8.16.3

Document for:	Discussion and Decision

- Introduction

At the RAN#94-e meeting, the SID on AI/ML for NR Air Interface RP-213599 [1] was approved. The objectives regarding the use case specific aspects of AI/ML for air are as follows:

For the use cases under consideration: - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we will discuss the use case specific aspects based on RAN1’s progress, the target is to reach a common understanding of the general procedure and potential spec impact for each use case.

- Discussion

According to the SID, the use case-specific aspects require sufficient progress in RAN1 as input. However, there is currently no full conclusion on each component in RAN1. In this case, RAN2 may focus on RAN1 current progress and agreements and identify the relevant specification impact from RAN2 perspective, the following aspects shall be considered:

- Locations of AI/ML functionalities, including data collection, model training, model inference and model monitoring.

- Signaling procedure of each (sub)use case at different collaboration levels

- Potential standard impact

Proposal 1: Based on RAN1 progress, RAN2 to study the use case specific aspects in terms of:

- Locations of AI/ML functionalities, including data collection, model training, model inference and model monitoring.

- Signaling procedure of each (sub)use case at different collaboration levels

- Potential standard impact

- 2.1 CSI feedback enhancement

As CSI compression has been approved as one of the representative sub-use cases in CSI feedback enhancement, we will take it as an example to analyze the overall procedure and potential standard impact.

At the RAN1 #110bis-e meeting, some related RAN1 agreements regarding the CSI compression were reached as follows:

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring. Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode. Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection Agreement In CSI compression using two-sided model use case, further study at least use cases of the following potential specification impact on quantization method alignment between CSI generation part at UE and CSI reconstruction part at gNB: - Alignment of the quantization/dequantization method and the feedback message size between Network and UE

- 2.1.1 Locations of AI/ML functionalities

Where some AI/ML functionalities, such as Data collection, Model Training and Model inference, for CSI feedback are located can be considered for supporting AI-based CSI compression, as described below in Table 2.1.1-1.

Table 2.1.1-1: typical locations of AI/ML functionalities

| Options | Data Collection | Model Training | Model Inference |
| --- | --- | --- | --- |
| Type 1 | gNB | gNB | gNB and UE |
| Type 2 | gNB and UE | gNB and UE | gNB and UE |
| Type 3 | gNB and UE | gNB and UE | gNB and UE |

Note: depending on RAN1 progress, other Options (s) are not precluded, thus the final option to select is based on RAN1's further progress.

In the following sections, we take Type 1 in Table 2.1.1-1 as an example to illustrate the potential overall procedure for CSI compression.

- 2.1.2 Potential input/output/performance feedback

(1) Input of generation model

To make the prediction for CSI compression, the following information may be needed as input data:

From the UE:

- CSI, e.g., PMI or raw channel.

(2) Output of generation model and Input of reconstruction model

AI/ML-based CSI compression can generate the following information as output for the model generation:

- compressed CSI report.

(3) Output of reconstruction model

AI/ML-based CSI compression can generate the following information as output for the model reconstruction:

- CSI, e.g., PMI or raw channel.

(4) Performance Feedback

AI/ML-based CSI compression needs the following information as performance feedback:

- inference accuracy such as SGCS of model input and output.

- system level performance such as throughput.

- 2.1.3 Overall procedure

Figure 2.1.3-1: signaling procedures of CSI compression

Step 0. The UE indicates its AI/ML-related capability to the RAN node.

Step 1. The RAN node sends the CSI RS resource and reporting configuration to the UE.

Step 2. The UE performs CSI measurement and feedback CSI report, e.g., PMI or raw channel.

Step 3. The RAN node performs model training based on the acquired report.

Step 4/5. The RAN node may send the [model management-related configuration] and transfer the model to UE.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the RAN node.

Step 7. The UE performs CSI measurement and obtains the data required for model inference as CSI generation.

Step 8. The UE reports the compressed CSI report and the RAN node performs subsequent model inference as CSI reconstruction.

Step 9. The RAN node may send model performance monitoring configuration to the UE.

Step 10. The UE reports the performance feedback to the RAN node. The report can be one-shot, event-triggered, or periodical.

Step 11. If the action performed by the UE results in suboptimal system performance (e.g. low throughput due to mismatch of models), or if the accuracy of the model inference result does not meet the expectations (e.g. the SGCS is below a configured threshold), the UE can deactivate the model itself or by the network indication.

- 2.1.4 Model training for two-sided model

At the RAN1#110 meeting, three types of model training were agreed to be further studied.

•	Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

•	Type 2: Joint training of the two-sided model at network side and UE side, repectively.

•	Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

At the RAN1#110bis-e meeting, the general procedures of type2 and type3 model training were concluded:

Conclusion For the evaluation of Type 2 (Joint training of the two-sided model at network side and UE side, respectively), following procedure is considered as an example: - For each FP/BP loop, - Step 1: UE side generates the FP results (i.e., CSI feedback) based on the data sample(s), and sends the FP results to NW side - Step 2: NW side reconstructs the CSI based on FP results, trains the CSI reconstruction part, and generates the BP information (e.g., gradients), which are then sent to UE side - Step 3: UE side trains the CSI generation part based on the BP information from NW side - Note: the dataset between UE side and NW side is aligned. - Other Type 2 training approaches are not precluded and reported by companies Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with NW side training (NW-first training): - Step1: NW side trains the NW side CSI generation part (which is not used for inference) and the NW side CSI reconstruction part jointly - Step2: After NW side training is finished, NW side shares UE side with a set of information (e.g., dataset) that is used by the UE side to be able to train the UE side CSI generation part - Step3: UE side trains the UE side CSI generation part based on the received set of information - Other Type 3 NW-first training approaches are not precluded and reported by companies Conclusion For the evaluation of an example of Type 3 (Separate training at NW side and UE side), the following procedure is considered for the sequential training starting with UE side training (UE-first training): - Step1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly - Step2: After UE side training is finished, UE side shares NW side with a set of information (e.g., dataset) that is used by the NW side to be able to train the CSI reconstruction part - Step3: NW side trains the NW side CSI reconstruction part based on the received set of information - Other Type 3 UE-first training approaches are not precluded and reported by companies

(1) Training collaboration type 1

In training collaboration type 1, the CSI generation part and CSI reconstruction part will be jointly designed and trained at a single entity based on either own collected data (e.g., at the UE side) or reported data (e.g., at the network side). Given the advantages of computation capability and storage at the gNB side, joint training at the network side will become the dominant choice. The training procedure itself does not have any explicit specification impacts, but the trained model needs to be transferred to the other side for model inference.

One obvious advantage of training collaboration type 1 is the optimal training performance compared with other collaboration types because the CSI generation and reconstruction parts can be jointly designed. Another promising advantage of training collaboration type 1 with the model transfer is the highest model flexibility among all training collaborations, i.e., the network could select and transfer one or several models that most suit the current situation/configuration for a specific UE. The transferred models do not need to generalize very well across various scenarios/configurations and only need to adapt to the current scenario/configuration. Such model flexibility could introduce an “overfitting” gain, i.e., scenario/configuration-specific model usually performs better than those generalizing well across multiple scenario/configurations.

However, there are still concerns on collaboration type 1, of which the most mentioned one is the model proprietary issue. To be specific, when transferring a model between entities from different vendors, it is inevitable that detailed model information will be disclosed. Nevertheless, we believe that concerns on model proprietary mainly focus on fully-designed complicated models. For simple models such as one-layer MLP, concerns on model proprietary will be much less.

(2) Training collaboration type 2

In training collaboration type 2, the CSI generation part and CSI reconstruction part will be separately designed and jointly trained at network and UE. The training procedure is enabled by exchanging essential information over the air. From our view, the interaction information required in collaboration type 2 includes forward-propagation information, backward-propagation information (gradients), required labels, etc. From the perspective of model performance, it could be observed that models trained through collaboration type 2 could achieve those of models trained through collaboration type1 if models are fully trained in enough epochs. For the generalization issue, as the CSI generation and reconstruction parts are distributed on different entities but jointly trained, both two sides (network and UE) must maintain the same number of models for various scenarios/configurations, which could be a heavy burden for UEs with limited storage room. Although there are techniques to train a common CSI generation part to multiple CSI reconstruction parts to reduce the number of required models at UE, multiple CSI generation parts are still needed. In addition, because the CSI generation and reconstruction part are designed separately at the network and UE, model proprietary could be kept within the network and UE. However, the concerns for collaboration type 2 are also obvious: the over-the-air overhead for joint training is very high as there are too many iterations in one complete training procedure (especially for training from scratch).

(3) Training collaboration type 3

In training collaboration type 3, the CSI generation part and CSI reconstruction part will be separately designed and trained at network and UE. The major challenge for separate training is how to guarantee the separately trained CSI generation and reconstruction parts could match each other. The most reported approach by companies to enable separate training is to share model input and output data with the other side. Note that it is both technically feasible to share data of CSI generation and reconstruction part to the other side. Performance for separate training highly depends on the amount of exchanged data. Exchanging an insufficient amount of data will result in obvious performance degradation. In addition, it is still necessary for UE and the network to align some information of the utilized model, such as the quantization/dequantization method. Otherwise, there will be a non-acceptable performance loss.

The advantage of separate training is to mostly keep the model proprietary within each side, and the corresponding overhead (i.e., overhead in exchanging data between UE and gNB) can be reduced to an acceptable level by applying some dataset compression approaches. However, the overhead of separate training is still higher than that of transferring a simple model, and the performance of separate training is usually sub-optimal compared with joint training. In addition, UEs that use separate training to train models still need to keep multiple models to match different CSI reconstruction parts for various scenarios/configurations.

The characteristics of the three training collaborations are in the following table:

Table 2.1.4-1: Characteristics for the three training collaboration types

|  | Type 1: Joint training at single entity | Type 2: Joint training at NW and UE | Type 3: Separate training at NW and UE |
| --- | --- | --- | --- |
| Interaction approach and necessary exchanging information | Model transfer through: Option 0: Sending updated parameters without changing the AI/ML model structure. Option 1: Sending AI/ML model parameter and structure information. | Exchanging the following information over the air (for each batch): forward- and backward- propagation results, label data, hyperparameters information for training and inference, etc. | Exchanging the following information over the air: paired model input/output data for the passive side (e.g., UE sends input/output of CSI reconstruction part to gNB), some (high level) information on model structure, etc. |
| Generalization issue | Transferring specific model for the current scenario/configuration. Only one side (usually the NW) needs to store many models for different scenarios/configurations. | Training multiple pairs of models targeting different scenarios/configurations, or common encoder/decoder to multiple decoders/encoders at the cost of some performance loss. Both sides should maintain models for various scenarios/configurations. | Training multiple pairs of models targeting different scenarios/configurations, or common encoder/decoder to multiple decoders/encoders at the cost of some performance loss. Both sides should maintain models for various scenarios/configurations. |
| Over-the-air overhead if supported by air interface enhancement | Overhead depends on the model size, could be smaller for very trivial model, e.g., one-layer MLP. | Overhead ≈ # of epoch*(forward-propagation information + back-propagation information + label information). Overhead grows linearly as the number of iterations, which is usually high. | Depending on the size of paired model input/output data. Usually lower than type 2 but higher than type 1 with trivial models. |
| Offline effort | N.A. | If the exchange of derivatives is done offline, this type would require complicated offline agreement for multi-parties to develop a usable model | If the exchange of data is done offline, this type would require offline agreement for multi-parties to share data. |
| Model Proprietary | Not keep | Mostly Keep | Mostly Keep Model structure may still need to be exposed to some extent. Quantization must be aligned between parties. |
| Performance | Upper bound for all training collaborations | Some performance degradation than type 1 due to potential hyperparameter misalignment. Suffers from performance degradation when one model needs to be pair with multiple models. | Depend on the scale of exchanged data. Could achieve that of joint training if enough data is exchanged. Suffers from performance degradation when one model needs to be pair with multiple models. |

Based on the above analysis, we propose:

Proposal 2: RAN2 to study the pros and cons of each model training collaboration type for the two-sided model.

- 2.1.5 Potential standard impact

AI/ML-based CSI compression may introduce the following potential Uu interface impact:

- New or enhanced existing signaling procedure to request/retrieve CSI, e.g., PMI or raw channel.

- New or enhanced existing signaling procedure to request/retrieve compressed CSI report.

- New or enhanced existing signaling procedure to request/retrieve performance monitoring parameters, e.g., SGCS of model input and output.

- New signaling procedure for two-sided model training, including model transfer.

- 2.2 Beam management

At the RAN1 #110bis-e meeting, some related RAN1 agreements regarding beam management were reached as follows:

Agreement For BM-Case1 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) that is based on the output of AI/ML model inference - FFS: Predicted L1-RSRP corresponding to the beam(s) - FFS: other information Agreement For BM-Case2 with a UE-side AI/ML model, study the potential specification impact of L1 signaling to report the following information of AI/ML model inference to NW - The beam(s) of N future time instance(s) that is based on the output of AI/ML model inference - FFS: value of N - FFS: Predicted L1-RSRP corresponding to the beam(s) - Information about the timestamp corresponding the reported beam(s) - FFS: explicit or implicit - FFS: other information Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation

- 2.2.1 Locations of AI/ML functionalities

The following Options can be considered for supporting AI-based beam management AI/ML functionalities.

Table 2.2.1-1: typical locations of AI/ML functionalities

| Options | Data Collection | Model Training | Model Inference |
| --- | --- | --- | --- |
| Option 1 | gNB | gNB | gNB |
| Option 2 | UE | UE | UE |
| Option 3 | gNB | gNB | UE |

Note: depending on RAN1 progress, other Options (s) are not precluded, thus the final option to select is based on RAN1's further progress.

In the following sections, we take Option 3 in Table 2.2.1-1 as an example to illustrate the over procedure.

- 2.2.2 Potential input/output/performance feedback

(1) Input

To make the prediction for Beam management, the following information may be needed as input data:

From the UE:

- L1-RSRP for part of the beams, each associated with a beam index

- Rx Beam angle

(2) Output

AI/ML-based Beam management can generate the following information as output:

- L1-RSRP for all of the beams, each associated with a beam index

(3) Performance Feedback

AI/ML-based Beam management needs the following information as performance feedback:

- Top-k predicted L1-RSRP, Tx beam indexes, and beam prediction accuracy.

- 2.2.3 Overall procedure

Figure 2.2.3-1: signaling procedures of beam management

Step 0. The UE indicates its AI/ML-related capability to the RAN node.

Step 1. The RAN node sends the beam sweeping resource and report configuration to the UE.

Step 2. The UE performs L1-RSRP measurement and feedback with the Rx beam angel.

Step 3. The RAN node performs model training based on the acquired report.

Step 4/5. The RAN node may send the [model management-related configuration] and transfer the model to UE.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the RAN node.

Step 7/8. The UE performs L1-RSRP measurement and obtains the data required for model inference.

Step 9. The RAN node may send model performance monitoring configuration to the UE.

Step 10. The UE reports the performance feedback to the RAN node. The report can be one-shot, event-triggered, or periodical.

Step 11. If the action performed by the UE results in suboptimal system performance (e.g. low throughput due to selecting an unreasonable beam), or if the accuracy of the model inference result does not meet the expectations (e.g. the accuracy of the prediction is below a configured threshold), the UE can deactivate the model itself or by the network indication.

- 2.2.4 Potential standard impact

AI/ML-based Beam management may introduce the following potential Uu interface impact:

- New or enhanced Beam sweeping configuration.

- New or enhanced existing signaling procedure to request/retrieve L1-RSRP for part of the beams and Rx beam information.

- New or enhanced existing signaling procedure to request/retrieve predicted L1-RSRP and Tx beam.

- New or enhanced existing signaling procedure to request/retrieve performance monitoring parameters, e.g., Top-k predicted L1-RSRP, Tx beam indexes, and beam prediction accuracy.

- New signaling procedure for model transfer.

- 2.3 Positioning accuracy enhancement

At the RAN1 #110bis-e meeting, some related RAN1 agreements regarding the positioning accuracy enhancement were reached as follows:

Agreement - Study and provide inputs on benefit(s) and potential specification impact at least for the following cases of AI/ML based positioning accuracy enhancement - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning Agreement Regarding AI/ML model indication[/configuration], to study and provide inputs on potential specification impact at least for the following aspects on conditions/criteria of AI/ML model for AI/ML based positioning accuracy enhancement - Validity conditions, e.g., applicable area/[zone/]scenario/environment and time interval, etc. - Model capability, e.g., positioning accuracy quality and model inference latency - Conditions and requirements, e.g., required assistance signalling and/or reference signals configurations, dataset information - Note: other aspects are not precluded Agreement Regarding AI/ML model monitoring for AI/ML based positioning, to study and provide inputs on potential specification impact for the following aspects - Assistance signaling and procedure at least for UE-side model - Report/feedback and procedure at least for Network-side model - Note1: study is applicable to both of the following cases - Model inference and model monitoring at the same entity - Entity to perform the model monitoring is not the same entity for model inference - Note2: other aspects are not precluded Agreement Regarding data collection for AI/ML model training for AI/ML based positioning, at least for each of the agreed cases (Case 1 to Case 3b) - Study whether (and if so how) an entity can be used to obtain ground truth label and/or other training data - Companies are requested to report their assumption of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) - Companies are requested to report their assumption of applicable ground truth label (e.g., location or other information) and/or other training data (e.g., measurement) for each case (Case 1 to Case 3b) - Feasibility study on the entity to obtain ground truth label and/or other training data takes into account at least - availability of the entity to obtain label and/or other training data - Note: further discussion and decision of the entity (or entities) used to obtain ground truth label and/or other training data for each case (Case 1 to Case 3b) is not precluded based on companies’ input - Study potential signalling and procedure to enable data collection - Potential specification impact on the details of request/report of label and/or other training data, and to enable delivering the collected label and/or other training data to the training entity when the training entity is not the same entity to obtain label and/or other training data - Potential specification impact on assistance signaling indicating reference signal configuration(s) to derive label and/or other training data

- 2.3.1 Locations of AI/ML functionalities

The following Options can be considered for supporting AI-based beam management.

Table 2.3.1-1: typical locations of AI/ML functionalities

| Options | Data Collection | Model Training | Model Inference |
| --- | --- | --- | --- |
| Option 1 | LMF | LMF | LMF |
| Option 2 | LMF | LMF | gNB |
| Option 3 | LMF | LMF | UE |

Note: depending on RAN1 progress, other Options (s) are not precluded, thus the final option to select is based on RAN1's further progress.

In the following sections, we take Option 3 in Table 2.3.1-1 as an example to illustrate the over procedure.

- 2.3.2 Potential input/output/performance feedback

(1) Input

To make the prediction for Positioning accuracy enhancement, the following information may be needed as input data:

From the UE:

- CIR measurement

(2) Output

AI/ML-based Positioning accuracy enhancement can generate the following information as output:

- TOA, LOS/NLOS identification

- Location estimate

(3) Performance Feedback

AI/ML-based Positioning accuracy enhancement needs the following information as performance feedback:

- positioning error, channel condition, movement status.

- 2.2.3 Overall procedure

Figure 2.2.3-1: signaling procedures of Positioning accuracy enhancement

Step 0. The UE indicates its AI/ML-related capability to the LMF.

Step 1. The LMF sends the PRS resource and reports configuration to the UE.

Step 2. The UE performs PRS measurement and feedback with the CIR, RSRP, or ToA, which may be associated with the ground-truth location of the LMF.

Step 3. The LMF performs model training based on the acquired report.

Step 4/5. The LMF may send the [model management-related configuration] and transfer the model to UE.

Step 6. The UE may activate the model based on the model configuration autonomously or with explicit activation indication from the LMF.

Step 7/8. The UE performs PRS measurement and obtains the data required for model inference.

Step 9. The LMF may send model performance monitoring configuration to the UE.

Step 10. The UE reports the performance feedback to the LMF. The report can be one-shot, event-triggered, or periodical.

Step 11. If the action performed by the UE results in suboptimal location performance (e.g. low location accuracy), the UE can deactivate the model itself or by the network indication.

- 2.3.4 Potential standard impact

AI/ML-based Positioning accuracy enhancement may introduce the following potential Uu interface impact:

- New or enhanced PRS configuration.

- New or enhanced existing signaling procedure to request/retrieve PRS measurement result, e.g., CIR report.

- New or enhanced existing signaling procedure to request/retrieve predicted location estimate, TOA, LOS/NLOS identification.

- New or enhanced existing signaling procedure to request/retrieve performance monitoring parameters, e.g., positioning error, channel condition, movement status.

- New signaling procedure for model transfer.

- 2.4 Summary

According to the progress and agreements in RAN1, the explicit input/output/performance feedback of each use case is still FFS. Besides, RAN1 had a good progress on each component and would provide sufficient guidance when available. RAN2 shall wait for RAN1’s further input to discuss the configuration and reporting of the essential metrics.

As to the overall procedure, the above procedures are based on our understanding of each use case and will be updated along with RAN1 progress. Meanwhile, we think it’s better to capture the overall procedure for each case into the TR when RAN2 reaches the consensus.

Proposal 3: FFS explicit input/output/performance feedback based on RAN1 further progress.

Proposal 4: Capture the overall procedure for each use case into the TR 38.843, when available.

- Conclusion

Proposal 1: Based on RAN1 progress, RAN2 to study the use case specific aspects in terms of:

- Locations of AI/ML functionalities, including data collection, model training, model inference and model monitoring.

- Signaling procedure of each (sub)use case at different collaboration levels

- Potential standard impact

Proposal 2: RAN2 to study the pros and cons of each model training collaboration type for the two-sided model.

Proposal 3: FFS explicit input/output/performance feedback based on RAN1 further progress.

Proposal 4: Capture the overall procedure for each use case into the TR 38.843, when available.

- Reference

[1]. RP-213599		Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface