3GPP TSG-RAN WG2 Meeting #120	   R2-2211248

Toulouse, France, November 14th – 18th, 2022

Agenda item:		8.11.2

Source:	vivo

Title:	Supporting Multicast Reception in RRC_INACTIVE from Lower Layer Aspects

Document for:		Discussion and Decision

1 Introduction

In the previous RAN2 meeting, the corresponding agreements regarding multicast reception in the RRC_INACTIVE state had been achieved [1],

Agreement - For PTM configuration delivery, RAN2 further investigates the following solutions: - Option 1: Dedicated signalling - Option 2: Solution based on SIB+MCCH - We do not preclude some “mix” of the options - For both option 1 and option 2, as a baseline, group paging can be used to switch UEs receiving multicast from RRC_INACTIVE to RRC_CONNECTED, and UEs continue the multicast reception in CONNECTED. FFS if there is any potential issue if Rel-17 group paging is reused. FFS if there are other cases when UE triggers resume. FFS if MCCH can also be used in case of option 2. - FFS whether to introduce PTM configuration applicable area, i.e., the mechanism that the PTM configurations, once acquired by a UE, may apply to a certain area (i.e., a set of cells instead of a single cell). - If option 1 is supported for PTM configuration As a baseline, group paging may be used to inform the UE when network changes the PTM configurations, and UE upon reception triggers RRC connection resume procedure to obtain the updated configurations (details of group paging can be FFS). FFS whether and how to solve the issue in signalling/system load when a large number of UEs in the cell need PTM configuration update. - As a baseline, group paging can be used to inform Rel-18 UE(s) about the session activation (Details FFS, e.g., UE behavior when receiving such group notification). - FFS how UE determines whether it can receive the multicast session in RRC_INACTIVE or not when the session is activated, taking into account the following solutions (can further update the descriptions if needed, and several solutions may be needed, some solutions may apply only for certain configuration options) 1. When the multicast session is activated, UE can receive the multicast session in RRC_INACTIVE if the PTM configuration used in RRC_INACTIVE for the session is available to the UE and the UE has joined the session already (e.g., configuration provided to UE via dedicated RRC signaling or via MCCH), otherwise it goes back to RRC_CONNECTED to receive the multicast session. 2. When the multicast session is activated, UE is indicated by group paging whether it can receive the multicast session in RRC_INACTIVE or not (detailed signaling FFS). 3. UE is configured "whether it can receive the multicast session in RRC_INACTIVE" by dedicated signaling before UE is released. When the multicast session is activated, UE stays in RRC_INACTIVE or resumes RRC connection accordingly (detailed signaling FFS). - If a UE is in RRC_INACTIVE and is configured to receive a multicast session in RRC_INACTIVE, the UE may be notified when the multicast session is deactivated. FFS how (e.g., informed via group paging, MCCH, or other ways).

However, all these agreements are not related to the actual reception of multicast data (e.g. CFR configuration, TCI state configuration, and group-common SPS configuration). Consequently, it is not clear how to realize and perform the multicast reception in RRC INACTIVE state. To facilitate the multicast reception in RRC INACTIVE, we would like to provide our initial understanding of multicast reception in the RRC_INACTIVE state from lower layer aspects (including CFR, BWP, PDCCH/PDSCH, RRM, and failure handling).

2 Discussion

2.1 CFR & BWP

Regarding Rel-15/16/17 RRC_INACTIVE UEs, it always assumes the bandwidth and location of the initial DL BWP are the same as those of CORESET 0. If we reuse this legacy principle for multicast reception in Rel-18, without a doubt, the bandwidth might be not large enough to support some multicast services requiring high throughput.

To as fully as possible fulfill the requirement of multicast reception, similarly to the Rel-17 NR MBS broadcast, we think the common frequency resource (CFR) for group-common PDCCH/PDSCH of multicast reception will cover the following cases:

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

Some may argue that the UE should alternatively use the dedicated BWP configuration with multicast CFR that is used when the UE is in the CONNECTED state. From the feasibility point of view, it is practical. However, if so, BWP switching in RRC INACTIVE may be needed when the UEs need to monitor the paging message or trigger the RA procedure (which may require RAN4’s attention on the evaluation of requirements about BWP operation for RRC INACTIVE UE). What’s worse, more studies and discussions may be needed to figure out what kind of UE-specific or cell-specific parameters that configured via BWP configuration are applicable to the RRC INACTIVE state or should be limited (e.g. whether RLM or BFD is needed for multicast reception in the RRC INACTIVE state).

By comparing the pros and cons of initial BWP and dedicated BWP solution, we suggest that the CFR concept for broadcast and initial BWP (or Redcap-specific initial BWP) can be reused for the Rel-18 multicast reception in RRC_INACTIVE state. Note that, by proper network configuration, the CFR resource used for RRC_CONNECTED UEs can be the same as that for RRC_INACTIVE UEs, as shown in Figure 1. Therefore, we have the following 2 proposals,

Proposal 1: Only initial BWP is used for multicast reception in RRC_INACTIVE.

Proposal 2: Separate initial BWP in case of REDCAP is considered as the initial BWP.

Proposal 3: Configuration for multicast reception in RRC INACTIVE state can be configured on separate BWP for REDCAP UEs.

Proposal 4: CFR used for multicast reception in RRC_INACTIVE can be configured to support the following cases (Same as MBS broadcast for RRC_IDLE and RRC RRC_INACTIVE):

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

Figure 1: CFR configuration for multicast reception in RRC_INACTIVE

2.2 PDCCH & PDSCH

To facilitate multicast reception in INACTIVE, necessary L1 configuration related to PDCCH/PDSCH, such as TCI state for QCL assumption, CORESET/CSS configuration, group common SPS.

First, we would like to investigate the TCI state for QCL assumption. In our understanding, the simplest solution is to keep the TCI state used in CONNECTED for INACTIVE multicast or reconfigure the TCI state used in INACTIVE (the TCI state might be linked to a CFR, instead of a CORESET). However, the problem of this method is that only the coverage area of a specific beam is suitable for multicast data reception. If the UE receiving multicast data in RRC INACTIVE moves from one beam area to another beam area, service discurption may occur due to beam failure. Alternatively, to make better coverage, an MTCH window alike solution can be used. For example, all the SSBs indicated by SIB1 are mapped with a PDCCH occasion. Then the UE could by implementation select an SSB for QCL assumption. Moreover, the NW may select some specific beams to make the association relationship (i.e. make it beam-area service, as shown in Figure 2).

Considering that seamless/lossless mobility is not required for multicast reception in RRC INACTIVE state, we assume that reusing the configuration for multicast reception in RRC CONNECTED is sufficient and the simplest for R18 multicast enhancement.

Next, we take a further look at the other parameters included in the PDCCH/PDSCH configuration. In Rel-17 MBS, DCI format 4_1/4_2 has been introduced for group scheduling. It seems a spontaneous logic to reuse it for the multicast reception in INACTIVE. And the PDSCH configuration can also be inherited from those used in RRC CONNECTED state (similar to SDT procedure that the PDSCH configuration can be included in the RRC Release message). It seems the whole procedure works. So, we have the following,

Proposal 5: Configurations for multicast reception in RRC CONNECTED state regarding PDCCH/PDSCH (e.g. TCI state, search space) can be reused for multicast reception RRC INACTIVE state.

Proposal 6: Beam sweeping is not supported for multicast reception RRC INACTIVE state.

Proposal 7: GC-SPS can be used for multicast reception RRC INACTIVE state.

Figure 2: Multicast reception in INACTIVE

2.3 RRM

Further, in the WI, it also mentions that “In Rel-17, RAN only specifies multicast for UEs in RRC_CONNECTED state, which may not fully fulfill the requirements of, e.g., Mission Critical Services, especially for cells with a large number of UEs according to TR 23.774. Also, to always keep UEs in RRC_CONNECTED state is not power efficient. It is ‎therefore important to support multicast for UEs in RRC_INACTIVE”. In our understanding, on one hand, multicast reception in RRC_INACTIVE generally is intended to help to relieve the heavy load on the scarce PUCCH resources (i.e. the NW may release the UEs that are only interested in multicast service to RRC_INACTIVE and reserve the PUCCH resources for the other UEs with frequent unicast transmission, accommodating more UEs to communicate within the same serving cell). On the other hand, it further helps the UEs in the cell center edge with low mobility to keep multicast reception without performing RRM-related measurements (e.g. RLM, BFD) and SRS transmission. In this sense, to maximize the benefit brought by multicast reception in RRC_INACTIVE, we think PUCCH and SRS transmission are not required. Meanwhile, RLM, BFD, and BFR procedures are also not required.

Observation: Multicast reception in RRC_INACTIVE is intended for UL resources offload and UE power saving.

Proposal 8: RLM, BFD, and BFR configurations are not required for multicast reception in RRC_INACTIVE.

Next, for Rel-15/16/17 RRC_INACTIVE UEs, configured measurement gap by the network is not supported for itner-frequency measurement. For Rel-18 multicast reception in RRC_INACTIVE, although data transmission misalignment between the gNB and UEs in the time domain can be avoided with an explicit measurement gap configuration, for simplicity, we prefer to not introduce optimization. If high QoS requirements are expected, the network anyway can use group paging to transit the UEs back to RRRC_CONNECTED. Thus, here are the last proposal,

Proposal 9: Measurement gap configuration is not supported for multicast reception in RRC_INACTIVE.

2.4 Failure handling

After transiting into RRC_INACTIVE, the UE may continuously monitor the PDCCH for multicast reception. However, also assuming Proposal 8 is agreeable, the UE may fail to receive the multicast data while keeping monitoring PDCCH for a time due to poor radio quality (e.g. beam failure). In such a case, as RLM/BFR mechanisms are not available for the INACTIVE UE and the NW can neither know the channel state of the UEs nor freely indicate the UE-specific reconfiguration, the UE has no choice but to monitor the PDCCH in vain. As a result, unnecessary power consumption is inevitable until cell reselection is implemented.

To avoid such embarrassment, RAN2 should consider early recovery for the RRC_INACTIVE UEs with multicast reception. For instance, upon detecting multicast reception failure (e.g. moving out of the coverage area of multicast service), the UE should be allowed to trigger RRC resumption, so that the network can access the UE and reconfigure it appropriately.

Proposal 10: For the UE receiving multicast in RRC_INACTIVE, RRC resumption is initiated upon detecting multicast reception failure.

3 Conclusion

In this contribution, we have provided our initial understanding of multicast reception in the RRC_INACTIVE state from lower layer aspects (including CFR, BWP, PDCCH/PDSCH, RRM, and failure handling). All our observation and proposals are summarized as follows,

CFR&BWP:

Proposal 1: Only initial BWP is used for multicast reception in RRC_INACTIVE.

Proposal 2: Separate initial BWP in case of REDCAP is considered as the initial BWP.

Proposal 3: Configuration for multicast reception in RRC INACTIVE state can be configured on separate BWP for REDCAP UEs.

Proposal 4: CFR used for multicast reception in RRC_INACTIVE can be configured to support the following cases (Same as MBS broadcast for RRC_IDLE and RRC RRC_INACTIVE):

    - [Case E] the case where a CFR is defined with the following properties:

      - The CFR is different than the initial BWP, and is not larger than the carrier bandwidth.

      - The CFR needs to fully contain the initial BWP in frequency domain and has the same SCS and CP as the initial BWP.

    - [Case A] A CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

    - [Case C] A CFR with same size as the initial BWP, where the initial BWP has the frequency resources configured by SIB1. In this case the CFR has the same frequency resources and same SCS and CP as the initial BWP.

PDCCH&PDSCH:

Proposal 5: Configurations for multicast reception in RRC CONNECTED state regarding PDCCH/PDSCH (e.g. TCI state, search space) can be reused for multicast reception RRC INACTIVE state.

Proposal 6: Beam sweeping is not supported for multicast reception RRC INACTIVE state.

Proposal 7: GC-SPS can be used for multicast reception RRC INACTIVE state.

RRM:

Observation: Multicast reception in RRC_INACTIVE is intended for UL resources offload and UE power saving.

Proposal 8: RLM, BFD, and BFR configurations are not required for multicast reception in RRC_INACTIVE.

Proposal 9: Measurement gap configuration is not supported for multicast reception in RRC_INACTIVE.

Failure handling:

Proposal 10: For the UE receiving multicast in RRC_INACTIVE, RRC resumption is initiated upon detecting multicast reception failure.

4 References

[1] R2-119bis-e Meeting Report.