3GPP TSG RAN WG2 Meeting #120	R2-2211271

Nov. 14th – Nov. 18th, 2022

Agenda item:	8.11.2

Source:	TD Tech, Chengdu TD Tech

Title:	Analysis of options for sending PTM configuration

Document for:	Discussion and Decision

- Introduction

During the RAN2#119bis-e meeting, the following agreement on multicast reception in RRC_INACTIVE state was made for the 3GPP R18 WI “Enhancements of NR Multicast and Broadcast services”.

The following general description is taken as baseline for PTM configuration delivery Option 1:

- (1-a) PTM configuration(s) (i.e., configurations used for multicast reception in RRC_INACTIVE) of one or more multicast sessions for at least one cell are provided via dedicated RRC signaling to a UE.

- (1-b) The RRC message for this includes RRCReconfiguration and/or RRCRelease and/or RRCResume (details FFS)

- (1-c) UE stores the received configurations while it is in RRC_INACTIVE, and if there is a need to update some or all the configurations, the UE is notified of such changes and may trigger RRC connection resume to obtain the updated configurations. In case of mobility in RRC_INACTIVE, the UE triggers RRC connection resume if the configuration of the session is not available for the new cell.

The following general description is taken as baseline for PTM configuration delivery Option 2:

- (2-a) PTM configurations (i.e., configurations used for multicast reception in RRC_INACTIVE) are provided via an MCCH-like channel (same or different as used for MBS broadcast), and information regarding MCCH scheduling is provided via SIB, FFS dedicated signalling

- (2-b) UE can receive such configurations when it is in RRC_INACTIVE, FFS whether it is allowed/needed to also receive when UE is in RRC_CONNECTED

- (2-c) If there is a need to update some or all the received configurations, UE does not need to resume RRC connection but is notified of such changes (e.g. via MCCH DCI) and obtains the updated configurations via MCCH.

Based on the above agreement for option 2, option 2 can be divided into the following three sub-options:

Option 2: SIB + cell specific MCCH (one MCCH per cell)

Option 2-1: dedicated signalling + cell specific MCCH (one MCCH per cell)

Option 2-2: dedicated signalling + session specific MCCH (one MCCH per G-RNTI/G-CS-RNTI)

In the contribution, the above three sub-options are analyzed from the following aspects.

- Difference between option 2-1 and option 2-2

- Difference between option 2 and option 2-1/2-2

- Other group common signalling on MCCH

- Difference between option 1 and option 2/2-1/2-2

- Difference between option 2-1 and option 2-2

The difference between option 2-1 and option 2-2 is analyzed as below.

- Scheduling and configuration of MCCH

The cell specific MCCH of option 2-1 is scheduled with a cell specific RNTI. The configuration information of the cell specific MCCH includes: higher layer configuration and L1 configuration. The L1 configuration of the cell specific MCCH includes: a CORESET/CSS, a MCCH transmission window and PDSCH configuration (frequency resource and PDSCH parameters）.

If the cell specific MCCH uses the following higher layer configuration like MCCH of broadcast sessions, only the L1 configuration of the cell specific MCCH needs to be provided by dedicated signalling for option 2-1.

Table 1: Parameters that are specified for MCCH of broadcast sessions.

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  | UM RLC |  |
| >sn-FieldLength | 6 |  |  |
| >t-Reassembly | ms0 |  |  |
| Logical channel ID | 0 |  |  |

The session specific MCCH of option 2-2 is scheduled with G-RNTI/G-CS-RNTI, which means the session specific MCCH and the MTCHs associated with a same G-RNTI/G-CS-RNTI can be multiplexed onto a same PDSCH and share same L1 configuration: a same G-RNTI/G-CS-RNTI, a same CORESET/CSS, a same DRX mode, a same SSB mapping window and same PDSCH configuration. Therefore the session specific MCCH needs no extra L1 configuration.

If the session specific MCCH uses the higher layer configuration as shown in Table 2, only an extra parameter “t-Reassembly” needs to be provided by dedicated signalling for option 2-2.

Table 2: Parameters for session specific MCCH with no integrity protection and ciphering

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  | UM RLC |  |
| >sn-FieldLength | 6 |  |  |
| >t-Reassembly | Default value: ms0 | The default value is used if PTM configuration for RRC_INACTIVE state has no L1 retransmission with PTM mode |  |
| Logical channel ID | 0 |  |  |

Based on the above analysis, option 2-1 needs the L1 configuration of the cell specific MCCH to be provided by dedicated signalling while option 2-2 only needs one extra parameter to be provided by dedicated signalling. Therefore, option 2-2 needs less standard effort than option 2-1. Furthermore, option 2-2 needs no extra radio resource for configuring the session specific MCCH because the extra resource for configuring an extra parameter can be ignored.

- Complexity and power consumption in UE

The cell specific MCCH of option 2-1 needs UE to monitor an extra RNTI within an extra MCCH transmission window, which may increase complexity and power consumption in UE.

The session specific MCCH of option 2-2 shares a same G-RNTI/G-CS-RNTI, a same DRX mode and a same SSB mapping window with the session specific MTCHs, which means no extra complexity and no extra power consumption for UE to monitor the session specific MCCH.

Based on the above analysis, option 2-2 has less complexity and less power consumption in UE than option 2-1.

- Content of MCCH

The content of the cell specific MCCH of option 2-1 includes the PTM configurations of all multicast sessions which are provided to RRC_INACITE UE. UE only joining one multicast session can obtain the PTM configurations of the other multicast sessions with the cell specific MCCH.

If the content of the cell specific MCCH is large, many timeslots are scheduled for sending its content. Correspondingly UE needs to capture the cell specific MCCH in all scheduled timeslots to obtain the entire content of MCCH even if UE is only receiving one multicast session, thus leading to more power consumption than expected.

The content of the session specific MCCH of option 2-2 only includes the PTM configurations associated with a G-RNTI/G-CS-RNTI. Only the UEs joining a multicast session associated with a G-RNTI/G-CS-RNTI can obtain the PTM configuration(s) associated with the G-RNTI/G-CS-RNTI.

- MCCH configuration through dedicated signalling, MCCH acquisition and MCCH transmission

MCCH configuration through dedicated signalling and MCCH acquisition procedure for option 2-1:

Step 1: After UE goes into RRC_CONNECTED state for joining a multicast session and before UE is switched to RRC_INACTIVE/RRC_IDLE state, dedicated signalling is used to send the following configurations to UE.

- L1 configuration of the cell specific MCCH

- PTM configuration of the multicast session if available

In detail, the PTM configuration of a multicast session includes:

- MRB configuration: PDCP/RLC configuration and MTCH ID per MRB

- L1 configuration: G-RNTI/G-CS-RNTI, CORESET/CSS, DRX mode, SSB mapping window and PDSCH configuration

Step 2: After UE receives dedicated signalling, UE starts to monitor the cell specific MCCH according to the L1 configuration of the cell specific MCCH.

If the PTM configuration is sent to UE along with the L1 configuration of the cell specific MCCH, UE monitors the cell specific MCCH for PTM configuration update. Otherwise, UE monitors the cell specific MCCH for both PTM configuration acquisition and PTM configuration update.

MCCH transmission procedure for option 2-1:

According to the above MCCH configuration and acquisition procedure for option 2-1, when the PTM configuration of a multicast session is available, gNB can send it on MCCH once or several times. Afterwards, each time the PTM configuration of the multicast session is updated, gNB can send the new PTM configuration once or several times. gNB has no need to send the new PTM configuration of the multicast session periodically.

Figure 1: MCCH configuration/MCCH acquisition/MCCH transmission procedure for option 2-1

Figure 1 is used to show the MCCH configuration, MCCH acquisition and MCCH transmission procedure for option 2-1.

- In Figure 1, UE1 joins a multicast session at t1 and is switched to RRC_INACTIVE state long before t2 while UE2 joins the multicast session at t4 and is switched to RRC_INACTIVE state long before t5.

- PTM configuration of the multicast session is available at t2 and updated at t5.

- Correspondingly MCCH sends the initial PTM configuration of the multicast session three times from t2 to t3 and updated PTM configuration of the multicast session three times from t5 to t6.

- gNB sends the L1 configuration of the cell specific MCCH to UE1 through dedicated signalling before UE1 is switched to RRC_INACTIVE state. After receiving dedicated signalling, UE1 starts to monitor the cell specific MCCH before t2. UE1 acquires the initial PTM configuration during the period from t2 to t3 and then updated PTM configuration during the period from t5 to t6.

- gNB sends the L1 configuration of the cell specific MCCH and the initial PTM configuration to UE2 through dedicated signalling before UE2 is switched to RRC_INACTIVE state. After receiving dedicated signalling, UE2 starts to monitor the cell specific MCCH before t5. UE2 acquires the updated PTM configuration during the period from t5 to t6.

MCCH configuration through dedicated signalling and MCCH acquisition procedure for option 2-2: similar to those for option 2-1

Step 1: After UE goes into RRC_CONNECTED state for joining a multicast session and before UE is switched to RRC_INACTIVE/RRC_IDLE state, dedicated signalling is used to send the following configurations to UE.

- MRB configuration of the multicast session if available: PDCP/RLC configuration and MTCH ID per MRB

- L1 configuration shared by the session specific MCCH and the above MTCH(s): G-RNTI/G-CS-RNTI, CORESET/CSS, DRX mode, SSB mapping window and PDSCH configuration

- t-Reassembly if the retransmission with PTM mode is used for the multicast session

Step 2: After UE receives dedicated signalling, UE starts to monitor the session specific MCCH according to the L1 configuration and t-Reassembly if configured.

If the MRB configuration is sent to UE along with the L1 configuration, UE monitors the session specific MCCH for PTM configuration update. Otherwise, UE monitors the session specific MCCH for both PTM configuration acquisition and PTM configuration update.

MCCH transmission procedure for option 2-2: similar to transmission procedure for option 2-1

For the session specific MCCH and a multicast session associated with a same G-RNTI/G-CS-RNTI, when the PTM configuration of the multicast session is available, gNB sends it on the session specific MCCH once or several times. Afterwards, each time the PTM configuration of the multicast session is updated, gNB sends the new PTM configuration on the session specific MCCH once or several times. gNB has no need to send the new PTM configuration of multicast session periodically.

- Standard effort

The configuration information of MCCH in option 2-1/option 2-2 includes:

- Higher layer configuration

- L1 configuration

For the cell specific MCCH of option 2-1, the following standard effort is needed

- Allocate a fixed RNTI to the cell specific MCCH

- Define the higher layer configuration for the cell specific MCCH

  - Use same higher layer configuration as MCCH of broadcast sessions

- Define an IE for sending the L1 configuration of the cell specific MCCH

  - Reuse the IE for sending the L1 configuration of MCCH of broadcast sessions

For the session specific MCCH of option 2-2, the following standard effort is needed

- Define higher layer configuration for the session specific MCCH

  - Introduce one extra parameter: t-Reassembly

- No need to define an IE for sending the L1 configuration of the session specific MCCH: MCCH and MTCHs share same L1 configuration

- Difference between option 2 and option 2-1/2-2

Option 2 has the following features:

- Configuration of the cell specific MCCH is sent on SIB

- Content of the cell specific MCCH includes the PTM configurations of all multicast sessions in RRC_INACTIVE state

The difference between option 2 and option 2-1/2-2 is reflected in the following aspects.

- Which UEs can obtain MCCH configuration

For option 2, all UEs in the cell can obtain MCCH configuration on the SIB configuring MCCH. For option 2-1/2-2, only authorized UEs can obtain MCCH configuration through dedicated signalling.

- How much resource is needed for sending MCCH configuration

For option 2, MCCH configuration is sent periodically on the SIB. For option 2-1/2-2, MCCH configuration is sent to UE through dedicated signalling for one times. Therefore, option 2-1/2-2 needs less resource for configuring MCCH than option 2.

- How much resource is needed for sending content of MCCH

For option 2, due to the fact that different UEs may join a multicast session at different time instances, gNB needs to send content of the cell specific MCCH periodically, which means the PTM configurations of all multicast sessions are sent periodically on the cell specific MCCH of option 2.

For option 2-1, each time the PTM configuration of a multicast session is updated, gNB only needs to send the updated PTM configuration on the cell specific MCCH once or several times. Therefore, option 2-1 needs less resource for sending content of MCCH than option 2.

For option 2-2, each time the PTM configuration of a multicast session is updated, gNB only needs to send the updated PTM configuration on the corresponding session specific MCCH once or several times. Therefore, although option 2-2 configures one session specific MCCH per G-RNTI/G-CS-RNTI, the sum of the content of each session specific MCCH is equal to content of the cell specific MCCH of option 2-1, therefore option 2-2 needs less resource than option 2.

- Standard effort

The configuration information of the cell specific MCCH in option 2 includes:

- Higher layer configuration

- L1 configuration

If the cell specific MCCH uses the higher layer configuration as shown in Table 1, only the L1 configuration of the cell specific MCCH needs to be provided on the SIB. Therefore the following standard effort for option 2 is needed

- Define an SIB for configuring the cell specific MCCH

- Allocate a fixed RNTI to the cell specific MCCH

- Define the higher layer configuration for the cell specific MCCH

  - Use same higher layer configuration as MCCH of broadcast sessions

- Define an IE for sending the L1 configuration of the cell specific MCCH

  - Reuse the IE for sending the L1 configuration of MCCH of broadcast sessions

Compare the standard effort for option 2 with the standard effort for option 2-1/2-2, option 2-1/2-2 has less standard effort because option 2-1/2-2 has no need for a new SIB.

- Other group common signalling on MCCH

In the above sections, MCCH of option 2/2-1/2-2 is only used to send PTM configuration. In addition to PTM configuration, there are other group common signalling as listed below. MCCH of option 2/2-1/2-2 can be used to send some of other group common signalling.

- PTM configuration of each neighbour cell

The PTM configuration of a neighbour cell may be different and change with time, it’s better to provide the latest PTM configuration of a neighbour cell to UE on MCCH.

- PRACH configuration

If the PRACH configuration for triggering random access procedure by UE is changed with time, the latest PRACH configuration can be sent to UE on MCCH.

- Multicast session deactivation notification

If multicast session deactivation notification is sent with group paging on PCCH, more resource is needed because different UEs may have different POs. Furthermore different UEs may receive multicast session deactivation notification at different time instances.

If multicast session deactivation notification is sent on MCCH, beam sweeping can be used to send multicast session deactivation notification. Different UEs can receive multicast session deactivation notification simultaneously.

Therefore, it’s better to send multicast session deactivation notification on MCCH because MCCH consumes less resource and introduces no different delays between different UEs.

- Multicast session release notification

According to the same reason as for multicast session deactivation, multicast session release notification can be sent on MCCH.

- RRC state switching notification from RRC_INACTIVE state to RRC_CONNECTED state

- FFS: other group common signalling

- Difference between option 1 and option 2/2-1/2-2

The difference between option 1 and option 2/2-1/2-2 is reflected in the following aspects.

- Delay

Option 1 needs UE to go into RRC_CONNECTED state and then sends PTM configuration to UE. Option 2/2-1/2-2 sends PTM configuration on MCCH without switching UE to RRC_CONNECTED state. Therefore, option 2/2-1/2-2 introduces less delay for UE to obtain PTM configuration.

- Resource consumption

For option 1, random access related resource and the resource for sending PTM configuration on DCCH are needed per UE per multicast session. For option 2-1/2-2, the resource for sending PTM configuration with PTM mode on MCCH is needed per beam per multicast session. Therefore, option 2-1/2-2 need less resource than option 1.

- Application range

MCCH of option 2/2-1/2-2 can be used to send other group common signalling with acceptable resource in addition to PTM configuration. But if option 1 is used to send other group common signalling, the resource consumed by option 1 is unacceptable. Therefore, option 2/2-1/2-2 has wider application range than option 1.

- Conclusion

Based on the discussion in the former sections, the following proposals are suggested.

Proposal 1: Study the following options for sending PTM configuration

- Option 2: SIB + cell specific MCCH

- Option 2-1: dedicated signalling + cell specific MCCH

- Option 2-2: dedicated signalling + session specific MCCH

Proposal 2: MCCH of option 2/2-1/2-2 can be used to send some of following group common signalling

- PTM configuration of each neighbour cell

- PRACH configuration

- Multicast session deactivation notification

- Multicast session release notification

- RRC state switching notification from RRC_INACTIVE state to RRC_CONNECTED state

- FFS: other group common signalling

Proposal 3: Selected among option 1, option 2, option 2-1 and option 2-2.

- References

- RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, RAN Meeting#96-e

- R2-2206988, Multicast reception in RRC_INACTIVE state, RAN2#119-e